| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0084-v2 |
| --- | --- |

| Title: | CE4-related: Motion compression for HMVP buffers and MV line buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao, Tzu-Der Chuang, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yuling.hsiao, peter.chuang, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to utilize the same mantissa-exponent representation for temporal motion vector predictors (TMVPs) to reduce the number of bits stored in the history motion vector predictor (HMVP) buffers and motion vector (MV) line buffer. With motion compression for regular merge HMVP buffer and intra block copy (IBC) HMVP buffer, in total 320 bits are reduced. With motion compression for MV line buffer, 32 bits are reduced per 4x4 luma block, and 30720 bits are reduced in case of 4K video. Results are summarized as follows.

Proposed motion compression for HMVP buffers
VTM4.0-AI: {Y-BD-rate=0.00%, EncT=100%, DecT=100%}
VTM4.0-RA: {Y-BD-rate=0.02%, EncT=100%, DecT=100%}
VTM4.0-LB: {Y-BD-rate=0.01%, EncT=100%, DecT=99%}

Proposed motion compression for MV line buffer
VTM4.0-AI: {Y-BD-rate=0.00%, EncT=100%, DecT=100%}
VTM4.0-RA: {Y-BD-rate=0.09%, EncT=100%, DecT=100%}
VTM4.0-LB: {Y-BD-rate=0.02%, EncT=100%, DecT=100%}

# Introduction

The motion compression method of reducing the number of bits stored in the TMVP buffer presented in JVET-M0512 [1] was adopted. In JVET-M0512, it is proposed to convert a MV to a 6-bit mantissa with sign and 4-bit exponent representation prior to storage. It is asserted that the selected representation effectively quantizes larger MV magnitudes more coarsely while retaining higher precision for smaller MV magnitudes.

To further reduce buffer sizes, the motion compression method by mantissa-exponent representation is extended to the two HMVP buffers (one for regular merge, one for IBC) and the MV line buffer. Details are described in the following sections.

# Proposed methods

## Motion compression for HMVP buffers

The motion compression method by mantissa-exponent representation is applied to the MVs in the two HMVP buffers, including regular merge HMVP buffer and IBC HMVP buffer. With this method, 320 bits for HMVP buffers are reduced (2 x 2 x (18-10) x 5 entries x 2 buffers).

## Motion compression for MV line buffer

The motion compression method by mantissa-exponent representation is applied to MV line buffer. When fetching MVs crossing a CTU row boundary, compressed MVs are fetched. As shown in Figure 1, the proposed method includes three steps.

Step 1: Load MVs from the compressed MV line buffer, decompress MVs, and store decompressed MVs to the uncompressed MV CTU buffer

Step 2: Update MVs in the current CTU in the uncompressed MV CTU buffer

Step 3: Compress the MVs from the uncompressed MV CTU buffer, store the compressed MVs to the compressed MV line buffer for next CTU row

With this method, 32 bits are reduced per 4x4 luma block, and 30720 bits are reduced in case of 4K video.

Figure 1.  Illustration of motion compression for MV line buffer (one block is 4x4 MV)

# Experiment results

The proposed motion compression for HMVP buffers and motion compression for MV line buffer have been implemented on top of VTM4.0. The common test conditions [2] are used for evaluation. Table 1 summarizes the results of the motion compression for HMVP buffers, and Table 2 summarizes the results of the motion compression for MV line buffer.

Table 1.  Motion compression for HMVP buffers

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class F | 0.00% | 0.12% | -0.01% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | -0.02% | 100% | 99% |
| Class A2 | -0.01% | 0.05% | 0.03% | 100% | 101% |
| Class B | 0.03% | -0.10% | 0.05% | 100% | 100% |
| Class C | 0.02% | 0.02% | -0.14% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.03% | -0.02% | 100% | 100% |
| Class D | 0.00% | -0.04% | -0.02% | 100% | 99% |
| Class F | -0.02% | 0.11% | -0.15% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.03% | 0.16% | 100% | 99% |
| Class C | 0.01% | 0.07% | -0.01% | 100% | 100% |
| Class E | -0.01% | 0.06% | -0.11% | 100% | 96% |
| Overall | 0.01% | 0.05% | 0.04% | 100% | 99% |
| Class D | -0.01% | -0.08% | -0.81% | 100% | 100% |
| Class F | -0.02% | 0.02% | -0.08% | 101% | 100% |

Table 2.  Motion compression for MV line buffer

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class F | 0.00% | -0.02% | -0.02% | 100% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.14% | 0.05% | 0.16% | 100% | 101% |
| Class A2 | 0.06% | -0.04% | 0.00% | 100% | 100% |
| Class B | 0.10% | -0.01% | 0.04% | 100% | 100% |
| Class C | 0.05% | 0.03% | -0.15% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.01% | 0.00% | 100% | 100% |
| Class D | -0.01% | 0.02% | -0.14% | 100% | 100% |
| Class F | 0.03% | 0.02% | -0.04% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.17% | 0.00% | 100% | 102% |
| Class C | -0.01% | 0.13% | -0.16% | 100% | 100% |
| Class E | 0.04% | -0.16% | -0.32% | 100% | 96% |
| Overall | 0.02% | -0.07% | -0.13% | 100% | 100% |
| Class D | -0.04% | 0.06% | -0.42% | 101% | 101% |
| Class F | -0.17% | -0.26% | 0.05% | 100% | 101% |

# Conclusion

In this contribution, it is proposed to compress motion for HMVP buffers and MV line buffer. Coding efficiency impact is minor. Compared with the current design in VTM4.0, the buffer usage can be reduced. The proposal results in saving of 320 bits for HMVP buffers and saving of 31K bits for MV line buffer in case of 4K video.

# References

[1] F. Bossen, K. Misra, and A. Segall, “Non-CE4: On temporal motion buffer compression,” Document of Joint Video Experts Team, JVET-M0512.

[2] F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).