| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0085-v1 |
| --- | --- |

| Title: | CE4-related: MV rounding unification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VTM4.0, two kinds of motion vector (MV) rounding methods, rounding to zero and rounding away from zero are used. The rounding to zero is used in the spatial/temporal MV scaling in inter mode motion vector predictor (MVP) derivation and regular merging candidate list generation. The rounding away from zero is used in inherited affine merging candidate derivation, affine subblock MV derivation, pair-wise merging candidate derivation, and MV precision rounding in adaptive motion vector difference resolution (AMVR). In this proposal, two unification methods of MV rounding are tested. The MV rounding in VTM4.0 are unified to either rounding to zero or rounding away from zero. Simulation results reportedly show that, for rounding to zero, the average luma BD-rates are -0.01% and -0.06% in RA and LB cases, respectively. For rounding away from zero, the average luma BD-rates are 0.01% and -0.01% in RA and LB cases, respectively. Therefore, it is proposed to unify the MV rounding process to rounding to zero.

# Introduction

In VTM4.0, there are two kinds of MV rounding methods, rounding to zero and rounding away from zero. The rounding to zero is used in the spatial/temporal MV scaling in inter mode MVP derivation and regular merging candidate list generation. The rounding away from zero is used in inherited affine merging candidate derivation, affine subblock MV derivation, pair-wise merging candidate derivation, and MV precision rounding in AMVR.

The equation of the rounding to zero is as shown in Equation 1, and the rounding away from zero is shown in Equation 2. The only difference between these two rounding methods is the value of the rounding offset. The offset of rounding to zero is smaller than the offset of rounding away from zero.

MV_rounded = MV >= 0 ? ((MV + offset) >> shift) : -(((-MV) + offset) >> M),

where offset = (1 << (shift – 1)) – 1 							(Equation 1)

MV_rounded = MV >= 0 ? ((MV + offset) >> shift) : -(((-MV) + offset) >> M),

where offset = (1 << (shift – 1))									(Equation 2)

# Proposed methods

To unify the MV rounding in VTM, the two MV rounding methods are proposed for testing. All the MV rounding operations use either rounding to zero or rounding away from zero.

# Simulation results

The common test conditions [1] are used for evaluation. In the first test, all the MV rounding operations are unified to rounding to zero. The results are shown in Table 1. Simulation results reportedly show that the average luma BD-rates are -0.01% and -0.06% in RA and LB cases, respectively. In the second test, all the MV rounding operations are unified to rounding away from zero. The results are shown in Table 2. Simulation results reportedly show that the average luma BD-rates are 0.01% and -0.01% in RA and LB cases, respectively.

Table 1.  Results of rounding to zero for all MV rounding operations

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.05% | -0.04% | 99% | 101% |
| Class A2 | -0.02% | 0.02% | -0.03% | 100% | 100% |
| Class B | -0.03% | -0.01% | 0.01% | 99% | 100% |
| Class C | 0.01% | -0.03% | -0.19% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.02% | -0.06% | 100% | 101% |
| Class D | -0.03% | -0.02% | -0.04% | 101% | 96% |
| Class F | -0.02% | 0.02% | -0.12% | 100% | 94% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.04% | -0.15% | 101% | 100% |
| Class C | -0.10% | -0.07% | -0.53% | 101% | 99% |
| Class E | -0.06% | 0.02% | -0.55% | 101% | 96% |
| Overall | -0.06% | -0.03% | -0.38% | 101% | 99% |
| Class D | -0.14% | -0.75% | -0.99% | 101% | 101% |
| Class F | -0.23% | -0.27% | -0.04% | 101% | 100% |

Table 2.  Results of rounding away from zero for all MV rounding operations

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.06% | 0.02% | 100% | 101% |
| Class A2 | 0.00% | 0.06% | 0.02% | 99% | 100% |
| Class B | 0.01% | -0.02% | -0.01% | 98% | 100% |
| Class C | 0.02% | -0.02% | 0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.01% | 99% | 100% |
| Class D | 0.04% | 0.10% | 0.07% | 101% | 98% |
| Class F | 0.01% | 0.01% | 0.02% | 99% | 94% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.06% | -0.07% | 100% | 102% |
| Class C | -0.03% | 0.19% | 0.17% | 100% | 100% |
| Class E | -0.02% | -0.05% | -0.03% | 99% | 96% |
| Overall | -0.01% | 0.03% | 0.02% | 100% | 100% |
| Class D | 0.00% | 0.01% | -0.55% | 100% | 101% |
| Class F | -0.04% | -0.28% | -0.11% | 100% | 105% |

# Conclusions

In this contribution, two MV rounding unification methods are tested. All the MV rounding operations are unified to rounding to zero or rounding away from zero. From the results, it is proposed to unify the MV rounding to rounding to zero.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).