| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0086-v1 |
| --- | --- |

| Title: | CE4-related: Reduction of interactions between bi-prediction coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chia Chen, Man-Shu Chiang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chunchia.chen, man-shu.chiang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to reduce interaction between four bi-prediction coding tools, decoder-side motion vector refinement (DMVR), bi-directional optical flow (BDOF), bi-prediction with coding unit weights (BiCW), and combined inter merge / intra prediction (CIIP). In VTM4.0, BDOF and BiCW are disallowed to be applied simultaneously, and all other multi-tool use of the four tools are allowed. Therefore, up to three of the four tools can be cascaded for a CU, and this may result in difficult hardware pipelining design and low hardware utilization rate. Two methods are proposed to address these issues. The first method only allows DMVR and BDOF to be applied to a CU simultaneously and disallows any other multi-tool use of the four tools for a CU. The second method disallows multi-tool use of the four tools for a CU without exceptions. Moreover, in order to make hardware design more regular, DMVR search is modified, and BDOF is additionally supported in case of subblock-based temporal motion vector prediction (SbTMVP). Simulation results are summarized as follows.

Method 1: disallow multi-tool use of {DMVR, BDOF, BiCW, CIIP} except DMVR+BDOF
VTM4.0-RA: {BD-rate=0.06% (Y), -0.04% (Cb), -0.01% (Cr); EncT=100%; DecT=100%}
VTM4.0-LB: {BD-rate=0.02% (Y), 0.07% (Cb), 0.12% (Cr); EncT=100%; DecT=100%}

Method 2: disallow multi-tool use of {DMVR, BDOF, BiCW, CIIP} without any exceptions, modify DMVR search, support BDOF in case of SbTMVP
VTM4.0-RA: {BD-rate=0.35% (Y), 0.12% (Cb), 0.10% (Cr); EncT=100%; DecT=99%}
VTM4.0-LB: {BD-rate=0.02% (Y), 0.07% (Cb), 0.12% (Cr); EncT=100%; DecT=100%}

For the method 1, 98% of the four tools’ coding gain (luma BD-rate= 2.82% RA, 0.83% LB) is preserved. For the method 2, 88% of the four tools’ coding gain is preserved.

# Introduction

In VTM4.0, four bi-prediction coding tools, DMVR, BDOF, BiCW, and CIIP are adopted. Note that BiCW was named generalized bi-prediction (GBi) and bi-prediction with weighted averaging (BWA) before when it appeared in initial proposals and VTM algorithm description, respectively. The processing order of these four tools are DMVR => BDOF => BiCW => CIIP. There are two hardware decoder implementation problems caused by the cascaded design. The first problem is related to hardware pipelining. It is very challenging to put the four cascaded tools in one pipeline stage because of the required long clock cycles. If the four tools are divided from one pipeline stage into multiple pipeline stages, the decoder suffers a significant cost increase because of internal buffer increases. The second problem is related to hardware utilization. The hardware has to be designed for the worst case. However, it is possible that only one, two, or three of the four tools are applied to a CU, and in these cases the hardware utilization rate is low because a portion of the hardware circuits for the deactivated tool(s) may be idle.

In VTM4.0, BDOF and BiCW are disallowed to be applied simultaneously, and all other multi-tool use of the four tools are allowed. Table 1 is used to illustrate the interactions between the four tools.

Table 1.  Interactions between DMVR, BDOF, BiCW, and CIIP in VTM4.0

|  | DMVR | BDOF | BiCW | CIIP |
| --- | --- | --- | --- | --- |
| DMVR | NA | O | O | O |
| BDOF | O | NA | X | O |
| BiCW | O | X | NA | O |
| CIIP | O | O | O | NA |

NA = not applicable; O = coupling exists between two tools; X = no coupling between two tools

Two methods are proposed to solve the hardware issues. More details are described in the following section.

# Proposed methods

In method 1, it is proposed to reduce the cascading behavior by the following modifications to VTM4.0:

- CIIP, if activated, in case of selecting a bi-prediction merging candidate, directly uses the prediction from list 0 (L0) or list 1 (L1) with the lower picture order count (POC) distance between the current picture and the reference picture. When CIIP is applied, uni-prediction is used as inter prediction, and DMVR, BDOF, and BiCW, which are only allowed for bi-prediction, are disallowed automatically.

- If BiCW is activated for the current CU, i.e., the BiCW index indicates unequal weights, DMVR is turned off. Note that, in this case, BDOF is also turned off, as in VTM4.0.

With the above modifications, only the coupling between DMVR and BDOF exists, as shown in Table 2.

Table 2.  Interactions between DMVR, BDOF, BiCW, and CIIP in method 1

|  | DMVR | BDOF | BiCW | CIIP |
| --- | --- | --- | --- | --- |
| DMVR | NA | O | X | X |
| BDOF | O | NA | X | X |
| BiCW | X | X | NA | X |
| CIIP | X | X | X | NA |

NA = not applicable; O = coupling exists between two tools; X = no coupling between two tools

In method 2, it is proposed to totally remove the cascading behavior. Based on method 1, further modifications are applied as follows.

- If DMVR is activated for the current CU by the implicit rule in VTM 4.0, BDOF is turned off

- For DMVR, an eight-neighbor search pattern is used for each integer sample search iteration.

- For BDOF, it is modified to coexist with SbTMVP.

The items (2) and (3) can make hardware design more regular, although the hardware complexity is slightly increased. They also can control the coding efficiency loss within an acceptable range. Note that method 2 still reduces the overall hardware complexity of the four tools significantly. Table 3 shows the interactions between the four tools in method 2.

Table 3.  Interactions between DMVR, BDOF, BiCW, and CIIP in method 2

|  | DMVR | BDOF | BiCW | CIIP |
| --- | --- | --- | --- | --- |
| DMVR | NA | X | X | X |
| BDOF | X | NA | X | X |
| BiCW | X | X | NA | X |
| CIIP | X | X | X | NA |

NA = not applicable; O = coupling exists between two tools; X = no coupling between two tools

Since the cascading behavior is reduced or removed, the four tools can be easily implemented in one hardware decoder pipeline stage. In addition, common operations in the four tools can be implemented as shared circuits to increase the hardware utilization rate.

# Simulation results

The proposed methods are integrated on VTM4.0. All tests were conducted under the common test conditions [1]. Table 4 shows the results of disabling DMVR, BDOF, BiCW, and CIIP to see how much coding gain is provided by the four tools. Table 5 shows the results of method 1. Table 6 shows the results of method 2 of only RA case. Note that because the differences of method 2 and method 1 are all related to BDOF and DMVR, the LDB results of method 2 are the same as method 1. The anchors of all the tests are VTM 4.0.

Table 4.  Results of disabling DMVR, BDOF, BiCW, and CIIP

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.64% | 3.05% | 3.36% | 85% | 91% |
| Class A2 | 3.73% | 3.17% | 2.98% | 84% | 94% |
| Class B | 2.75% | 2.08% | 2.13% | 84% | 88% |
| Class C | 2.37% | 2.05% | 1.97% | 85% | 87% |
| Class E |  |  |  |  |  |
| Overall | 2.82% | 2.48% | 2.50% | 84% | 89% |
| Class D | 2.86% | 2.18% | 2.11% | 87% | 84% |
| Class F | 1.47% | 1.45% | 1.26% | 86% | 85% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.06% | 1.57% | 1.83% | 91% | 99% |
| Class C | 0.75% | 1.23% | 1.57% | 91% | 101% |
| Class E | 0.52% | 1.71% | 1.16% | 91% | 94% |
| Overall | 0.83% | 1.49% | 1.58% | 91% | 99% |
| Class D | 0.57% | 1.49% | 0.72% | 92% | 100% |
| Class F | 1.28% | 1.18% | 0.96% | 95% | 102% |

Table 5.  Results of method 1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.03% | 0.10% | 100% | 100% |
| Class A2 | 0.04% | -0.02% | 0.11% | 100% | 100% |
| Class B | 0.04% | -0.19% | -0.09% | 100% | 100% |
| Class C | 0.07% | 0.11% | -0.07% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | -0.04% | -0.01% | 100% | 100% |
| Class D | 0.05% | 0.00% | 0.00% | 101% | 99% |
| Class F | -0.05% | -0.01% | -0.03% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.08% | 0.25% | 100% | 100% |
| Class C | 0.04% | 0.06% | 0.00% | 100% | 101% |
| Class E | -0.02% | 0.06% | 0.08% | 100% | 96% |
| Overall | 0.02% | 0.07% | 0.12% | 100% | 100% |
| Class D | -0.03% | 0.13% | -0.39% | 101% | 100% |
| Class F | 0.00% | 0.35% | 0.27% | 100% | 100% |

Table 6.  Results of method 2 (LB results are the same as those of method 1, as shown in Table 5)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.21% | 0.03% | 0.11% | 100% | 100% |
| Class A2 | 0.47% | 0.09% | 0.17% | 100% | 100% |
| Class B | 0.33% | 0.03% | 0.05% | 100% | 99% |
| Class C | 0.40% | 0.34% | 0.10% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.35% | 0.12% | 0.10% | 100% | 99% |
| Class D | 0.58% | 0.38% | 0.30% | 101% | 98% |
| Class F | 0.10% | 0.08% | 0.00% | 100% | 100% |

# Conclusions

In this contribution, two methods are proposed to reduce or remove the cascading behavior of DMVR, BDOF, BiCW, and CIIP by mutual excluding each other in a CU.

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).