| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0087-v1 |
| --- | --- |

| Title: | CE4-related: Simplification of triangle merging candidate list derivation on top of CE4-4.1c |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VTM4.0 triangle merging candidate list derivation, at most 94 times of motion vector (MV) pruning, seven times of MV averaging, and seven times of MV scaling are required. To reduce the complexity, a method on top of CE4-4.1c is proposed to replace the current triangle merging candidate list with the regular merging candidate list to generate a uni-prediction MV for each of the two triangular partitions. Two merging candidates from the regular merging candidate list are first selected. Next, the first triangle merging candidate is set equal to the list 0 (L0) MV and list 1 (L1) MV of the first initial merging candidate if the L0 MV of the first initial merging candidate exists and otherwise, respectively. Similarly, the second triangle merging candidate is set equal to the L1 MV and L0 MV of the second initial merging candidate if the L1 MV of the second initial merging candidate exists and otherwise, respectively. An MV pruning is performed on these two uni-prediction candidates. If they are the same and the second merging candidate is bi-prediction, the L0 MV is used instead of the L1 MV for the second triangular partition. Otherwise, if they are the same and the first merging candidate is bi-prediction, the L1 MV is used instead of the L0 MV for the first triangular partition. Otherwise, an offset MV of (1/4 pel, 0) is added to the merging candidate of the second partition if the two merging candidates of the two triangular partitions are the same. In this way, the complexity of the proposed triangle merging candidate list derivation is kept as low as that of the regular merging candidate list derivation plus only four inter prediction indicator (inter_pred_idc, to indicate L0, L1, or bi-prediction) comparisons, one MV comparison, and one MV addition. Simulation results reportedly show that the average luma BD-rates are 0.04% and 0.07% in RA and LB cases, respectively

# Introduction

In VTM4.0 triangle prediction, a skip or merge coding unit (CU) is divided into two triangular partitions. Each triangular partition is inter-predicted using its own uni-prediction MV and reference index which are derived from a new triangle merging candidate list of five uni-prediction candidates. In the triangle merging candidate list derivation, the MVs of seven neighbouring blocks including five spatial neighbouring blocks (block-1 to block-5 as shown in Figure 1) and two temporal collocated blocks (block-6 and block-7) are used. A full pruning process is applied to spatial candidates to remove redundant candidates. At most 20 times of MV pruning are required at this candidate pruning stage. The MVs of the pruned candidates are collected and put into the triangle merging candidate list as follows. The uni-prediction MVs are put into the candidate list first and followed by the L0 MVs of the bi-prediction candidates, the L1 MVs of the bi-prediction candidates, and averaged MVs of the L0 and L1 MVs of bi-prediction candidates. To average L0 and L1 MVs, the L1 MV is scaled to the reference picture of the L0 MV. When adding one uni-prediction MV, a full pruning process comparing the to-be-added MV with all the MVs that are already in the list is performed. At most 74 times of MV pruning, seven times of MV averaging, and seven times of MV scaling are required at this candidate insertion stage. If the number of candidates is less than five, zero MVs are added to the list. For the entire triangle merging candidate list derivation, at most 94 times of MV pruning, seven times of MV averaging, and seven times of MV scaling are required. The triangle merging candidate list derivation is very complex and undesirable especially for hardware decoders.

Figure 1.  Positions of the neighbouring blocks for triangle merging candidate derivation

In CE4-4.1c, it is proposed to reuse the regular merging candidate list for generating the two uni-prediction MVs for the two triangular partitions. In this contribution, it is proposed to improve the coding efficiency of CE4-4.1c. The proposed method is described in Section 2, and the results are reported in Section 3.

# Proposed method

## Recap of CE4-4.1.c

In CE4-4.1c, it is proposed to reuse the regular merging candidate list for generating the two uni-prediction MVs for the two triangular partitions. When the triangle prediction is applied, two merge indices are signalled. The current triangle merging candidate list in VTM4.0 is replaced with the regular merging candidate list. After the two initial candidates from the regular merging candidate list are selected, for the first triangle merging candidate, if the L0 MV of the first initial merging candidate exists, the first triangle merging candidate is set equal to the L0 MV of the first initial merging candidate; otherwise, the first triangle merging candidate is set equal to the L1 MV of the first initial merging candidate. Likewise, if the L1 MV of the second initial merging candidate exists, the second triangle merging candidate is set equal to the L1 MV of the second initial merging candidate; otherwise, the second triangle merging candidate is set equal to the L0 MV of the second initial merging candidate. A MV pruning is performed for these two triangle merging candidates. If the first triangular partition and the second triangular partition have exactly the same merging candidate, an offset MV of (1/4 pel, 0) is added to the merging candidate of the second triangular partition. The complexity of the proposed triangle merging candidate list derivation is the same as that of the regular merging candidate list derivation plus only two inter prediction indicator comparisons, one MV comparison, and one MV addition.

## Proposed modification

Compared with CE4-4.1c, the proposed method adds two more branches before adding the offset MV. In the proposed method, the two initial uni-prediction candidates for two triangular partitions selection is the same as CE4-4.1c. Two merge indices are signalled when the triangle prediction is applied. The current triangle merging candidate list in VTM4.0 is replaced with the regular merging candidate list. Two initial candidates from the regular merging candidate list are selected. For the first triangle merging candidate, if the L0 MV of the first initial merging candidate exists, the first triangle merging candidate is set equal to the L0 MV of the first initial merging candidate; otherwise, the first triangle merging candidate is set equal to the L1 MV of the first initial merging candidate. Likewise, if the L1 MV of the second initial merging candidate exists, the second triangle merging candidate is set equal to the L1 MV of the second initial merging candidate; otherwise, the second triangle merging candidate is set equal to the L0 MV of the second initial merging candidate. A MV pruning is performed for these two triangle merging candidates. If they are the same and the second merging candidate is bi-prediction, the L0 MV is used instead of the L1 MV for the second triangular partition. Otherwise, if they are the same and the first merging candidate is bi-prediction, the L1 MV is used instead of the L0 MV for the first triangular partition. Otherwise, an offset MV of (1/4 pel, 0) is added to the merging candidate of the second partition if the two merging candidates of the two triangular partitions are the same. The complexity of the proposed triangle merging candidate list derivation is the same as that of the regular merging candidate list derivation plus only four inter prediction indicator comparisons, one MV comparison, and one MV addition.

The candidate generation flow can be described as follows.

1. Select two initial candidates from the regular merging candidate list.

Cand0: {Cand0_L0, Cand0_L1}

Cand1: {Cand1_L0, Cand1_L1}

2. Generating two triangle merging candidates

TriCand0 = (Cand0_L0 exists) ? Cand0_L0 : Cand0_L1;

TriCand1 = (Cand1_L1 exists) ? Cand1_L1 : Cand1_L0;

3. Perform MV pruning

isSameCand = (TriCand0 = = TriCand1);

4. Modify one of the triangle merging candidates if they are the same

if (isSameCand){

if(Cand1 is bi-prediction)

TriCand1 = Cand1_L0;

else if (Cand0 is bi-prediction)

TriCand0 = Cand0_L1;

else

TriCand1 = TriCand1 + (1/4 pel, 0);

}

The blue part in the step 2 to step 4 is the candidate generation flow in CE4-4.1c. The red part in step 4 is the proposed modification on top of CE4-4.1c. Only two extra inter prediction indicator comparisons are added compared with CE4-4.1c.

# Simulation results

The proposed method is implemented on VTM4.0. The common test conditions [1] are used for evaluation. The results of the proposed method are shown in Table 1. Simulation results reportedly show that the average luma BD-rates are 0.04% and 0.07% in RA and LB cases, respectively.

Table 1.  Results of the proposed method

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.19% | 0.10% | 101% | 101% |
| Class A2 | -0.03% | -0.04% | 0.07% | 100% | 99% |
| Class B | 0.02% | 0.00% | 0.02% | 99% | 99% |
| Class C | 0.08% | 0.10% | -0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.06% | 0.02% | 100% | 100% |
| Class D | 0.06% | 0.01% | 0.18% | 101% | 98% |
| Class F | 0.10% | 0.09% | 0.03% | 100% | 94% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.07% | -0.32% | 100% | 104% |
| Class C | 0.09% | 0.14% | -0.05% | 101% | 103% |
| Class E | 0.04% | 0.82% | 0.31% | 101% | 96% |
| Overall | 0.07% | 0.28% | -0.07% | 101% | 101% |
| Class D | 0.13% | -0.20% | 0.12% | 101% | 101% |
| Class F | -0.09% | 0.10% | 0.19% | 101% | 100% |

# Conclusions

In this contribution, it is proposed to reuse the regular merging candidate list for generating the two uni-prediction MVs for the two triangular partitions. The triangle merging candidate list derivation is significantly simplified. Simulation results reportedly show that the average luma BD-rates are 0.04% and 0.07% in RA and LB cases, respectively.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).