| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0089-v2 |
| --- | --- |

| Title: | CE6-related: Unified implicit MTS between ISP mode and regular intra mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang,cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to replace “the implicit multiple transform selection (MTS) scheme of the regular intra mode in VTM4.0” with “the implicit MTS scheme applied to the intra subpartitions (ISP) mode in VTM4.0” as unification and complexity reduction. It is reported that, compared to VTM4.0 with implicit MTS enabled for regular intra mode, the proposed method (unified MTS between ISP and regular intra) provides -0.24%, -0.04%, and -0.03% luma BD-rates for AI, RA, and LB, respectively. Compared to VTM4.0 with MTS off (i.e., only DCT-II used), the proposed method achieves -1.83%, -0.96%, and -0.54% luma BD-rates for AI, RA, and LB, respectively. Compared to VTM4.0 common test condition (CTC) that uses explicit MTS for regular intra mode, the proposed method results in 0.65%, 0.31%, and 0.09% luma BD-rates with 38%, 3%, and 2% encoding time reductions for AI, RA, and LB, respectively.

# Introduction

ISP [1] is adopted into VTM4.0 [2]. The ISP mode divides a luma intra coding block (CB) vertically or horizontally into two or four subpartitions depending on the coding unit (CU) size, where all subpartitions have at least 16 samples and share one luma intra prediction mode. For each subpartition, residual signals are generated using the prediction signals according to the luma intra prediction mode. After transform, quantization, inverse quantization, and inverse transform, the reconstructed samples of each subpartition can be used to generate the prediction of the next subpartition. Moreover, there is an implicit MTS scheme for ISP, and it is shown in Table 1, where TB means transform block.

Shape adaptive transform (SAT) [3] is adopted into VTM4.0 for regular intra mode, and SAT is enabled when sps_mts_enabled_flag is equal to 1 and sps_explicit_mts_intra_enabled_flag is equal to 0. When SAT is enabled, an implicit MTS scheme different from that of ISP is applied to each luma TB of the luma CB coded with regular intra mode. The implicit MTS for regular intra mode is described as follows. The transform type for horizontal (or vertical) transform is changed to DST-VII when the TB width (or height) is within [4, 16] and is smaller than or equal to the TB height (or width).

As introduced in the previous two paragraphs, in VTM4.0, there are two implicit MTS schemes. One is for ISP mode, and the other is for regular intra mode.

Table 1.  The MTS scheme for the ISP mode

| Intra prediction mode | Horizontal transform type | Vertical transform type |
| --- | --- | --- |
| INTRA_PLANAR, INTRA_ANGULAR31, INTRA_ANGULAR32, INTRA_ANGULAR34, INTRA_ANGULAR36, INTRA_ANGULAR37 | (TB width >= 4 && TB width <= 16) ? DST-VII : DCT-II | (TB height >= 4 && TB height <= 16) ? DST-VII : DCT-II |
| INTRA_ANGULAR33, INTRA_ANGULAR35 | DCT-II | DCT-II |
| INTRA_ANGULAR2, INTRA_ANGULAR4,…,INTRA_ANGULAR28, INTRA_ANGULAR30, INTRA_ANGULAR39, INTRA_ANGULAR41,…,INTRA_ANGULAR63, INTRA_ANGULAR65 | (TB width >= 4 && TB width <= 16) ? DST-VII : DCT-II | DCT-II |
| INTRA_ANGULAR3, INTRA_ANGULAR5,…, INTRA_ANGULAR27, INTRA_ANGULAR29, INTRA_ANGULAR38, INTRA_ANGULAR40,…,INTRA_ANGULAR64, INTRA_ANGULAR66 | DCT-II | (TB height >= 4 && TB height <= 16) ? DST-VII : DCT-II |

# Proposed method

It is proposed to replace the current implicit MTS scheme of the regular intra mode with the implicit MTS scheme applied to the ISP mode, as shown in Table 1. For DC mode, which is not allowed in ISP mode, DCT-II is always used for both horizontal and vertical transforms for an intra CU.

# Experimental results

The proposed unified implicit MTS for ISP and regular intra has been implemented on VTM4.0. The simulations were performed following three test conditions, denoted as TC1, TC2, and TC3.

- [TC1]
Anchor: VTM4.0 with the implicit MTS of SAT for regular intra mode
Test: VTM4.0 with the proposed method (using the implicit MTS of ISP for regular intra mode)

- [TC2]
Anchor: VTM4.0 with MTS off (i.e., only DCT-II is used)
Test: VTM4.0 with the proposed method (using the implicit MTS of ISP for regular intra mode)

- [TC3]
Anchor: VTM4.0 CTC [4] (using explicit MTS for regular intra mode)
Test: VTM4.0 with the proposed method (using the implicit MTS of ISP for regular intra mode)

Table 2 shows the summary results. The full results are provided in Table 3, Table 4, and Table 5.

Table 2.  Summary of BD-rates for the proposed method

| Test condition Configuration |  | TC1 | TC2 | TC3 |
| --- | --- | --- | --- | --- |
| AI | Y | -0.24% | -1.83% | 0.65% |
|  | Cb | 0.10% | -1.93% | 0.11% |
|  | Cr | 0.09% | -1.94% | 0.09% |
| RA | Y | -0.04% | -0.96% | 0.31% |
|  | Cb | 0.14% | -0.66% | 0.10% |
|  | Cr | 0.07% | -0.70% | 0.01% |
| LB | Y | -0.03% | -0.54% | 0.09% |
|  | Cb | -0.68% | -0.74% | -0.12% |
|  | Cr | -0.25% | -0.12% | -0.05% |

Table 3.  Results of the proposed method under TC1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 with MTS 0 and MTSImplicit 1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | 0.12% | 0.15% | 102% | 101% |
| Class A2 | -0.18% | 0.12% | 0.03% | 101% | 98% |
| Class B | -0.27% | 0.21% | 0.17% | 101% | 99% |
| Class C | -0.22% | -0.02% | 0.08% | 102% | 99% |
| Class E | -0.33% | 0.03% | -0.04% | 101% | 99% |
| Overall | -0.24% | 0.10% | 0.09% | 101% | 99% |
| Class D | -0.29% | -0.06% | 0.09% | 103% | 100% |
| Class F | -0.09% | -0.08% | -0.01% | 103% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 with MTS 0 and MTSImplicit 1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.14% | -0.12% | 100% | 103% |
| Class A2 | -0.03% | 0.22% | 0.07% | 100% | 94% |
| Class B | -0.03% | 0.18% | 0.16% | 100% | 96% |
| Class C | -0.05% | 0.22% | 0.11% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.14% | 0.07% | 100% | 98% |
| Class D | -0.16% | -0.01% | -0.22% | 100% | 101% |
| Class F | -0.04% | 0.02% | -0.14% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 with MTS 0 and MTSImplicit 1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.21% | -0.37% | 100% | 98% |
| Class C | -0.05% | -0.21% | -0.19% | 100% | 100% |
| Class E | 0.03% | -2.07% | -0.14% | 100% | 100% |
| Overall | -0.03% | -0.68% | -0.25% | 100% | 99% |
| Class D | -0.08% | 0.02% | -0.09% | 101% | 100% |
| Class F | -0.07% | -0.36% | 0.08% | 100% | 99% |

Table 4.  Results of the proposed method under TC2

|  |  |  |  |  | All Intra Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Over VTM-4.0 with MTS 0 and MTSImplicit 1 (DCT-II only) |  |  |  |  |  |  |
|  | Y |  | U |  | V |  |  |  | EncT |  | DecT |
| Class A1 | -1.98% |  | -1.90% |  | -1.97% |  |  |  | 100% |  | 106% |
| Class A2 | -1.70% |  | -1.71% |  | -1.77% |  |  |  | 103% |  | 104% |
| Class B | -1.78% |  | -2.00% |  | -2.01% |  |  |  | 103% |  | 104% |
| Class C | -1.35% |  | -1.39% |  | -1.36% |  |  |  | 101% |  | 101% |
| Class E | -2.52% |  | -2.81% |  | -2.73% |  |  |  | 100% |  | 102% |
| Overall | -1.83% |  | -1.93% |  | -1.94% |  |  |  | 102% |  | 103% |
| Class D | -1.27% |  | -1.16% |  | -1.19% |  |  |  | 103% |  | 103% |
| Class F | -0.92% |  | -1.15% |  | -1.06% |  |  |  | 103% |  | 101% |
|  |  |  |  |  | Random access Main10 |  |  |  |  |  |  |
|  |  |  |  |  | Over VTM-4.0 with MTS 0 and MTSImplicit 1 (DCT-II only) |  |  |  |  |  |  |
|  | Y |  | U |  | V |  |  |  | EncT |  | DecT |
| Class A1 | -0.86% |  | -0.57% |  | -0.90% |  |  |  | 100% |  | 104% |
| Class A2 | -0.85% |  | -0.38% |  | -0.33% |  |  |  | 100% |  | 96% |
| Class B | -1.12% |  | -0.86% |  | -0.92% |  |  |  | 100% |  | 99% |
| Class C | -0.93% |  | -0.67% |  | -0.56% |  |  |  | 101% |  | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.96% |  | -0.66% |  | -0.70% |  |  |  | 100% |  | 100% |
| Class D | -0.82% |  | -0.69% |  | -0.75% |  |  |  | 100% |  | 101% |
| Class F | -0.71% |  | -0.65% |  | -0.63% |  |  |  | 100% |  | 99% |
|  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |  |
|  |  |  |  |  | Over VTM-4.0 with MTS 0 and MTSImplicit 1 (DCT-II only) |  |  |  |  |  |  |
|  | Y |  | U |  | V |  |  |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.68% |  | -0.66% |  | -0.05% |  |  |  | 101% |  | 99% |
| Class C | -0.53% |  | -0.50% |  | -0.25% |  |  |  | 100% |  | 101% |
| Class E | -0.33% |  | -1.21% |  | -0.04% |  |  |  | 100% |  | 99% |
| Overall | -0.54% |  | -0.74% |  | -0.12% |  |  |  | 100% |  | 99% |
| Class D | -0.52% |  | -0.46% |  | -0.14% |  |  |  | 101% |  | 102% |
| Class F | -0.27% |  | -1.15% |  | 0.21% |  |  |  | 99% |  | 99% |

Table 5.  Results of the proposed method under TC3

|  |  |  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Over VTM-4.0 |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  | 0.99% |  | 0.93% |  | 1.01% |  | 64% |  | 94% |  |
| Class A2 |  | 0.65% |  | 0.41% |  | 0.23% |  | 65% |  | 95% |  |
| Class B |  | 0.76% |  | 0.04% |  | 0.04% |  | 62% |  | 99% |  |
| Class C |  | 0.37% |  | -0.50% |  | -0.47% |  | 62% |  | 101% |  |
| Class E |  | 0.48% |  | -0.04% |  | -0.13% |  | 61% |  | 100% |  |
| Overall |  | 0.65% |  | 0.11% |  | 0.09% |  | 62% |  | 98% |  |
| Class D |  | 0.35% |  | -0.58% |  | -0.68% |  | 63% |  | 101% |  |
| Class F |  | 0.33% |  | -0.28% |  | -0.17% |  | 83% |  | 101% |  |
|  |  |  |  |  |  | Random access Main10 |  |  |  |  |  |
|  |  |  |  |  |  | Over VTM-4.0 |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  | 0.39% |  | 0.38% |  | 0.52% |  | 97% |  | 103% |  |
| Class A2 |  | 0.26% |  | 0.17% |  | 0.05% |  | 97% |  | 99% |  |
| Class B |  | 0.35% |  | 0.16% |  | -0.22% |  | 96% |  | 99% |  |
| Class C |  | 0.23% |  | -0.23% |  | -0.11% |  | 96% |  | 99% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | 0.31% |  | 0.10% |  | 0.01% |  | 97% |  | 100% |  |
| Class D |  | 0.10% |  | -0.45% |  | -0.49% |  | 96% |  | 100% |  |
| Class F |  | 0.30% |  | -0.12% |  | -0.16% |  | 96% |  | 99% |  |
|  |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |
|  |  |  |  |  |  | Over VTM-4.0 |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | 0.05% |  | -0.31% |  | -0.18% |  | 98% |  | 100% |  |
| Class C |  | 0.00% |  | 0.23% |  | -0.07% |  | 98% |  | 100% |  |
| Class E |  | 0.27% |  | -0.25% |  | 0.20% |  | 99% |  | 96% |  |
| Overall |  | 0.09% |  | -0.12% |  | -0.05% |  | 98% |  | 99% |  |
| Class D |  | -0.07% |  | 0.12% |  | -0.46% |  | 99% |  | 100% |  |
| Class F |  | -0.04% |  | -0.61% |  | -0.09% |  | 98% |  | 99% |  |

# Conclusions

This contribution proposes to unify the implicit MTS for regular intra mode with that for ISP mode. Compared to VTM4.0 with the implicit MTS of SAT for regular intra mode, the proposed method provides -0.24%, -0.04%, and -0.03% luma BD-rates for AI, RA, and LB, respectively.

# References

- S. De-Luxán-Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe, and T. Wiegand, “CE3: Intra sub-partitions coding mode (Tests 1.1.1 and 1.1.2),” Document of Joint Video Experts Team, JVET-M0102.

- VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- J. Lainema, “CE6: Shape adaptive transform selection (Test 3.1),” Document of Joint Video Experts Team, JVET-M0303.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).