| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0091-v1 |
| --- | --- |

| Title: | CE7-related: TB-level constraints on context-coded bins for coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Shih-Ta Hsiang, Zhi-Yi Lin, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, shih-ta.hsiang, zhi-yi.lin, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the VTM4.0 coefficient coding, a coefficient subblock-level maximum number of context-coded bins is imposed. The averaged maximum number of context-coded bins per 16 samples is increased by 28% compared with that in HEVC. In this contribution, it is proposed to use a transform block (TB) level constraint for maximum number of context-coded bins instead of coefficient subblock-level constraint. For a luma TB, the maximum allowed number of context-coded bin is equal to tb_width*tb_height*1.75. For a chroma TB, the maximum allowed number of context-coded bin is equal to tb_width*tb_height*1.25. The averaged maximum number of context-coded bins per 16 samples is only increased by 1.3% compared with that in HEVC. Simulation results reportedly show that the average luma BD-rates are 0.03%, 0.01%, and 0.07% in AI, RA, and LB cases, respectively.

# Introduction

In HEVC, for a 4x4 coefficient subblock, the maximum number of context-coded bins per 16 samples is 24 or 25, including 15 or 16 significant flags depending on whether the last significant coefficient is in the subblock, eight greater than one (gt1) flags, and one greater than two (gt2) flag. In VTM4.0, for a 4x4 coefficient coding subblock, at most 32 context-coded bins can be used. For a 2x2 chroma coefficient coding subblock, at most eight context-coded bins can be used. The averaged maximum number of context-coded bins per 16 samples in VTM3.0 is 32 bins. Compared with that in HEVC, the worst-case number of context-coded bins is increased by 28%. During the discussion in the last JVET meeting, it was commented that limiting context coded bins at 4x4 coefficient subblock level is not desirable. Imposing such a limit at a higher level may be better. In this contribution, it is proposed to use TB-level constraints. The proposed method is described in Section 2, and the results are reported in Section 3.

# Proposed TB-level constraints on context-coded bins

In this contribution, TB-level constraints for maximum number of context-coded bins are proposed. The area of the TB and the color component are used to derive the maximum allowed number of context-coded bin for a TB. For a luma TB, the maximum allowed number of context-coded bin is equal to tb_width*tb_height*1.75. For a chroma TB, the maximum allowed number of context-coded bin is equal to tb_width*tb_height*1.25. In transform coefficient coding, a variable, remBinsPass1, is first set to the maximum allowed number of context-coded bin and will be decreased by one when a context-coded bin is signalled. When the remBinsPass1 is larger than or equal to four, the significant flag, gt1 flag, parity bit flag, and greater than three (gt3) flag can be coded by using context-coded bins. When the remBinsPass1 is smaller than 4, the coefficients are coded with the syntax element of dec_abs_level by using Golomb-Rice code and bypass-coded bins. The remBinsPass1 is reset for every TB and not reset for non-first coefficient subblocks in decoding order. The transition of using context-coded bins for the significant flag, gt1 flag, parity bit flag, and gt3 flag to using bypass-coded bins for the rest coefficients only happens at most once per TB, unlike VTM4.0 that the might happen for each coefficient subblock. For a coefficient subblock, if the remBinsPass1 is smaller than 4, the entire coefficient subblock is coded by using bypass-coded bins.

When the subblock transform (SBT) is applied, only half or quarter of the coding unit (CU) coefficients are coded. The remBinsPass1 can be enlarged when the SBT is applied. For example, if the SBT is applied and the cu_sbt_quad_flag is equal to 1, only quarter of the CU coefficients are coded. The remBinsPass1 is multiplied by four. If the SBT is applied and the cu_sbt_quad_flag is equal to 0, only half of the CU coefficients are coded. The remBinsPass1 is multiplied by 2.

By using the proposed method, from the analysis, the averaged maximum number of context-coded bins per 16 samples is 25.33 bins and is only 1.3% larger than that in HEVC. The analysis is summarized in Table 1.

Table 1.  The averaged maximum number of context-coded bins per 16 samples in HEVC, VTM4.0, and the proposed method

| Number of maximum context-coded bins | 4x4 luma subblock | 4x4 chroma subblock | 2x2 chroma subblock | Maximum number of context-coded bins in one 64-sample luma block and two 16-sample chroma blocks | Averaged maximum number of context-coded bins per 16 samples |
| --- | --- | --- | --- | --- | --- |
| HEVC | 24 or 25 | 24 or 25 | N/A | 25*4 + 25*2 = 150 | 25.00 (100%) |
| VTM4.0 | 32 | 32 | 8 | 32*4 + 8*8 = 192 | 32.00 (128%) |
| Proposed | 28 | 20 | 5 | 28*4 + 5*8 = 152 | 25.33 (101.3%) |

# Simulation results

The proposed method is implemented on VTM4.0. The common test conditions (CTC) [1] are used for evaluation. The results of the proposed method in CTC QP (QP = 22, 27, 32, 37) are shown in Table 2. Simulation results reportedly show that the average luma BD-rates are 0.03%, 0.01%, and 0.01% in AI, RA, and LB cases, respectively. The results of the proposed method in low QP (QP = 2, 7, 12, 17) are shown in Table 3. Simulation results reportedly show that the average luma BD-rates are 0.08%, 0.xx%, and 0.08% in AI, RA, and LB cases, respectively.

Table 2.  Results of the proposed method in CTC (QP = 22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.08% | 0.02% | 101% | 105% |
| Class A2 | 0.04% | 0.19% | 0.13% | 102% | 102% |
| Class B | 0.02% | 0.04% | 0.11% | 102% | 104% |
| Class C | 0.02% | -0.04% | -0.01% | 101% | 99% |
| Class E | 0.05% | 0.10% | 0.06% | 100% | 99% |
| Overall | 0.03% | 0.06% | 0.06% | 101% | 102% |
| Class D | 0.04% | 0.16% | -0.12% | 99% | 100% |
| Class F | 0.00% | 0.13% | 0.05% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | 0.01% | 100% | 99% |
| Class A2 | 0.03% | -0.04% | 0.18% | 100% | 99% |
| Class B | 0.00% | -0.01% | 0.00% | 99% | 102% |
| Class C | 0.02% | -0.04% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.03% | 0.03% | 100% | 100% |
| Class D | 0.01% | -0.16% | -0.15% | 103% | 99% |
| Class F | -0.02% | 0.00% | 0.21% | 100% | 94% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.21% | -0.10% | 101% | 103% |
| Class C | -0.03% | 0.39% | 0.03% | 101% | 100% |
| Class E | 0.09% | 0.34% | 0.21% | 100% | 97% |
| Overall | 0.01% | 0.13% | 0.02% | 101% | 100% |
| Class D | 0.03% | 0.89% | 0.34% | 101% | 101% |
| Class F | -0.13% | 0.08% | -0.01% | 100% | 99% |

Table 3.  Results of the proposed method in low QP conditions (QP = 2, 7, 12, 17) (to be updated)

# Conclusions

In this contribution, it is proposed to use TB-level constraints for maximum number of context-coded bins instead of coefficient subblock-level. Simulation results reportedly show that the average luma BD-rates are 0.03%, 0.01%, and 0.07% in AI, RA, and LB cases, respectively. The averaged maximum number of context-coded bins per 16 samples is reduced from 128% to 101.3% compared with that in HEVC.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).