| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0092-v1 |
| --- | --- |

| Title: | CE7-related: Unified Rice parameter derivation of abs_remainder and dec_abs_level syntax elements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Mohammed Golam Sarwer, Olena Chubach, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {mohammed.sarwer, olena.chubach, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this proposal, a unified template-based Rice parameter derivation process is applied for coding both abs_remainder and dec_abs_level syntax elements. In addition to Rice parameter derivation, the syntax element abs_remainder and its associated sign value (i.e. coeff_sign_flag) are jointly coded using a joint syntax element abs_remainder_with_sign. In addition, in the proposed method, the decoding of coeff_sign_flag is moved to the second pass. Therefore, the number of coding passes in residual levels coding is reduced from three to two (i.e., pass 1: context coding; pass 2: bypass coding). It is reported that in AI condition this proposal can achieve -0.06%/-0.02%/-0.04% Y/Cb/Cr BD-rate for VTM4.0. In RA condition, the BD-rate is -0.05%/-0.10%/-0.12% for Y/Cb/Cr. Encoding time and decoding time of this proposal are similar to those of VTM4.0.

# Introduction

In VTM-4.0 [1], a three-pass encoding is performed to encode the residual levels of a transformed coefficient group (CG).

- Pass 1: Context coding sig_coeff_flag, abs_level_gt1_flag, par_level_flag, and abs_level_gt3_flag

- Pass 2: Bypass coding abs_remainder and dec_abs_level

- Pass 3: Coding coeff_sign_flag of nonzero coefficients

In pass 2, there are two syntax elements, called abs_remainder and dec_abs_level, which are coded using Rice-Golomb (RG) coding with adaptive value of Rice parameter. However, it should be noted that the Rice parameter derivation processes for abs_remainder and dec_abs_level are different. The abs_remainder uses HEVC-style Rice parameter derivation, whereas the dec_abs_level uses template-based Rice parameter derivation.

The pseudo code for Rice parameter derivation of abs_remainder is as follows.

ricePar = 0;
for( scanPos = firstSigPos; scanPos > firstPosMode2; scanPos-- )
{
  if( tcoeff >= 4 )
  {
    rem = decode abs_remainder;
    if( ricePar < 3 && rem > (3<<ricePar)-1 )
    {
      ricePar++;
    }
  }
}

The Rice parameter of dec_abs_level is derived based on the sum of absolute values (sumAll) of the decoded neighboring coefficients. A predefined look-up table (g_auiGoRiceParsCoeff[]) is used to define a Rice parameter (rice) for a given value sumAll, as follows.

g_auiGoRiceParsCoeff[32] = { 0, 0, 0, 0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 2, 2, 2, 2, 2, 2,                                                 2, 2, 2, 2, 2, 2, 2, 2, 3, 3, 3, 3 };
sumAll = clip2 (Sum of the previously decoded levels, 31);.
rice = g_auiGoRiceParsCoeff [sumAll];

# Proposed method

In this proposal, a unified template-based Rice parameter derivation process is suggested for coding both abs_remainder and dec_abs_level syntax elements. It is proposed that the Rice parameter depends on the transform_skip_flag and is derived as follows.

uMinRicePar = transform_skip_flag ? 1 : 0;
sumAll = clip2 (Sum of the previously decoded levels, 31);.
rice = g_auiGoRiceParsCoeff [sumAll];
rice = (isRemainder && rice < uMinRicePar) ? uMinRicePar : rice;

The look-up table g_auiGoRiceParsCoeff is the same as that in VTM4.0.

The variable isRemainder can take the following values: isRemainder=1 in case of abs_remainder coding, isRemainder=0 in case of dec_abs_level coding.

The syntax element abs_remainder and its associated sign value are mapped to a new syntax element abs_remainder_with_sign and jointly coded using Rice-Golomb coding with Rice parameter (rice) value computed as described above. At the encoder side, the syntax element abs_remainder_with_sign is derived from the absolute value (absCoeff) and sign of a level, as follows.

absCoeff = absolute value of the Coeff

sign = Coeff < 0 ? 0: 1
abs_remainder_with_sign = ((absCoeff – 4) >> 1) << 1 +sign
encode abs_remainder_with_sign

At the decoder side, the extraction of absolute value and sign from the syntax element abs_remainder_with_sign is computed as follows.

sign = abs_remainder_with_sign & 1
absCoeff = (abs_remainder_with_sign >> 1) << 1 + 4

In order to improve the throughput of the coefficient coding, in the proposed method it is also suggested to reduce the number of coding passes from three to two, by moving the decoding of coeff_sign_flag to the second pass. Thus, encoding of the residual levels of a transformed CG can be performed as follows.

- Pass 1: Context coding sig_coeff_flag, abs_level_gt1_flag, par_level_flag, and abs_level_gt3_flag

- Pass 2: Bypass coding abs_remainder_with_sign, dec_abs_level, and coeff_sign_flag

The pseudocode of the bypass coding (pass 2) of the proposed method at the decoder side is as follows.

uMinRicePar = transform_skip_flag ? 1 : 0;
	for(scanPos = firstSigPos; scanPos >= minSubPos; scanPos-- )
	{
		isRemainder = (scanPos > firstPosMode2) ? 1 : 0;
		uMinLevel = isRemainder ? 4 : 0;
		TCoeff& tcoeff = coeff[ cctx.blockPos( scanPos ) ];
		sign = 0;
		bCombinedSign = (isRemainder && tcoeff >= uMinLevel ) ? 1 : 0;
		if( tcoeff >= uMinLevel )
		{
			sumAll = cctx.templateAbsSum(scanPos, coeff);
			rice = g_auiGoRiceParsCoeff [sumAll];
			rice = (isRemainder && rice < uMinRicePar) ? uMinRicePar : rice;
			pos0 = isRemainder ? 0 : g_auiGoRicePosCoeff0[std::max(0, state - 1)][sumAll];
			rem = decode();
			sign = isRemainder ? (rem & 1) : 1;
			rem = ( rem == pos0 ? 0 : rem < pos0 ? rem+1 : rem );
			tcoeff += (rem >> isRemainder) << isRemainder;
		}
		if( tcoeff && !bCombinedSign)
			sign =decode coeff_sign_flag;
		tcoeff = (sign == 0) ? -tcoeff : tcoeff;
	}

# Simulation results

The proposed method is implemented on top of VTM-4.0. Both common test conditions (CTC) [2] and low quantization parameter (QP) settings [3] are tested. In case of low QP settings, only first 100 frames of the sequences are encoded at four QP values (2, 7, 12, and 17). Table 1 shows the results of the proposed method under CTC described in [2]. Table 2 shows the results of low QP settings described in [3].

Table1.  Results of proposed method over VTM 4.0 (CTC) (To be updated)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.07% | -0.06% | 102% | 96% |
| Class A2 | -0.05% | 0.08% | -0.02% | 103% | 98% |
| Class B | -0.07% | -0.08% | 0.02% | 102% | 101% |
| Class C | -0.04% | -0.09% | -0.04% | 102% | 102% |
| Class E | -0.09% | -0.02% | -0.13% | 102% | 103% |
| Overall | -0.06% | -0.02% | -0.04% | 102% | 100% |
| Class D | -0.07% | -0.01% | -0.10% | 101% | 102% |
| Class F | 0.02% | -0.05% | 0.03% | 102% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.18% | -0.06% | 101% | 101% |
| Class A2 | -0.04% | -0.04% | 0.09% | 101% | 100% |
| Class B | -0.05% | -0.03% | -0.22% | 101% | 100% |
| Class C | -0.05% | -0.19% | -0.19% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.10% | -0.12% | 101% | 100% |
| Class D | -0.04% | -0.20% | -0.31% | 101% | 100% |
| Class F | 0.07% | -0.20% | -0.08% | 101% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.84% | -0.35% | -1.18% | 8% | 43% |
| Class C | -0.05% | 0.15% | 0.00% | 101% | 101% |
| Class E | -0.05% | 0.18% | 1.60% | 100% | 95% |
| Overall | 0.32% | -0.05% | -0.09% | 35% | 69% |
| Class D | -0.05% | 0.54% | 0.29% | 102% | 101% |
| Class F | -0.08% | -0.08% | 0.10% | 101% | 101% |

Table2.  Results of proposed method over VTM 4.0 (low QP settings) (To be updated)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.02% | 0.00% | 104% | 102% |
| Class A2 | -0.10% | -0.08% | -0.07% | 104% | 104% |
| Class B | -0.09% | -0.07% | -0.07% | 104% | 102% |
| Class C | -0.02% | -0.10% | -0.08% | 105% | 101% |
| Class E | -0.11% | -0.04% | -0.03% | 105% | 101% |
| Overall | -0.07% | -0.07% | -0.05% | 104% | 102% |
| Class D | 0.03% | -0.01% | 0.04% | 103% | 101% |
| Class F | -0.05% | 0.01% | -0.01% | 103% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | 0% | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | 0% | #DIV/0! |
| Class B | -0.04% | -0.03% | -0.03% | 102% | 103% |
| Class C | -0.02% | -0.05% | -0.05% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | 9% | #DIV/0! |
| Class D | 0.00% | 0.00% | -0.14% | 102% | 102% |
| Class F | -0.03% | -0.08% | -0.01% | 102% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.01% | -0.01% | 102% | 102% |
| Class C | -0.01% | 0.00% | 0.01% | 102% | 101% |
| Class E | -0.01% | -0.05% | 0.01% | 102% | 101% |
| Overall | -0.02% | -0.02% | 0.00% | 102% | 101% |
| Class D | -0.03% | -0.06% | -0.02% | 102% | 101% |
| Class F | 0.07% | 0.13% | 0.18% | 101% | 100% |

# Conclusions

This proposal applies a unified template-based Rice parameter derivation process to abs_remainder and dec_abs_level. In addition, it allows combined coding of the abs_remainder and coeff_sign_flag, which reduces the number of coding passes from three to two. Compared to VTM4.0, this proposal reportedly achieves following BD-rates under CTC.

AI: -0.06%/-0.02%/-0.04% for Y/Cb/Cr
RA: -0.05%/-0.10%/-0.12% for Y/Cb/Cr
LB: -0.04%/0.04%/0.24% for Y/Cb/Cr

# References

[1] VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

[2] F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

[3] H. Schwarz, M. Coban, and C. Auyeung, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” Document of Joint Video Experts Team, JVET-M1027.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).