| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0094-v1 |
| --- | --- |

| Title: | CE8-related: Context modelling of transform skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Olena Chubach, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {mohammed.sarwer, olena.chubach, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this proposal a context modelling method for coefficient coding of the transform skip mode is introduced. It is proposed to derive the context models for coefficient coding from three previously decoded neighboring coefficients located at the right, bottom, and bottom-right positions of the current to-be-decoded coefficient. A separate context set is defined for transform skip mode. It is reported that under RA configuration this proposal can achieve -0.04%/0.00%/-0.08% Y/Cb/Cr BD-rate for VTM4.0 under common test conditions (CTC). Encoding time and decoding time of this proposal are similar to those of VTM4.0. BD-rates of class F and TGM sequences as follows.

Test-1: Reducing the number of neighboring coefficients for transform skip mode (total number of context variables kept the same as that in VTM4.0, i.e., 232)

[Class F sequences]
AI: -0.59%/-0.38%/-0.39% for Y/Cb/Cr
RA: -0.56%/-0.41%/-0.42% for Y/Cb/Cr
LB: -0.xx%/0.xx%/0.xx% for Y/Cb/Cr

[TGM sequences]
AI: -1.01%/-0.66%/-0.67% for Y/Cb/Cr
RA: -0.68%/-0.32%/-0.39% for Y/Cb/Cr
LB: -0.xx%/-0.xx%/-0.xx% for Y/Cb/Cr

Test-2: Test-1 plus separate context sets for transform skip mode for sig_coeff_flag, par_level_flag, abs_level_gt1_flag, and abs_level_gt3_flag (total number of context variables = 265)

[Class F sequences]
AI: -1.19%/-0.92%/-0.90% for Y/Cb/Cr
RA: -1.04%/-0.70%/-0.72% for Y/Cb/Cr
LB: -0.xx%/0.xx%/0.xx% for Y/Cb/Cr

[TGM sequences]
AI: -1.83%/-1.25%/-1.25% for Y/Cb/Cr
RA: -1.24%/-0.82%/-0.77% for Y/Cb/Cr
LB: -0.xx%/-0.xx%/-0.xx% for Y/Cb/Cr

Test-3: Test-2 plus separate context sets for last_sig_coeff_x_prefix and last_sig_coeff_y_prefix (total number of context variables = 295

[Class F sequences]
AI: -1.41%/-1.01%/-1.23% for Y/Cb/Cr
RA: -1.19%/-0.91%/-0.93% for Y/Cb/Cr
LB: -0.xx%/0.xx%/0.xx% for Y/Cb/Cr

[TGM sequences]
AI: -2.05%/-1.34%/-1.36% for Y/Cb/Cr
RA: -1.40%/-0.81%/-0.91% for Y/Cb/Cr
LB: -0.xx%/-0.xx%/-0.xx% for Y/Cb/Cr

# Introduction

In VTM4.0 [1], a local template containing five previously decoded neighboring coefficients is used to derive the context indices of the current decoding coefficient, as shown in Figure 1.

Figure 1.  The template showing the locations of neighbors for context modelling and Rice parameter derivation in VTM4.0

When deriving the context indices for sig_coeff_flag, par_level_flag, abs_level_gt1_flag, and abs_level_gt3_flag, two variables, locSumAbsPass1 and locNumSig, are computed. One more variable, locSumAbs, is computed when Rice parameter is derived for the dec_abs_level syntax element.

The definitions of these three variables are as follows.

- locSumAbsPass1: sum of decoded neighboring coefficients after the first pass of decoding

- locNumSig : number of nonzero neighboring coefficients

- locSumAbs: sum of decoded neighboring coefficients

In VTM4.0 the same set of neighbors shown in Figure 1 is used for context modelling for both transformed coefficients and transform skip mode. However, the distribution of quantized levels in case of the transform skip mode differs from that of transformed coefficients. This proposal presents a modified approach to context modelling of transform skip mode.

# Proposed methods

## Reduced number of neighbors:

It is proposed to reduce the number of neighbors in the template for context modelling to three, i.e., use only neighbors at the right, bottom, and bottom-right positions of the current to-be-decoded coefficient, for context modelling of transform skip mode, as shown in Figure 2. The same set of 3 neighbors (as shown in Figure 2) is also used to derive Rice parameter of dec_abs_level syntax. It should be noted that for the case of transformed residual coding, five neighbours are still used for both context modelling and Rice parameter adaptation, as it is shown in Figure 1.

Figure 2.  The proposed template showing the locations of neighbors for context modelling and Rice parameter derivation

The pseudo code for computation of the locSumAbsPass1, locNumSig and locSumAbs according to the propsoed method is presented in the following.

locNumSig = 0
locSumAbsPass1 = 0
if( xC < (1 << log2TbWidth) − 1 ) {
	locNumSig += sig_coeff_flag[ xC + 1 ][ yC ]
	locSumAbsPass1 += AbsLevelPass1[ xC + 1 ][ yC ]
	if( xC < (1 << log2TbWidth) − 2 && !transform_skip_flag) {
		locNumSig += sig_coeff_flag[ xC + 2 ][ yC ]
		locSumAbsPass1 += AbsLevelPass1[ xC + 2 ][ yC ]
	}
	if( yC < (1 << log2TbHeight) − 1 ) {
		locNumSig += sig_coeff_flag[ xC + 1 ][ yC + 1 ] 
		locSumAbsPass1 += AbsLevelPass1[ xC + 1 ][ yC + 1 ]
	}
}
if( yC < (1 << log2TbHeight) − 1 ) {
	locNumSig += sig_coeff_flag[ xC ][ yC + 1 ]
	locSumAbsPass1 += AbsLevelPass1[ xC ][ yC + 1 ]
	if( yC < (1 << log2TbHeight) – 2 && !transform_skip_flag) {
		locNumSig += sig_coeff_flag[ xC ][ yC + 2 ]
		locSumAbsPass1 += AbsLevelPass1[ xC ][ yC + 2 ]
	}
}
locSumAbs = 0
if( xC < (1 << log2TbWidth) − 1 ) {
	locSumAbs += AbsLevel[ xC + 1 ][ yC ] − sig_coeff_flag[ xC + 1 ][ yC ]
	if( xC < (1 << log2TbWidth) − 2 && !transform_skip_flag)
		locSumAbs += AbsLevel[ xC + 2 ][ yC ] − sig_coeff_flag[ xC + 2 ][ yC ]
	if( yC < (1 << log2TbHeight) − 1 )
		locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] − sig_coeff_flag[ xC + 1 ][ yC + 1 ]
}
if( yC < (1 << log2TbHeight) − 1 ) {
	locSumAbs += AbsLevel[ xC ][ yC + 1 ] − sig_coeff_flag[ xC ][ yC + 1 ]
	if( yC < (1 << log2TbHeight) − 2 && !transform_skip_flag)
		locSumAbs += AbsLevelPass1 [ xC ][ yC + 2 ] − sig_coeff_flag[ xC ][ yC + 2 ]
}
if( locSumAbs > 31 )
	locSumAbs = 31

## Separate context sets for transform skip mode for sig_coeff_flag, par_level_flag, abs_level_gt1_flag, and abs_level_gt3_flag

In VTM4.0, the numbers of contexts used for sig_coeff_flag, par_level_flag, abs_level_gt1_flag, and abs_level_gt3_flag are equal to 90, 32, 32, and 32, respectively. In the proposed method, the position independent separate context sets are used for transform skip mode. The numbers of contexts used for coding sig_coeff_flag, par_level_flag, abs_level_gt1_flag, and abs_level_gt3_flag are increased to 108, 37, 37 and 37, respectively.

The derivation process of the context index of the syntax element sig_coeff_flag is as follows:

d = xC + yC
	In case of luma:
		offset = (transform_skip_flag) ? 18 :( d < 2  ?  12  :  ( d < 5  ?  6  :  0 ) )
		ctxInc = 24 * Max( 0, QState − 1) + Min( locSumAbsPass1, 5 ) + offset
	In case of chroma
		offset  = ( d < 2  ?  6  :  0 )
		ctxInc = 72 + 12 * Max( 0, QState − 1) + Min( locSumAbsPass1, 5 ) + offset

The derivation process of the context index of the syntax elements par_level_flag, abs_level_gt1_flag, and abs_level_gt3_flag are given below:

ctxOffset = Min( locSumAbsPass1 − locNumSig, 4 )
d = xC + yC
If last coefficient:
	ctxInc =  (cIdx  = =  0  ? (transform_skip_flag ? 21 : 0) :  26)
Otherwise, if luma:
	offset = (transform_skip_flag) ? 21 : (1 +( d  = =  0  ?  15  :  ( d < 3  ?  10  :  ( d < 10  ?  5  :  0 ) ) ))
	ctxInc = offset + ctxOffset
Otherwise, if chroma:
	ctxInc = 27 + ctxOffset + ( d  = =  0  ?  5  :  0 )

## Separate context sets for last_sig_coeff_x_prefix and last_sig_coeff_y_prefix

In the proposed method, a separate context set is used for signalling the last_sig_coeff_x_prefix and last_sig_coeff_y_prefix for transform skip mode. It is proposed to increase the number of contexts for last coefficient coding to 76 (i.e. 35 for luma last_sig_coeff_x_prefix, 35 for luma last_sig_coeff_y_prefix, 3 for chroma last_sig_coeff_x_prefix, and 3 for chroma luma last_sig_coeff_y), whereas the number of contexts for in VTM4.0 is equal to 46 (i.e. 20 for luma last_sig_coeff_x_prefix, 20 for luma last_sig_coeff_y_prefix, 3 for chroma last_sig_coeff_x_prefix, and 3 for chroma luma last_sig_coeff_y).

The derivation process of the context index of the syntax element last_sig_coeff_y_prefix and last_sig_coeff_y_prefix is as follows:

The variable log2TbSize is derived as follows:

If the syntax element to be parsed is last_sig_coeff_x_prefix, log2TbSize is set equal to log2TbWidth.

Otherwise (the syntax element to be parsed is last_sig_coeff_y_prefix), log2TbSize is set equal to log2TbHeight.

The variables ctxOffset and ctxShift are derived as follows:

In case of luma:

ctxOffset = (transform_skip_flag) ? 20 : (3 * ( log2TbSize − 2 ) + ( ( log2TbSize − 1 )  >>  2 ))

ctxShift = ( log2TbSize + 1 )  >>  2.

In case of chroma:

ctxOffset is set equal to 35

ctxShift is set equal to Clip3( 0, 2, 2log2TbSize >> 3 ).

ctxInc =  ( binIdx  >>  ctxShift ) + ctxOffset

# Simulation results

The proposed methods are implemented on top of VTM4.0. Both CTC [2] and low quantization parameter (QP) settings [3] are tested. In case of low QP settings, only first 100 frames of the sequences are encoded at four QP values (2, 7, 12, and 17). Following three tests are conducted.

- Test 1: In this test only the number of neighbors for transform skip mode is reduced. Same context sets are used for both transform and transform skipped TU. Therefore, the number of contexts are exactly the same as that in VTM4.0 (i.e, 232).

- Test 2: In this test, separate context sets for transform skip mode for sig_coeff_flag, par_level_flag, abs_level_gt1_flag, and abs_level_gt3_flag are implemented on top of test 1. The number of contexts is 265 whereas in VTM4.0, the number of contexts is 232.

- Test 3: In this test, separate context sets for last_sig_coeff_x_prefix and last_sig_coeff_y_prefix: are implemented on top of test 2. The number of contexts is 295 whereas in VTM4.0, the number of contexts is 232.

Table 1, Table 2 and Table 3 shows the simulation results of test 1, test 2, and test3, respectively.

# Conclusions

Compared to VTM4.0, this proposal reportedly achieves BD-rates of -0.04%/-0.00%/-0.08% for Y/Cb/Cr, respectively, under RA. In case of TGM sequences under RA, BD-rates are -1.40%/-0.81%/-0.91% for Y/Cb/Cr, respectively.

# References

[1] VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

[2] F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

[3] X. Xu, Y-H Chao, Y-C Sun, and J. Xu , “ Description of Core Experiment 8 (CE8): Screen content coding tools,” Document of Joint Video Experts Team, JVET-M1028.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table1 (a).  Results of test 1 over VTM 4.0 (CTC) (LB results to be updated)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.02% | -0.02% | 100% | 99% |
| Class A2 | 0.01% | 0.02% | 0.00% | 100% | 99% |
| Class B | -0.01% | 0.01% | 0.02% | 100% | 99% |
| Class C | -0.06% | 0.07% | 0.04% | 101% | 98% |
| Class E | 0.01% | 0.03% | 0.02% | 101% | 99% |
| Overall | -0.02% | 0.02% | 0.02% | 100% | 99% |
| Class D | -0.05% | -0.08% | -0.04% | 103% | 98% |
| Class F | -0.59% | -0.38% | -0.39% | 100% | 98% |
| TGM | -1.01% | -0.66% | -0.67% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.20% | 0.04% | 100% | 100% |
| Class A2 | -0.02% | -0.06% | 0.01% | 100% | 99% |
| Class B | 0.01% | -0.07% | -0.06% | 100% | 95% |
| Class C | -0.05% | 0.27% | -0.05% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.00% | -0.02% | 100% | 98% |
| Class D | -0.04% | -0.12% | 0.10% | 100% | 97% |
| Class F | -0.56% | -0.41% | -0.42% | 100% | 95% |
| TGM | -0.68% | -0.32% | -0.39% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 4% | 37% |
| Class C | #VALUE! | #VALUE! | #VALUE! | 0% | 12% |
| Class E | #VALUE! | #VALUE! | #VALUE! | 0% | 11% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0% | 19% |
| Class D | #VALUE! | #VALUE! | #VALUE! | 1% | 55% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% | 30% |
| TGM | #VALUE! | #VALUE! | #VALUE! | 0% | 10% |

Table1 (b).  Results of test 1 over VTM 4.0 (low QP setting)

To be added

Table2 (a).  Results of test 2 over VTM 4.0 (CTC) (LB results to be updated)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.00% | 0.03% | 102% | 101% |
| Class A2 | 0.00% | 0.01% | -0.02% | 101% | 101% |
| Class B | -0.01% | -0.04% | 0.02% | 101% | 100% |
| Class C | -0.11% | 0.02% | -0.02% | 101% | 100% |
| Class E | 0.01% | -0.12% | 0.06% | 102% | 102% |
| Overall | -0.03% | -0.02% | 0.01% | 101% | 101% |
| Class D | -0.10% | -0.08% | 0.05% | 102% | 99% |
| Class F | -1.19% | -0.92% | -0.90% | 101% | 102% |
| TGM | -1.83% | -1.25% | -1.25% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.03% | 0.04% | 100% | 100% |
| Class A2 | 0.00% | 0.03% | 0.01% | 101% | 100% |
| Class B | 0.00% | -0.10% | -0.25% | 101% | 100% |
| Class C | -0.10% | 0.18% | 0.14% | 101% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.01% | -0.04% | 101% | 102% |
| Class D | -0.03% | 0.19% | 0.26% | 101% | 100% |
| Class F | -1.04% | -0.70% | -0.72% | 100% | 99% |
| TGM | -1.24% | -0.82% | -0.77% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 4% | 37% |
| Class C | #VALUE! | #VALUE! | #VALUE! | 0% | 12% |
| Class E | #VALUE! | #VALUE! | #VALUE! | 0% | 11% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0% | 19% |
| Class D | #VALUE! | #VALUE! | #VALUE! | 0% | 44% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% | 26% |
| TGM | #VALUE! | #VALUE! | #VALUE! | 0% | 10% |

Table2 (b).  Results of test 2 over VTM 4.0 (low QP setting)

To be added

Table3 (a).  Results of test 3 over VTM 4.0 (CTC) (LB results to be updated)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.01% | 0.04% | 102% | 101% |
| Class A2 | 0.01% | -0.02% | -0.01% | 102% | 100% |
| Class B | -0.02% | -0.06% | 0.01% | 101% | 99% |
| Class C | -0.19% | -0.10% | -0.08% | 101% | 99% |
| Class E | -0.01% | 0.00% | -0.02% | 101% | 99% |
| Overall | -0.06% | -0.04% | -0.01% | 102% | 100% |
| Class D | -0.10% | -0.12% | -0.03% | 102% | 98% |
| Class F | -1.41% | -1.01% | -1.23% | 101% | 98% |
| TGM | -2.05% | -1.34% | -1.36% | 101% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.06% | -0.02% | 100% | 99% |
| Class A2 | -0.01% | -0.10% | 0.01% | 101% | 99% |
| Class B | -0.01% | -0.06% | -0.07% | 101% | 95% |
| Class C | -0.13% | 0.21% | -0.22% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.00% | -0.08% | 101% | 98% |
| Class D | -0.04% | 0.04% | 0.34% | 101% | 97% |
| Class F | -1.19% | -0.91% | -0.93% | 100% | 95% |
| TGM | -1.40% | -0.81% | -0.91% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 32% | 71% |
| Class C | -0.07% | -0.38% | -0.05% | 101% | 100% |
| Class E | 0.08% | -1.05% | 1.05% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 63% | 87% |
| Class D | -0.05% | 0.45% | 0.11% | 101% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |
| TGM | #VALUE! | #VALUE! | #VALUE! | 101% | 100% |

Table3 (b).  Results of test 3 over VTM 4.0 (low QP setting) (RA & LB results to be updated)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.11% | -0.14% | 102% | 99% |
| Class A2 | -0.01% | 0.00% | 0.00% | 102% | 100% |
| Class B | -0.03% | -0.02% | 0.00% | 102% | 100% |
| Class C | -0.03% | -0.02% | -0.03% | 102% | 98% |
| Class E | -0.05% | -0.01% | -0.01% | 101% | 100% |
| Overall | -0.05% | -0.03% | -0.03% | 102% | 99% |
| Class D | 0.01% | 0.01% | 0.09% | 103% | 99% |
| Class F | -0.77% | -0.68% | -0.67% | 101% | 99% |
| TGM | -1.62% | -1.18% | -1.15% | 101% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | 33% | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | 34% | #DIV/0! |
| Class B | -0.01% | -0.03% | -0.03% | 101% | 100% |
| Class C | 0.06% | 0.03% | 0.04% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | 65% | #DIV/0! |
| Class D | 0.11% | -0.11% | 0.09% | 101% | 99% |
| Class F | -0.88% | -0.67% | -0.74% | 101% | 99% |
| TGM | -0.58% | -0.77% | -0.77% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 57% | 83% |
| Class C | 0.05% | -0.01% | -0.11% | 101% | 101% |
| Class E | -0.02% | -0.06% | -0.02% | 102% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 80% | 93% |
| Class D | 0.12% | -0.06% | 0.03% | 100% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 102% | 100% |
| TGM | -1.51% | -0.77% | -0.84% | 101% | 100% |