| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0095-v1 |
| --- | --- |

| Title: | CE8-related: Unified method for coding BVD and MVD |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a unified method for coding motion vector differences (MVDs) and block vector differences (BVDs) for coding screen contents. The proposed method corresponds to the same binarization as that of VVC Draft 4 while increasing the number of context-coded bins for coding the prefix part. Each vector component can use up to five context-coded bins. The proposed method reportedly provides -1.41%, -1.20% and -0.99% luma BD-rates for the AI, RA, and LB settings, respectively, for Class SCC under the CE8 common test conditions (CTCs) while increasing the number of context variables by four.

# Introduction

Screen content tends to result in large magnitudes of vector differences from predicting motion vectors and block vectors. This contribution proposes a unified method for coding MVDs and BVDs.

# Proposed method

The proposed method represents the absolute value, | d |, of each component of the block difference vector in a binary form by its most significant bit (MSB) index followed by its refinement bits. The MSB index for syntax value 0 is assigned to -1. The MSB index plus one, referred to as msb_plus_one, is binarized by the unary code. The remaining refinement bits are represented in the fixed-length binary code with the codeword length determined by the MSB index. Incidentally, this binarization scheme leads to exactly the same bin string as that of the binarization scheme employed in the current HEVC and VVC Draft 4 for coding motion vector difference, as compared in Table 1.

Table 1.  The bin strings generated by the proposed method and HEVC mvd coding

|  | Proposed Method |  | HEVC mvd_coding() |  |  |
| --- | --- | --- | --- | --- | --- |
| ¦ d ¦ | msb_plus_one | refinement bins | TU prefix | EG1 prefix | EG suffix |
| 0 | 0 | - | 0 | - | - |
| 1 | 10 | - | 10 | - | - |
| 2 | 110 | 0 | 11 | 0 | 0 |
| 3 | 110 | 1 | 11 | 0 | 1 |
| 4 | 1110 | 00 | 11 | 10 | 00 |
| 5 | 1110 | 01 | 11 | 10 | 01 |
| 6 | 1110 | 10 | 11 | 10 | 10 |
| 7 | 1110 | 11 | 11 | 10 | 11 |

The resulting bin stings are coded by context-based adaptive binary arithmetic coding (CABAC). The three new syntax elements abs_mvd_msb_idx_p1, abs_mvd_refinement_bits, and mvd_sign_flag are introduced for representing msb_plus_one, refinement bits and sign flag of a block vector component difference, respectively. The bin string for abs_mvd_msb_idx_p1 is entropy coded in the CABAC mode if the bin index is smaller than a threshold and in the bypass mode, otherwise. The context selection is conditioned on bin index and vector component index. Table 2 provides the mapping table for context selection with the bypass threshold equal to four, leading to an increase in context variables by four.

Table 2.  Context assignment for entropy coding abs_mvd_msb_idx_p1 of the MVD components

| bin index | 0 | 1 | 2 | 3 | 4 | > 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Horizontal component | 5 | 1 | 2 | 3 | 4 | bypass |
| Vertical component | 0 | 1 | 2 | 3 | 4 | bypass |

# Experimental results

The proposed method was integrated into the VTM4.0 reference software [2]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified VTM4.0 reference software. Table 3 summarizes the experimental results conducted under the SCC CTCs defined for CE8 in [3]. Please note the run time results are not reliable. The detailed simulation results can be found in the attached spreadsheets.

Table 3: Summary of the average BD bitrate results using the proposed method (to be updated)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.02% | 0.08% | -0.06% | 142% | 122% |
| Class E | -0.04% | -0.12% | -0.07% | 142% | 126% |
| Overall |  |  |  |  |  |
| Class D | 0.01% | -0.19% | 0.14% | 143% | 128% |
| Class F (optional) | -0.47% | -0.51% | -0.60% | 140% | 123% |
| SCC | -1.41% | -1.48% | -1.48% | 139% | 121% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.06% | 0.03% | -0.13% | 140% | 128% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.03% | -0.23% | -0.01% | 142% | 132% |
| Class F (optional) | -0.41% | -0.57% | -0.68% | 138% | 135% |
| SCC | -1.20% | -1.13% | -1.21% | 138% | 132% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.06% | -0.01% | 0.25% | 69% | 104% |
| Class E | -0.08% | -1.43% | 0.91% | 138% | 128% |
| Overall |  |  |  |  |  |
| Class D | -0.11% | 1.24% | 0.43% | 141% | 132% |
| Class F (optional) | -0.41% | -0.58% | -0.72% | 63% | 96% |
| SCC | -0.99% | -0.98% | -0.99% | 137% | 128% |

# Conclusions

This contribution proposes a unified method for coding MVDs and BVDs for coding screen contents. The proposed method reportedly achieves -1.41%, -1.2% and -0.99% luma BD-rates for the AI, RA, and LB settings, respectively, for Class SCC under the CE8 CTCs while increasing the number of context variables by four.

# References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 4),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1001, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

- VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- X. Xu, Y.-H. Chao, Y.-C. Sun, and J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1028, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

MediaTek Inc.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).