| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0097-v1 |
| --- | --- |

| Title: | CE9-related: Simplification of cascading DMVR and BDOF processes |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Tzu-Der Chuang, Chun-Chia Chen, Chen-Yen Lai, Zhi-Yi Lin, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chingyeh.chen, peter.chuang, chunchia.chen, jenny.lai, zhi-yi.lin, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VTM4.0, the critical path of inter prediction is the cascaded operations of decoder-side motion vector refinement (DMVR) and bi-directional optical flow (BDOF). In DMVR and BDOF, sum of absolute differences (SAD) between L0 and L1 predictors are calculated several times, but the SADs are not reused between DMVR and BDOF because of different fractional sample interpolation processes, different predictor bitdepths, and other causes. In order to reduce the critical path of inter prediction and hardware cost, this contribution proposes to unify the SAD operations in DMVR and BDOF and reuse the SAD in DMVR for the early termination in BDOF. It is reported that BD-rate and runtime changes are negligible. It is asserted that the proposed method is effective for hardware cost reduction.

# Introduction

In VTM4.0, DMVR is adopted to refine motion vectors (MVs) at the decoder for a coding unit (CU) when the CU is coded with regular merge/skip mode and bi-prediction, one reference picture is before the current picture and the other reference picture is after the current picture in display order, and the temporal distance between the current picture and one reference picture is equal to that between the current picture and the other reference picture. DMVR refines MVs by minimizing the SAD between L0 and L1 predictors. When DMVR is applied, one CU is divided into several independently processed subblocks with size equal to min(cuWidth,16) × min(cuHeight,16). For each subblock, integer △MV search around the initial MV is first performed using SAD, and then fractional △MV derivation is performed to obtain the final MVs. In the integer △MV search, fractional samples are generated using bilinear interpolation.

BDOF refines the luma sample predictors for a CU when the CU is coded with bi-prediction, one reference picture is before the current picture and the other reference picture is after the current picture in display order, and the bi-prediction with CU weights (BiCW) selects equal weights for L0 and L1 predictors. The eight-tap interpolation is used for generating the initial L0 and L1 predictors according to the input MVs (e.g., final MVs of DMVR in case of enabling DMVR). Next, a two-level early termination process is performed. The first early termination is at CU level, and the second early termination is at 4×4 block level. At each level, the SAD between L0 and L1 predictors in one unit, which is one CU in the first level or one 4×4 block in the second level, is calculated first. If the SAD is small than one threshold, then BDOF is not applied to the current unit. Otherwise, BDOF parameters are derived and used to generate the final luma sample predictors.

When the CU is coded with regular merge/skip mode, one reference picture is before the current picture and the other reference picture is after the current picture in display order, the temporal distance between the current picture and one reference picture is equal to that between the current picture and the other reference picture, and BiCW selects equal weights for L0 and L1 predictors, DMVR and BDOF are both applied. The flow of cascading DMVR and BDOF processes is shown in Fig. 1 (a). This is the critical path of inter prediction in VTM4.0.

(a)                         (b)

Figure 1.  The flow of cascading DMVR and BDOF processes: (a) VTM4.0, (b) the proposed method

# Proposed method

As shown in Fig. 1(a), the SAD operations are required in DMVR and BDOF processes. In order to reduce the latency and operations in this critical path, it is proposed to unify the SAD operations in DMVR and BDOF and reuse the SAD in DMVR for the early termination in BDOF.

First, the two-level early termination is replaced with a one-level early termination in BDOF. The early termination units are modified to be the same as the subblocks of the integer △MV search in DMVR. Second, the subsampling pattern used in the SAD operations of the integer △MV search in DMVR is also applied to BDOF. Finally, the calculated SAD in DMVR in case of enabling DMVR is reused for the early termination in BDOF. The proposed method is shown in Fig. 1(b).

# Experimental results

The proposed method has been implemented on VTM4.0 [1]. The simulations were performed following the JVET common test conditions (CTC) [2]. Table 1 shows the simulation results of the proposed method.

Table 1.  Simulation results of the proposed method

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.02% | 101% | 102% |
| Class A2 | -0.03% | 0.01% | 0.05% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | -0.01% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | 0.00% | 100% | 101% |
| Class D | -0.01% | -0.03% | -0.09% | 101% | 100% |
| Class F | -0.02% | -0.01% | -0.02% | 101% | 99% |

# Conclusions

This contribution proposed to unify the SAD operations in DMVR and BDOF and reuse the SAD in DMVR for the early termination in BDOF, in order to reduce the required operations when cascading DMVR and BDOF processes. There are almost no impacts on BD-rate, encoding time, and decoding time. The hardware cost can be effectively reduced.

# References

- VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).