| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0098-v1 |
| --- | --- |

| Title: | CE11-2.1: Deblocking for 4xN, Nx4 and 8xN and Nx8 block boundaries not aligned with 8x8 grids |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Jack Enhorn, Zhi Zhang, Rickard Sjöberg, | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
|  | Anand Meher Kotra, Jianle Chen, Semih Esenlik, Biao Wang, Han Gao, Yin Zhao, | Tel: Email: | Anand.meher.kotra@huawei.com jianle.chen@huawei.com |
|  | Chia-Ming Tsai, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | Ericsson, Huawei Technologies. Co., Ltd, MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution document reports the results of CE11-2.1, which is a joint proposal from Ericsson, Huawei and MediaTek Inc. VTM-4.0 does not apply deblocking filter for some edges belonging to the blocks whose size is Nx4, 4xN blocks and also for some blocks whose size is Nx8, 8xN where N value can be upto 64 samples. This test applies deblocking on 4x4 grids to not only allow deblocking of Nx4, 4xN, Nx8 and 8xN blocks aligned with the current 8x8 sample grid but also for the Nx4, 4xN, Nx8, 8xN blocks that are not aligned on the current 8x8 sample grid. Moreover, the sub-block edges of sub-block transform (SBT) and Intra subpartition tool (ISP) which align with 4x4 sample grid are also deblocked. And, the motion vector (MV) component difference threshold is also reduced from 1 luma sample to a quarter of a luma sample to more carefully include deblocking for blocks with small differences in motion. The number of samples to read and modify during deblocking filtering is limited to allow for parallel friendly processing. For a given edge, if at least one of the blocks sharing the edge has a size of 4 samples (orthogonally) then the VTM-4.0 weak deblocking filter is chosen to be applied. Furthermore, the weak filter is constrained to only modify one sample on either side of the edge.

Subjective analysis shows that when compared to the Anchor (VTM-4.0), the proposed test improves the subjective quality of sequences.

Objective results are reported as follows:

Over VTM-4.0 Anchor with ALF OFF (AI, RA, LDB, LDP): Luma BD-Rate of -0.03%, -0.12%, -0.08%, -0.53% is achieved with negligible encoding time, and decoding time increase is 6%.

Over VTM-4.0 Anchor with ALF ON (AI, RA, LDB, LDP): Luma BD-Rate of 0.05%, 0.03%, -0.05%, -0.15% is achieved with negligible encoding time, and decoding time increase is 4%.

# Introduction

VTM-4.0 [1] performs deblocking on 8 x 8 grids. Therefore deblocking is only performed for all the CU and TU edges which overlap with the 8 x 8 grids. Due to the VTM-4.0 partitioning scheme (Binary-tree (BT) and Ternary-tree (TT)), some long CU edges which may not overlap with the 8 x 8 grid but still have sufficient blocking artifacts. The scenario is depicted in the Fig. 1 below. The edge shown in red color belongs to a 4 x 16 block and current VTM-4.0 deblocking does not perform deblocking on this edge, as it does not overlap with an 8 x 8 sample grid.

Figure 1.  VTM 4.0 performs deblocking on 8 x 8 sample grid. Few 4 x N edges are not filtered

# Proposed Method

As shown in Fig. 2, proposed method enforces the Luma weak filter with one or no sample modification, when any of the blocks sharing the edge has a size of 4 samples. (i.e. width = 4 samples for vertical edge, height = 4 samples for horizontal edge).

The different steps performed for a vertical edge are illustrated below:

- Check if filter on/off condition is evaluated to true i.e. d < Beta, where d = d0 + d3 and d0 = dp0 + dq0 and d3= dp3 + dq3) is evaluated to true for the respective edge. If true then

- Check if the one of the blocks has a block width of 4 samples, if true, then

- Enforce weak filter with maximum one sample modification. Therefore the following condition is checked δ<10tC where δ=(9*q0-p0- (3*q1-p1+8)≫4, and if the condition is evaluated to true, then samples p0 and q0 are modified, otherwise no filtering is applied.

Besides the above filtering changes for the edge has a size of 4 samples, the sub-block edges of SBT and ISP which align with 4x4 sample grid are also deblocked. For ISP internal sub-block edges, the boundary strength (Bs) value for Luma is set to 2. Moreover, the MV component difference threshold for boundary strength calculation is also reduced from 1 luma sample to a quarter of a luma sample to more carefully include deblocking for blocks with small differences in motion.

Figure 2.  Weak filter with one sample modification for 4xN, Nx4 and 8xN, Nx8 blocks.

# Complexity evaluation

Complexity of the proposal is evaluated based on the following criteria described in the CE11 document, JVET-M1031 [1].

- Number of samples from the block boundary that can be modified by deblocking (P+Q):

  - For blocks size >=8 samples: it is the same as in VTM-4.0. Except when the edge is shared with a block whose size is 4 samples. Then only one sample is modified

  - For block size < 8 samples: one sample is modified

- Number of samples from the block boundary that are needed for deblocking decisions:

  - For blocks size >=8 samples: 4 samples are used (same as in VTM-4.0)

  - for block size < 8 samples: Three samples are only used.

- Number of filtering operations:

  - The maximum number of filter operations per edge, is still the same as in VTM-4.0

- Number of line buffers: 4 lines (same as in VTM-4.0)

- Parallelization capabilities: All edges overlapping with 4 x 4 grid can be deblocked in parallel.

- Luma only (Chroma filter is not changed)

# Experimental results

The proposed method is implemented on top of VTM-4.0 [2] and the simulation is performed under common test conditions as described in JVET-M1010 [3].

The results are presented for both ALF ON and ALF OFF conditions as suggested to be reported by the CE11 description document [4].

The results are as follows:

Table 1.   Objective results of the proposed deblocking method on top of VTM-4.0 for CTC configurations

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.00% | 0.01% | 94% | 104% |
| Class A2 | 0.03% | 0.00% | 0.00% | 93% | 107% |
| Class B | 0.09% | 0.00% | 0.00% | 91% | 105% |
| Class C | 0.05% | 0.00% | 0.00% | 83% | 107% |
| Class E | -0.01% | -0.01% | -0.01% | 93% | 105% |
| Overall | 0.05% | 0.00% | 0.00% | 90% | 106% |
| Class D | 0.05% | -0.01% | -0.01% | 92% | 109% |
| Class F | 0.02% | 0.01% | 0.00% | 95% | 105% |
|  | Random Access |  |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.09% | 0.09% | 105% | 103% |
| Class A2 | 0.03% | -0.03% | -0.03% | 107% | 104% |
| Class B | 0.11% | -0.15% | -0.12% | 97% | 103% |
| Class C | -0.04% | -0.07% | -0.13% | 102% | 105% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.06% | -0.06% | 102% | 104% |
| Class D | 0.05% | -0.12% | -0.02% | 101% | 103% |
| Class F | 0.05% | 0.06% | 0.02% | 100% | 105% |
|  | Low Delay B |  |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.07% | 0.23% | 101% | 105% |
| Class C | -0.17% | 0.09% | 0.39% | 94% | 106% |
| Class E | 0.01% | 0.15% | -0.22% | 86% | 103% |
| Overall | -0.05% | 0.04% | 0.17% | 95% | 104% |
| Class D | -0.03% | 0.16% | -0.56% | 97% | 106% |
| Class F | -0.06% | 0.02% | 0.21% | 101% | 104% |
|  | Low Delay P |  |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.04% | 0.03% | 100% | 104% |
| Class C | -0.19% | -0.44% | -0.45% | 98% | 105% |
| Class E | -0.25% | -0.39% | -0.23% | 100% | 102% |
| Overall | -0.15% | -0.26% | -0.20% | 99% | 104% |
| Class D | -0.01% | -0.07% | 0.26% | 95% | 108% |
| Class F | -0.12% | -0.54% | -0.68% | 110% | 102% |

Table 2. Objective results of the proposed deblocking method on top of VTM-4.0 for CTC with ALF switched off configurations

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.00% | 100% | 107% |
| Class A2 | -0.03% | 0.00% | 0.00% | 100% | 106% |
| Class B | 0.01% | 0.00% | 0.00% | 100% | 107% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 109% |
| Class E | -0.18% | -0.01% | -0.01% | 100% | 108% |
| Overall | -0.03% | 0.00% | 0.00% | 100% | 107% |
| Class D | -0.01% | 0.00% | 0.00% | 99% | 106% |
| Class F | -0.02% | -0.01% | 0.00% | 100% | 108% |
|  | Random Access |  |  |  |  |
|  | Over VTM-4.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.15% | 0.12% | 100% | 102% |
| Class A2 | -0.25% | -0.08% | -0.10% | 100% | 106% |
| Class B | -0.02% | -0.08% | -0.11% | 100% | 106% |
| Class C | -0.09% | -0.03% | -0.09% | 100% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.08% | -0.06% | 100% | 105% |
| Class D | 0.01% | -0.08% | 0.06% | 101% | 106% |
| Class F | 0.00% | -0.03% | -0.01% | 100% | 105% |
|  |  |  | Low Delay B |  |  |
|  | Over VTM-4.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.13% | 0.12% | 100% | 107% |
| Class C | -0.16% | -0.18% | -0.15% | 100% | 107% |
| Class E | -0.03% | 0.36% | -0.03% | 100% | 106% |
| Overall | -0.08% | -0.02% | -0.01% | 100% | 107% |
| Class D | 0.06% | -0.33% | -0.03% | 100% | 109% |
| Class F | 0.02% | -0.30% | 0.00% | 101% | 104% |
|  |  |  | Low Delay P |  |  |
|  | Over VTM-4.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.46% | -0.25% | -0.51% | 100% | 107% |
| Class C | -0.43% | -0.12% | 0.39% | 100% | 106% |
| Class E | -0.77% | -0.37% | -0.41% | 100% | 103% |
| Overall | -0.53% | -0.24% | -0.18% | 100% | 106% |
| Class D | -0.33% | -0.47% | -0.13% | 100% | 109% |
| Class F | -0.11% | 0.59% | 0.27% | 100% | 105% |

# Conclusion

This contribution presents the results for CE11-2.1. The proposed method applies deblocking on 4x4 grid to ensure that all blocks whose size is 4xN, Nx4, 8xN, Nx8 are deblocked and not only those that are aligned with the 8x8 sample grid. Moreover, the sub-block edges of SBT and ISP which align with 4x4 sample grid are also deblocked and the threshold for motion vector component comparison in boundary strength calculation is reduced to a quarter of a luma sample. To ensure parallel processing, the method uses constrained version of the VTM-4.0 weak filter. This filter only modifies one sample on either side of the edge, when one of the blocks sharing the edge has a size of 4 samples. The proposed method substantially improves the subjective quality when compared to VTM-4.0. Objective results show a Luma BD-Rate gain of upto of -0.53% for LDP configuration (ALF off). It is suggested to adopt this technique to the VVC draft specification and the VTM reference software.

# References

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 4)," Document of Joint Video Experts Team, JVET-M1001.

- VTM-4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-M1010.

- A. Norkin, A. M. Kotra, "Description of Core Experiment 11 (CE11): Deblocking," Document of Joint Video Experts Team, JVET-M1031.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).