| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0100-v2 |
| --- | --- |

| Title: | On Reference Picture List Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Modifications are proposed to reference picture list signalling. Following is proposed:

- Proposal 1: It is proposed to allow signalling information about long-term reference picture POC LSB in reference picture list syntax structure or collocated with corresponding long-term reference picture’s delta POC MSB related syntax elements in the tile group header.

- Proposal 2: A conditional signaling is proposed for signaling of num_ref_idx_active_override_flag. An inference rule is proposed when this flag is not signaled. In v2 the proposed condition is simplified based on BoG discussion.

- Proposal 3: A bug-fix is proposed for calculation of FullPocLt[ i ][ RplsIdx[ i ] ][ j ]. It is asserted that without this bug-fix, the incorrect value of poc_lsb_lt signalled in ref_pic_list_struct( listIdx, rplsIdx ) is used for the calculation of FullPOCLt – which provides full POC for a long term-reference picture.

# Introduction

VVC WD [1] includes reference picture management based on reference picture list signaling.

We propose modifications to reference picture list signaling for asserted functionality improvement, bit efficiency, and bug-fix purposes.

# Proposals

Following is proposed:

- Proposal 1: It is proposed to allow signalling information about long-term reference picture POC LSB in reference picture list syntax structure or collocated with corresponding long-term reference picture’s delta POC MSB related syntax elements in the tile group header.

- Proposal 2: A conditional signaling is proposed for signaling of num_ref_idx_active_override_flag. An inference rule is proposed when this flag is not signaled. In v2 the proposed condition is simplified based on BoG discussion.

- Proposal 3: A bug-fix is proposed for calculation of FullPocLt[ i ][ RplsIdx[ i ] ][ j ]. It is asserted that without this bug-fix, the incorrect value of poc_lsb_lt signalled in ref_pic_list_struct( listIdx, rplsIdx ) is used for the calculation of FullPOCLt – which provides full POC for a long term-reference picture.

## Proposal 1

It is asserted that currently in VVC WD 4 [1] for a long-term reference picture the POC LSB information is signalled in reference picture list syntax structure, whereas the corresponding long-term reference picture’s delta POC MSB related information (delta POC MSB presence flag and delta POC MSB cycle information) is signalled in tile group header. Thus, the information about POC for LTRPs is not collocated when signaling an index to the candidate reference picture list syntax structure which is signalled in SPS. It is proposed to allow signalling the information about long-term reference pictures including the POC LSB, delta POC MSB presence flag and delta POC MSB cycle information more flexibly compared to in the VVC WD 4 [1].

It is proposed that the location of Picture Order Count (POC) LSB information for long-term reference pictures be controlled by a flag. This flag allows signalling information about long-term reference picture POC LSB in reference picture list syntax structure or collocated with corresponding long-term reference picture’s delta POC MSB related syntax elements in the tile group header. This allows that the POC information about short-term reference pictures can be signaled in the reference picture list syntax structure and information about long-term reference picture POC LSB may be signaled in the reference picture list syntax structure or in tile group header collocated with the delta POC MSB presence and cycle information for the long-term reference picture. This allows reuse of a candidate short-term reference picture structure from SPS while providing long-term reference picture POC information in tile group header (i.e. at the time of use). It is asserted that such a flexibility has been allowed by the signalling in HEVC and is currently lacking in the signaling in VVC WD 4 [1] as shown by the table below with a “x” mark.

| Configuration | HEVC | VVC WD 4 | VVC with Proposal |
| --- | --- | --- | --- |
| STRP info in tile group header LTRP POC LSB info and delta POC MSB info in tile group header |  |  |  |
| STRP info in SPS with index to it in tile group header LTRP POC LSB info in SPS and delta POC MSB info in tile group header |  |  |  |
| STRP info in SPS with index to it in tile group header LTRP POC LSB info and delta POC MSB info in tile group header |  | x |  |

The proposed changes are shown highlighted below.

### 7.3.5 Reference picture list structure syntax

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( long_term_ref_pics_flag ) |  |
| ltrp_in_tilegroup_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j=0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if(!ltrp_in_tilegroup_header_flag[ listIdx ][ rplsIdx ]) { |  |
| poc_lsb_lt[ listIdx ][ rplsIdx ][ ij ] | u(v) |
| j=j+1 |  |
| } |  |
| } |  |
| } |  |

### 7.4.6 Reference picture list structure semantics

…

ltrp_in_tilegroup_header_flag[ listIdx ][ rplsIdx ] equal to 1 specifies that the long-term reference picture’s Picture Order Count (POC) LSB information for the listIdx-th list for the rplsIdx-th reference picture list is signaled in the tile group header. ltrp_in_tilegroup_header_flag[ listIdx ][ rplsIdx ] equal to 0 specifies that the long-term reference picture’s Picture Order Count (POC) LSB information for the listIdx-th list for the rplsIdx-th reference picture list is signaled in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure.

### 7.3.4 Tile group header syntax

#### 7.3.4.1 General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| tile_group_type | ue(v) |
| tile_group_pic_order_cnt_lsb | u(v) |
| if( nal_unit_type != IRAP_NUT ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_sps_flag[ i ] | u(1) |
| if( ref_pic_list_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if(ltrp_in_tilegroup_header_flag[ i ][ RplsIdx[ i ] ]) |  |
| poc_lsb_lt[ i ][ j ] | u(v) |
| delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( delta_poc_msb_present_flag[ i ][ j ] ) |  |
| delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| if(tile_group_type = = P ¦ ¦ tile_group_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < tile_group_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| byte_alignment( ) |  |
| } |  |

#### General tile group header syntax

poc_lsb_lt[ i ][ j ] specifies the value of the picture order count modulo MaxPicOrderCntLsb of the j-th long term reference picture in the i-th reference picture list. The length of the poc_lsb_lt[ i ][ j ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits.

The variable PocLsbLt[ i ][ j ] is derived as follows: 
PocLsbLt[ i ][ j ] = (ltrp_in_tilegroup_header_flag[ i ][ RplsIdx[ i ] ]) ? poc_lsb_lt[ i ][ j ] : poc_lsb_lt[ listIdx ][ RplsIdx[ i ] ][ j ]

delta_poc_msb_present_flag[ i ][ j ] equal to 1 specifies that delta_poc_msb_cycle_lt[ i ][ j ] is present. delta_poc_msb_present_flag[ i ][ j ] equal to 0 specifies that delta_poc_msb_cycle_lt[ i ][ j ] is not present.

Let prevTid0Pic be the previous picture in decoding order that has TemporalId equal to 0 and is not a RASL or RADL picture. Let setOfPrevPocVals be a set consisting of the following:

–	the PicOrderCntVal of prevTid0Pic,

–	the PicOrderCntVal of each picture referred to by entries in RefPicList[ 0 ] and entries in RefPicList[ 1 ] of prevTid0Pic,

–	the PicOrderCntVal of each picture that follows prevTid0Pic in decoding order and precedes the current picture in decoding order.

When there is more than one value in setOfPrevPocVals for which the value modulo MaxPicOrderCntLsb is equal to poc_lsb_lt[ i ][ RplsIdx[ i ] ][ j ] PocLsbLt[ i ][ j ], the value of delta_poc_msb_present_flag[ i ][ j ] shall be equal to 1.

delta_poc_msb_cycle_lt[ i ][ j ] specifies the value of the variable FullPocLt[ i ][ j ] as follows:

if( j  = =  0 )
	deltaMsbCycle[ i ][ j ] = delta_poc_msb_cycle_lt[ i ][ j ]
else				
	deltaMsbCycle[ i ][ j ] = delta_poc_msb_cycle_lt[ i ][ j ] + deltaMsbCycle[ i ][ j − 1 ]
FullPocLt[ i ][ RplsIdx[ i ] ][ j ] = PicOrderCntVal − deltaMsbCycle[ i ][ j ] * MaxPicOrderCntLsb  −
		( PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) ) + poc_lsb_lt[ i ][ RplsIdx[ i ] ][ j ] PocLsbLt[ i ][ j ]

The value of delta_poc_msb_cycle_lt[ i ][ j ] shall be in the range of 0 to 2(32 − log2_max_pic_order_cnt_lsb_minus4 − 4 ), inclusive. When not present, the value of delta_poc_msb_cycle_lt[ i ][ j ] is inferred to be equal to 0.

### Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each tile group of a non-IDR picture.

Reference pictures are addressed through reference indices. A reference index is an index into a reference picture list. When decoding an I tile group, no reference picture list is used in decoding of the tile group data. When decoding a P tile group, only reference picture list 0 (i.e., RefPicList[ 0 ]), is used in decoding of the tile group data. When decoding a B tile group, both reference picture list 0 and reference picture list 1 (i.e., RefPicList[ 1 ]) are used in decoding of the tile group data.

At the beginning of the decoding process for each tile group of a non-IDR picture, the reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ] are derived. The reference picture lists are used in marking of reference pictures as specified in clause 8.3.3 or in decoding of the tile group data.

NOTE 1 – For an I tile group of a non-IDR picture that it is not the first tile group of the picture, RefPicList[ 0 ] and RefPicList[ 1 ] may be derived for bitstream conformance checking purpose, but their derivation is not necessary for decoding of the current picture or pictures following the current picture in decoding order. For a P tile group that it is not the first tile group of a picture, RefPicList[ 1 ] may be derived for bitstream conformance checking purpose, but its derivation is not necessary for decoding of the current picture or pictures following the current picture in decoding order.

The reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ] are constructed as follows:

for( i = 0; i < 2; i++ ) {
		for( j = 0, k = 0, pocBase = PicOrderCntVal; j < num_ref_entries[ i ][ RplsIdx[ i ] ]; j++) {
			if( st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
				RefPicPocList[ i ][ j ] = pocBase − DeltaPocSt[ i ][ RplsIdx[ i ] ][ j ]
				if( there is a reference picture picA in the DPB with PicOrderCntVal equal to RefPicPocList[ i ][ j ] )
					RefPicList[ i ][ j ] = picA
				else
					RefPicList[ i ][ j ] = "no reference picture"					(8-5)
				pocBase = RefPicPocList[ i ][ j ]
			} else {
				if( !delta_poc_msb_cycle_lt[ i ][ k ] ) {
					if( there is a reference picA in the DPB with PicOrderCntVal & ( MaxPicOrderCntLsb − 1 )
										equal to poc_lsb_lt[ i ][ RplsIdx[ i ] ][ j ] PocLsbLt[ i ][ k ] )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"
				} else {
					if( there is a reference picA in the DPB with PicOrderCntVal equal to
										FullPocLt[ i ][ RplsIdx[ i ] ][ jk ] )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"
			}
				k++
		}
	}
}

For each i equal to 0 or 1, the first NumRefIdxActive[ i ] entries in RefPicList[ i ] are referred to as the active entries in RefPicList[ i ], and the other entries in RefPicList[ i ] are referred to as the inactive entries in RefPicList[ i ].

## Proposal 2

A conditional signaling is proposed for signaling of num_ref_idx_active_override_flag. An inference rule is proposed when this flag is not signaled. This can save a bit in each tile group header.

In the proposal below under following condition:

- If tile group type is P tile group and number of reference entries for the selected reference picture list (list 0) is equal to 1 and if number of reference index default active minus1 is equal to 0 OR

- If tile group type is B tile group and number of reference entries for both the selected reference picture lists (list 0 and list 1) is equal to 1 and if number of reference index default active minus1 is equal to 0,

the flag num_ref_idx_active_override_flag is not signalled as its value can be inferred, otherwise it is signalled.

The value can be inferred in the above case because calculation of NumRefIdxActive[ i ] with or without signaling num_ref_idx_active_override_flag results in the same value. It is asserted that in this case it is wasteful to signal this flag which is not required.

In v2 version the proposed condition is simplified based on BoG discussion.

The proposed changes are shown highlighted below.

### 7.3.4 Tile group header syntax

#### 7.3.4.1 General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| tile_group_address | u(v) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag ) |  |
| num_tiles_in_tile_group_minus1 | ue(v) |
| tile_group_type | ue(v) |
| tile_group_pic_order_cnt_lsb | u(v) |
| if( nal_unit_type != IRAP_NUT ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_sps_flag[ i ] | u(1) |
| if( ref_pic_list_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( delta_poc_msb_present_flag[ i ][ j ] ) |  |
| delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| if( (tile_group_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1) ¦¦ (tile_group_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 )) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( tile_group_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

#### 7.4.5.1 General tile group header semantics

num_ref_idx_active_override_flag equal to 1 specifies that the syntax element num_ref_idx_active_minus1[ 0 ] is present for P and B tile groups and that the syntax element num_ref_idx_active_minus1[ 1 ] is present for B tile groups. num_ref_idx_active_override_flag equal to 0 specifies that the syntax elements num_ref_idx_active_minus1[ 0 ] and num_ref_idx_active_minus1[ 1 ] are not present. When not present num_ref_idx_active_override_flag is inferred to be equal to 1.

num_ref_idx_active_minus1[ i ] is used for the derivation of the variable NumRefIdxActive[ i ] as specified by Equation 7-38. The value of num_ref_idx_active_minus1[ i ] shall be in the range of 0 to 14, inclusive.

For i equal to 0 or 1, when the current tile group is a B tile group, num_ref_idx_active_override_flag is equal to 1, and num_ref_idx_active_minus1[ i ] is not present, num_ref_idx_active_minus1[ i ] is inferred to be equal to 0.

When the current tile group is a P tile group, num_ref_idx_active_override_flag is equal to 1, and num_ref_idx_active_minus1[ 0 ] is not present, num_ref_idx_active_minus1[ 0 ] is inferred to be equal to 0.

The variable NumRefIdxActive[ i ] is derived as follows:

for( i = 0; i < 2; i++ ) {
	if( tile_group_type  = =  B  | |  ( tile_group_type  = =  P  &&  i  = = 0 ) ) {
		if( num_ref_idx_active_override_flag  = =  1 )
			NumRefIdxActive[ i ] = num_ref_idx_active_minus1[ i ] + 1	(7-38)
		else {
			if( num_ref_entries[ i ][ RplsIdx[ i ] ]  >=  num_ref_idx_default_active_minus1[ i ] + 1 )
				NumRefIdxActive[ i ] = num_ref_idx_default_active_minus1[ i ] + 1
			else
				NumRefIdxActive[ i ] = num_ref_entries[ i ][ RplsIdx[ i ] ]
		}
	} else // tile_group_type  = =  I  | |  ( tile_group_type  = =  P  &&  i  = = 1 )
		NumRefIdxActive[ i ] = 0
}

The value of NumRefIdxActive[ i ] − 1 specifies the maximum reference index for reference picture list i that may be used to decode the tile group. When the value of NumRefIdxActive[ i ] is equal to 0, no reference index for reference picture list i may be used to decode the tile group.

Additionally the for loop shown below

| for( i = 0; i < ( tile_group_type = = B ? 2: 1 ); i++ ) |  |
| --- | --- |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |

can be unrolled as follows and is left to editors’ discretion:

| if( num_ref_idx_active_override_flag ) { |  |
| --- | --- |
| if( num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ 0 ] | ue(v) |
| if( num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ 1 ] | ue(v) |
| } |  |

Analysis:

The Analysis below shows that inference rule is adequate in the case of the conditions described above:

| num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] | num_ref_idx_default_active_minus1[ 0 ] | num_ref_idx_active_override_flag | num_ref_idx_active_minus1[ i ] | NumRefIdxActive[ i ] with signaling | NumRefIdxActive[ i ] with inference of num_ref_idx_active_override_flag to 0 |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | If(num_ref_entries[ i ][ RplsIdx[ i ] ] >= num_ref_idx_default_active_minus1[ i ] + 1) NumRefIdxActive[ i ] = num_ref_idx_default_active_minus1[ i ] + 1 => NumRefIdxActive[ i ] = 0 + 1 = 1 | Same as cell to the left, i.e. => num_ref_idx_active_override_flag=0 NumRefIdxActive[ i ] = 0 + 1 = 1 |
| 1 | 0 | 1 | 0 | NumRefIdxActive[ i ] = num_ref_idx_active_minus1[ i ] + 1 => NumRefIdxActive[ i ] = 0 + 1 = 1 | Same as cell above, i.e. => num_ref_idx_active_override_flag=0 NumRefIdxActive[ i ] = 0 + 1 = 1 |

The same analysis applies to reference picture list 1 for tile_group_type equal to B.

## Proposal 3

A bug-fix is proposed for calculation of FullPocLt[ i ][ RplsIdx[ i ] ][ j ]. It is asserted that without this bug-fix, the incorrect value of poc_lsb_lt signalled in ref_pic_list_struct( listIdx, rplsIdx ) is used for the calculation of FullPOCLt – which provides full POC for a long term-reference picture.

Two alternative options are proposed for the bug-fix.

Option 1:

The proposed changes are shown highlighted below.

### Reference picture list structure syntax

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| for( i = 0, j=0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else { |  |
| poc_lsb_lt[ listIdx ][ rplsIdx ][ ij ] | u(v) |
| j = j +1 |  |
| } |  |
| } |  |
| } |  |

### Reference picture list structure semantics

poc_lsb_lt[ listIdx ][ rplsIdx ][ ij ] specifies the value of the picture order count modulo MaxPicOrderCntLsb of the j-th long-term reference picture referred to by the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. The length of the poc_lsb_lt[ listIdx ][ rplsIdx ][ ij ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits.

### 8.3.2 Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each tile group of a non-IRAP picture.

Reference pictures are addressed through reference indices. A reference index is an index into a reference picture list. When decoding an I tile group, no reference picture list is used in decoding of the tile group data. When decoding a P tile group, only reference picture list 0 (i.e., RefPicList[ 0 ]), is used in decoding of the tile group data. When decoding a B tile group, both reference picture list 0 and reference picture list 1 (i.e., RefPicList[ 1 ]) are used in decoding of the tile group data.

At the beginning of the decoding process for each tile group of a non-IRAP picture, the reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ] are derived. The reference picture lists are used in marking of reference pictures as specified in clause 8.3.3 or in decoding of the tile group data.

NOTE 1 – For an I tile group of a non-IRAP picture that it is not the first tile group of the picture, RefPicList[ 0 ] and RefPicList[ 1 ] may be derived for bitstream conformance checking purpose, but their derivation is not necessary for decoding of the current picture or pictures following the current picture in decoding order. For a P tile group that it is not the first tile group of a picture, RefPicList[ 1 ] may be derived for bitstream conformance checking purpose, but its derivation is not necessary for decoding of the current picture or pictures following the current picture in decoding order.

The reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ] are constructed as follows:

for( i = 0; i < 2; i++ ) {
		for( j = 0, k = 0, pocBase = PicOrderCntVal; j < num_ref_entries[ i ][ RplsIdx[ i ] ]; j++) {
			if( st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
				RefPicPocList[ i ][ j ] = pocBase − DeltaPocSt[ i ][ RplsIdx[ i ] ][ j ]
				if( there is a reference picture picA in the DPB with PicOrderCntVal equal to RefPicPocList[ i ][ j ] )
					RefPicList[ i ][ j ] = picA
				else
					RefPicList[ i ][ j ] = "no reference picture"					(8-5)
				pocBase = RefPicPocList[ i ][ j ]
			} else {
				if( !delta_poc_msb_cycle_lt[ i ][ k ] ) {
					if( there is a reference picA in the DPB with PicOrderCntVal & ( MaxPicOrderCntLsb − 1 )
										equal to poc_lsb_lt[ i ][ RplsIdx[ i ] ][ jk ] )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"
				} else {
					if( there is a reference picA in the DPB with PicOrderCntVal equal to
										FullPocLt[ i ][ RplsIdx[ i ] ][ jk ] )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"
			}
				k++
		}
	}
}

Option 2:

The proposed changes are shown highlighted below.

#### 7.4.5.1 General tile group header semantics

…

delta_poc_msb_cycle_lt[ i ][ j ] specifies the value of the variable FullPocLt[ i ][ j ] as follows:

if( j  = =  0 )
	deltaMsbCycle[ i ][ j ] = delta_poc_msb_cycle_lt[ i ][ j ]
else					(7-52)
	deltaMsbCycle[ i ][ j ] = delta_poc_msb_cycle_lt[ i ][ j ] + deltaMsbCycle[ i ][ j − 1 ] 
for(k = 0, m= 0; k < num_ref_entries[ i ][ RplsIdx[ i ] ]; k++) {
	if(!st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ k ]) 
		m++
	if(m==j) 
		break;
}
FullPocLt[ i ][ RplsIdx[ i ] ][ j ] = PicOrderCntVal − deltaMsbCycle[ i ][ j ] * MaxPicOrderCntLsb  −
		( PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) ) + poc_lsb_lt[ i ][ RplsIdx[ i ] ][ jk ]

The value of delta_poc_msb_cycle_lt[ i ][ j ] shall be in the range of 0 to 2(32 − log2_max_pic_order_cnt_lsb_minus4 − 4 ), inclusive. When not present, the value of delta_poc_msb_cycle_lt[ i ][ j ] is inferred to be equal to 0.

### Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each tile group of a non-IDR picture.

Reference pictures are addressed through reference indices. A reference index is an index into a reference picture list. When decoding an I tile group, no reference picture list is used in decoding of the tile group data. When decoding a P tile group, only reference picture list 0 (i.e., RefPicList[ 0 ]), is used in decoding of the tile group data. When decoding a B tile group, both reference picture list 0 and reference picture list 1 (i.e., RefPicList[ 1 ]) are used in decoding of the tile group data.

At the beginning of the decoding process for each tile group of a non-IDR picture, the reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ] are derived. The reference picture lists are used in marking of reference pictures as specified in clause 8.3.3 or in decoding of the tile group data.

NOTE 1 – For an I tile group of a non-IDR picture that it is not the first tile group of the picture, RefPicList[ 0 ] and RefPicList[ 1 ] may be derived for bitstream conformance checking purpose, but their derivation is not necessary for decoding of the current picture or pictures following the current picture in decoding order. For a P tile group that it is not the first tile group of a picture, RefPicList[ 1 ] may be derived for bitstream conformance checking purpose, but its derivation is not necessary for decoding of the current picture or pictures following the current picture in decoding order.

The reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ] are constructed as follows:

for( i = 0; i < 2; i++ ) {
		for( j = 0, k = 0, pocBase = PicOrderCntVal; j < num_ref_entries[ i ][ RplsIdx[ i ] ]; j++) {
			if( st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
				RefPicPocList[ i ][ j ] = pocBase − DeltaPocSt[ i ][ RplsIdx[ i ] ][ j ]
				if( there is a reference picture picA in the DPB with PicOrderCntVal equal to RefPicPocList[ i ][ j ] )
					RefPicList[ i ][ j ] = picA
				else
					RefPicList[ i ][ j ] = "no reference picture"					(8-5)
				pocBase = RefPicPocList[ i ][ j ]
			} else {
				if( !delta_poc_msb_cycle_lt[ i ][ k ] ) {
					if( there is a reference picA in the DPB with PicOrderCntVal & ( MaxPicOrderCntLsb − 1 )
										equal to poc_lsb_lt[ i ][ RplsIdx[ i ] ][ jk  ] )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"
				} else {
					if( there is a reference picA in the DPB with PicOrderCntVal equal to
										FullPocLt[ i ][ RplsIdx[ i ] ][ jk ] )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"
			}
				k++
		}
	}
}

# Conclusion

This document proposes modifications to reference picture list signaling in VVC. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-M1001-v6, Versatile Video Coding (Draft 4), Marrakech, March 2019