| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0101 |
| --- | --- |

| Title: | On Gradual and Clean Random Access |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document provides proposals related to gradual random access and clean random access. Following is proposed:

- Proposal 1: Signalling is proposed for a gradual random access (GRA) picture related information and to allow support for random access to be performed at a GRA picture. Two alternative syntax options are proposed for this.

- Proposal 2: Decoding process for generating unavailable reference pictures is proposed. This process is proposed to be invoked for decoding of CRA picture with NoRaslOutputFlag equal to 1. Additionally if GRA functionality is agreed then it is also proposed to be invoked for GRA picture decoding.

# Introduction

VVC WD [1] includes Clean Random Access (CRA) pictures. However decoding process for generating unavailable reference pictures which is asserted to be needed is currently missing in VVC WD.

This document provides proposals related to gradual random access and clean random access.

# Proposals

Following is proposed:

- Proposal 1: Signalling is proposed for a gradual random access (GRA) picture related information and to allow support for random access to be performed at a GRA picture. Two alternative syntax options are proposed for this.

  - Option 1: For this option following is proposed:

    - A VCL NAL unit for gradual random access (GRA) picture.

    - Signalling of flags in PPS to indicate if a flag which provides GRA picture information and a pic_output_flag is signalled in tile group header.

    - Conditional signalling in tile group header of a flag which provides GRA picture information and a pic_output_flag.

    - Derivation of a GraFlag variable.

    - Decoding process for GRA pictures and generation of unavailable reference pictures.

  - Option 2: For this option following is proposed:

    - A VCL NAL unit for gradual random access (GRA) picture.

    - Conditional signalling in tile group header of GRA picture information similar to recovery point picture related information.

    - output_flag_present_flag and pic_out_flag syntax elements.

    - Derivation of a GraFlag variable.

    - Decoding process for GRA pictures and generation of unavailable reference pictures.

- Proposal 2: Decoding process for generating unavailable reference pictures is proposed. This process is proposed to be invoked for decoding of CRA picture with NoRaslOutputFlag equal to 1. Additionally if GRA functionality is agreed then it is also proposed to be invoked for GRA picture decoding.

## Proposal 1

Signalling is proposed for a gradual random access (GRA) picture related information and to allow support for random access to be performed at a GRA picture. Two alternative syntax options are proposed for this.

### Option 1

For this option following is proposed:

- A VCL NAL unit for gradual random access (GRA) picture.

- Signalling of flags in PPS to indicate if a flag which provides GRA picture information and a pic_output_flag is signalled in tile group header.

- Conditional signalling in tile group header of a flag which provides GRA picture information and a pic_output_flag.

- Derivation of a GraFlag variable.

- Decoding process for GRA pictures and generation of unavailable reference pictures.

The proposed changes are shown highlighted below.

  - Gradual Random Access (GRA) picture: An IRAP Picture for which each VCL NAL unit has a nal_unit_type equal to GRA_NUT.

#### 7.4.2.2 NAL unit header semantics

forbidden_zero_bit shall be equal to 0.

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table X.

Table 7-1 NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | TRAIL | Coded tile group of a non-STSA trailing picture tile_group_layer_rbsp( ) | VCL |
| 1 | STSA | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 2 | RADL | Coded tile group of a RADL picture tile_group_layer_rbsp( ) | VCL |
| 3 | RASL | Coded tile group of a RASL picture tile_group_layer_rbsp( ) | VCL |
| 4..7 | RSV_VCL_2.. RSV_VCL_7 | Reserved non-IRAP VCL NAL unit types | VCL |
| 8 9 | IDR_W_RADL IDR_N_LP | Coded tile group of an IDR picture tile_group_layer_rbsp( ) | VCL |
| 10 | CRA_NUT | Coded tile group of a CRA picture tile_group_layer_rbsp( ) | VCL |
| 11 | GRA_NUT | Coded tile group of a Gradual Random Access picture tile_group_layer_rbsp() | VCL |
| 11 12 13 | RSV_IRAP_VCL11 RSV_IRAP_VCL12 RSV_IRAP_VCL13 | Reserved IRAP VCL NAL unit types | VCL |
| 14..15 | RSV_VCL14.. RSV_VCL15 | Reserved non-IRAP VCL NAL unit types | VCL |
| 16 | DPS_NUT | Decoding parameter set decoding_parameter_set_rbsp( ) | non-VCL |
| 17 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 18 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 19 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 20 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 21 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 22 23 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 24..27 | RSV_NVCL24.. RSV_NVCL27 | Reserved | non-VCL |
| 28..31 | UNSPEC28.. UNSPEC31 | Unspecified | non-VCL |

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit. The value of nuh_temporal_id_plus1 shall not be equal to 0.

The variable TemporalId is derived as follows:

TemporalId = nuh_temporal_id_plus1 − 1

When nal_unit_type is equal to IRAP_NUT, the coded slice belongs to an IRAP picture, TemporalId shall be equal to 0.

When nal_unit_type is equal to GRA_NUT, TemporalId shall be equal to 0.

The value of TemporalId shall be the same for all VCL NAL units of an access unit. The value of TemporalId of a coded picture or an access unit is the value of the TemporalId of the VCL NAL units of the coded picture or the access unit.

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If nal_unit_type is equal to SPS_NUT, TemporalId shall be equal to 0 and the TemporalId of the access unit containing the NAL unit shall be equal to 0.

–	Otherwise, if nal_unit_type is equal to EOS_NUT or EOB_NUT, TemporalId shall be equal to 0.

–	Otherwise, TemporalId shall be greater than or equal to the TemporalId of the access unit containing the NAL unit.

NOTE – When the NAL unit is a non-VCL NAL unit, the value of TemporalId is equal to the minimum value of the TemporalId values of all access units to which the non-VCL NAL unit applies. When nal_unit_type is equal to PPS_NUT, TemporalId may be greater than or equal to the TemporalId of the containing access unit, as all picture parameter sets (PPSs) may be included in the beginning of a bitstream, wherein the first coded picture has TemporalId equal to 0. When nal_unit_type is equal to PREFIX_SEI_NUT or SUFFIX_SEI_NUT, TemporalId may be greater than or equal to the TemporalId of the containing access unit, as an SEI NAL unit may contain information that applies to a bitstream subset that includes access units for which the TemporalId values are greater than the TemporalId of the access unit containing the SEI NAL unit.

nuh_reserved_zero_7bits shall be equal to '0000000'. Other values of nuh_reserved_zero_7bits may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore (i.e. remove from the bitstream and discard) NAL units with values of nuh_reserved_zero_7bits not equal to '0000000'.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| gra_flag_present_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| } |  |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

output_flag_present_flag equal to 1 indicates that the pic_output_flag syntax element is present in the associated tile group headers. output_flag_present_flag equal to 0 indicates that the pic_output_flag syntax element is not present in the associated tile group headers.

gra_flag_present_flag equal to 1 indicates that the gra_output_flag syntax element is present in the associated tile group headers. gra_flag_present_flag equal to 0 indicates that the gra_output_flag syntax element is not present in the associated tile group headers.

### Tile group header syntax

#### 7.3.4.1 General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| tile_group_type | ue(v) |
| if(output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( gra_flag_present_flag ) |  |
| gra_output_flag | u(1) |
| tile_group_pic_order_cnt_lsb | u(v) |
| … |  |
| byte_alignment( ) |  |
| } |  |

#### General tile group header semantics

When present, the value of the tile group header syntax elements slice_pic_parameter_set_id, pic_output_flag, gra_output_flag, slice_pic_order_cnt_lsb, short_term_ref_pic_set_sps_flag, short_term_ref_pic_set_idx, num_long_term_sps, num_long_term_pics, and slice_temporal_mvp_enabled_flag shall be the same in all tile group headers of a coded picture.

…

pic_output_flag affects the decoded picture output and removal processes as specified in Annex C. When pic_output_flag is not present, it is inferred to be equal to 1.

gra_output_flag equal to 1 specifies that if decoding process was started at the previous GRA picture that precedes the current picture, either because the previous GRA picture is the first picture in the CVS or if a random access is performed at that GRA picture, the current decoded picture is an exact match to the picture produced by starting the decoding process at the previous non-GRA IRAP picture  preceding the current picture. gra_output_flag equal to 0 specifies that if decoding process was started at the previous GRA picture that precedes the current picture, either because the previous GRA picture is the first picture in the CVS or if a random access is performed at that GRA picture, the current decoded picture may not be an exact match to the picture produced by starting the decoding process at the previous non-GRA IRAP picture  preceding the current picture.When gra_output_flag is not present, it is inferred to be equal to 1.

Decoding process:

    - General

Input to this process is a bitstream. Output of this process is a list of decoded pictures.

The decoding process is specified such that all decoders that conform to a specified profile and level will produce numerically identical cropped decoded output pictures when invoking the decoding process associated with that profile for a bitstream conforming to that profile and level. Any decoding process that produces identical cropped decoded output pictures to those produced by the process described herein (with the correct output order or output timing, as specified) conforms to the decoding process requirements of this Specification.

At the beginning of decoding a CVS, after activating the SPS RBSP that is active for the entire CVS and before decoding any VCL NAL units of the CVS, clause 8.1.2 is invoked with the CVS as input.

    - CVS decoding process

Input to this process is a CVS. Output of this process is a list of decoded pictures.

The variable HighestTid, which identifies the highest temporal sub-layer to be decoded, is specified as follows:

–	If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

–	Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1.

The sub-bitstream extraction process as specified in clause XX is applied with the CVS and HighestTid as inputs, and the output is assigned to a bitstream referred to as BitstreamToDecode.

Clause 8.1.3 is repeatedly invoked for each coded picture in BitstreamToDecode in decoding order.

    - Decoding process for a coded picture

The decoding processes specified in this clause apply to each coded picture, referred to as the current picture and denoted by the variable CurrPic, in BitstreamToDecode.

Depending on the value of chroma_format_idc, the number of sample arrays of the current picture is as follows:

–	If chroma_format_idc is equal to 0, the current picture consists of 1 sample array SL.

–	Otherwise (chroma_format_idc is not equal to 0), the current picture consists of 3 sample arrays SL, SCb, SCr.

The decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause 7. When interpreting the semantics of each syntax element in each NAL unit, the term "the bitstream" (or part thereof, e.g., a CVS of the bitstream) refers to BitstreamToDecode (or part thereof).

When the current picture is an IRAP picture, the following applies:

–	If the current picture is an IDR picture, the first picture in the bitstream in decoding order, or the first picture that follows an end of sequence NAL unit in decoding order, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsCvsStartFlag to a value for the current picture, the variable HandleCraAsCvsStartFlag is set equal to the value provided by the external means and the variable NoRaslOutputFlag is set equal to HandleCraAsCvsStartFlag.

–	Otherwise, the variable HandleCraAsCvsStartFlag is set equal to 0 and the variable NoRaslOutputFlag is set equal to 0.

Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to one of the 3 sample arrays of the current picture, with the NAL units with colour_plane_id equal to 0, 1 and 2 being assigned to SL, SCb and SCr, respectively.

NOTE 1 – The variable ChromaArrayType is derived as equal to 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures (when chroma_format_idc is equal to 0).

When the current picture is GRA picture, the following applies:

–	If the current picture is a GRA picture, and is the first picture in the CVS or if a random access is started at the current picture, the variable GraFlag is set equal to 1.

–	Otherwise, the variable GraFlag is set equal to 0.

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in clause TBD.

- The processes in clauses TBD specify decoding processes using syntax elements in in the tile group layer and above.

  - When the current picture is a GRA picture with GraFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 0.1.1 (see proposal 2 below for this clause) is invoked, which needs to be invoked only for the first tile group of a picture.

- PicOutputFlag is set as follows:

–	If the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–   Otherwise if GraFlag is equal to 1 and gra_output_flag is equal to 0, PicOutputFlag is set equal to gra_output_flag (i.e. it is set equal to 0).

–   Otherwise, PicOutputFlag is set equal to pic_output_flag.

- If GraFlag is equal to 1, following applies:

–   If gra_output_flag is equal to 1, GraFlag is set equal to 0.

- The processes in clauses TBD specify decoding processes using syntax elements in all syntax structure layers.

In a variant the cyan highlighted part in step 3 may be omitted.

### Option 2

For this option following is proposed:

- A VCL NAL unit for gradual random access (GRA) picture.

- Conditional signalling in tile group header of GRA picture information similar to recovery point picture related information.

- output_flag_present_flag and pic_out_flag syntax elements.

- Derivation of a GraFlag variable.

- Decoding process for GRA pictures and generation of unavailable reference pictures.

The proposed changes are shown highlighted below.

  - Gradual Random Access (GRA) picture: An IRAP Picture for which each VCL NAL unit has a nal_unit_type equal to GRA_NUT.

#### 7.4.2.2 NAL unit header semantics

forbidden_zero_bit shall be equal to 0.

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table X.

Table 7-1 NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | TRAIL | Coded tile group of a non-STSA trailing picture tile_group_layer_rbsp( ) | VCL |
| 1 | STSA | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 2 | RADL | Coded tile group of a RADL picture tile_group_layer_rbsp( ) | VCL |
| 3 | RASL | Coded tile group of a RASL picture tile_group_layer_rbsp( ) | VCL |
| 4..7 | RSV_VCL_2.. RSV_VCL_7 | Reserved non-IRAP VCL NAL unit types | VCL |
| 8 9 | IDR_W_RADL IDR_N_LP | Coded tile group of an IDR picture tile_group_layer_rbsp( ) | VCL |
| 10 | CRA_NUT | Coded tile group of a CRA picture tile_group_layer_rbsp( ) | VCL |
| 11 | GRA_NUT | Coded tile group of a Gradual Random Access picture tile_group_layer_rbsp() | VCL |
| 11 12 13 | RSV_IRAP_VCL11 RSV_IRAP_VCL12 RSV_IRAP_VCL13 | Reserved IRAP VCL NAL unit types | VCL |
| 14..15 | RSV_VCL14.. RSV_VCL15 | Reserved non-IRAP VCL NAL unit types | VCL |
| … | … | … | non-VCL… |

In a variant: GRA_NUT may be a non-VCL NAL unit.

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit. The value of nuh_temporal_id_plus1 shall not be equal to 0.

The variable TemporalId is derived as follows:

TemporalId = nuh_temporal_id_plus1 − 1

When nal_unit_type is equal to IRAP_NUT, the coded slice belongs to an IRAP picture, TemporalId shall be equal to 0.

When nal_unit_type is equal to GRA_NUT, TemporalId shall be equal to 0.

…

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| } |  |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

output_flag_present_flag equal to 1 indicates that the pic_output_flag syntax element is present in the associated tile group headers. output_flag_present_flag equal to 0 indicates that the pic_output_flag syntax element is not present in the associated tile group headers.

### 7.3.4 Tile group header syntax

#### 7.3.4.1 General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| tile_group_type | ue(v) |
| if(output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( nal_unit_Type == GRA_NUT ) { |  |
| recovery_poc_cnt | se(v) |
| exact_match_flag | u(1) |
| broken_link_flag | u(1) |
| } |  |
| tile_group_pic_order_cnt_lsb | u(v) |
| … |  |
| } |  |

recovery_poc_cnt specifies the recovery point of decoded pictures in output order. If there is a picture picA that follows the current GRA picture in decoding order in the CVS and that has PicOrderCntVal equal to the PicOrderCntVal of the current GRA picture plus the value of recovery_poc_cnt, the picture picA is referred to as the recovery point picture. Otherwise, the first picture in output order that has PicOrderCntVal greater than the PicOrderCntVal of the current picture plus the value of recovery_poc_cnt is referred to as the recovery point picture. The recovery point picture shall not precede the current GRA picture in decoding order. All decoded pictures in output order are indicated to be correct or approximately correct in content starting at the output order position of the recovery point picture. The value of recovery_poc_cnt shall be in the range of −MaxPicOrderCntLsb / 2 to MaxPicOrderCntLsb / 2 − 1, inclusive.

The variable RPPicOrderCntVal is derived as follows:

RPPicOrderCntVal = PicOrderCntlVal + recovery_poc_cnt

exact_match_flag indicates whether decoded pictures at and subsequent to the specified recovery point in output order derived by starting the decoding process at the access unit associated with the current GRA picture will be an exact match to the pictures that would be produced by starting the decoding process at the location of a previous non-GRA IRAP access unit, if any, in the bitstream. The value 0 indicates that the match may not be exact and the value 1 indicates that the match will be exact. When exact_match_flag is equal to 1, it is a requirement of bitstream conformance that the decoded pictures at and subsequent to the specified recovery point in output order derived by starting the decoding process at the access unit associated with the current GRA picture shall be an exact match to the pictures that would be produced by starting the decoding process at the location of a previous non-GRA IRAP access unit, if any, in the bitstream.

When exact_match_flag is equal to 0, the quality of the approximation at the recovery point is chosen by the encoding process and is not specified in this Specification.

broken_link_flag indicates the presence or absence of a broken link in the NAL unit stream at the location of the current GRA picture and is assigned further semantics as follows:

–	If broken_link_flag is equal to 1, pictures produced by starting the decoding process at the location of a previous non-GRA IRAP access unit may contain undesirable visual artefacts to the extent that decoded pictures at and subsequent to the access unit associated with the current GRA picture in decoding order should not be displayed until the specified recovery point in output order.

–	Otherwise (broken_link_flag is equal to 0), no indication is given regarding any potential presence of visual artefacts.

Regardless of the value of the broken_link_flag, pictures subsequent to the specified recovery point in output order are specified to be correct or approximately correct in content.

Decoding process:

    - Decoding process for a coded picture

The decoding processes specified in this clause apply to each coded picture, referred to as the current picture and denoted by the variable CurrPic, in BitstreamToDecode.

Depending on the value of chroma_format_idc, the number of sample arrays of the current picture is as follows:

–	If chroma_format_idc is equal to 0, the current picture consists of 1 sample array SL.

–	Otherwise (chroma_format_idc is not equal to 0), the current picture consists of 3 sample arrays SL, SCb, SCr.

The decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause 7. When interpreting the semantics of each syntax element in each NAL unit, the term "the bitstream" (or part thereof, e.g., a CVS of the bitstream) refers to BitstreamToDecode (or part thereof).

When the current picture is an IRAP picture, the following applies:

–	If the current picture is an IDR picture, the first picture in the bitstream in decoding order, or the first picture that follows an end of sequence NAL unit in decoding order, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsCvsStartFlag to a value for the current picture, the variable HandleCraAsCvsStartFlag is set equal to the value provided by the external means and the variable NoRaslOutputFlag is set equal to HandleCraAsCvsStartFlag.

–	Otherwise, the variable HandleCraAsCvsStartFlag is set equal to 0 and the variable NoRaslOutputFlag is set equal to 0.

Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to one of the 3 sample arrays of the current picture, with the NAL units with colour_plane_id equal to 0, 1 and 2 being assigned to SL, SCb and SCr, respectively.

NOTE 1 – The variable ChromaArrayType is derived as equal to 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures (when chroma_format_idc is equal to 0).

When the current picture is GRA picture, the following applies:

–	If the current picture is a GRA picture, and is the first picture in the CVS or if a random access is started at the current picture, the variable GraFlag is set equal to 1.

–	Otherwise, the variable GraFlag is set equal to 0.

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in clause TBD.

- The processes in clauses TBD specify decoding processes using syntax elements in in the tile group layer and above.

  - When the current picture is a GRA picture with GraFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 0.1. (see proposal 2 below for this clause)  is invoked, which needs to be invoked only for the first tile group of a picture.

- PicOutputFlag is set as follows:

–	If the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–   Otherwise if GraFlag is equal to 1 and PicOrderCntVal of the current picture is less than RPPicOrderCntVal, PicOutputFlag is set equal to 0.

–   Otherwise, PicOutputFlag is set equal to pic_output_flag.

- If GraFlag is equal to 1, following applies:

–	If gra_output_flag is equal to 1, PicOrderCntVal of the current picture is greater than or equal to RPPicOrderCntVal, GraFlag is set equal to 0.

- The processes in clauses TBD specify decoding processes using syntax elements in all syntax structure layers.

In a variant the cyan highlighted part in step 3 may be omitted.

## Proposal 2

CRA pictures were included in VVC WD [1] in Marrakech meeting. It is asserted that when starting a CVS with CRA pictures, decoding process for generation of unavailable reference pictures should be specified for conformance purpose. It is asserted that this is currently missing in VVC WD [1]. The specification text is proposed for decoding process for generating unavailable reference pictures. This process is proposed to be invoked for decoding of CRA picture with NoRaslOutputFlag equal to 1.

Additionally if GRA functionality (e.g. Proposal 1 or similar other proposal) is agreed then it is also proposed to invoke the proposed decoding process for generation of unavailable reference pictures for GRA picture decoding.

The proposed changes are shown highlighted below for CRA functionality. Additionally, text for GRA functionality is shown highlighted.

    - General

Input to this process is a bitstream. Output of this process is a list of decoded pictures.

The decoding process is specified such that all decoders that conform to a specified profile and level will produce numerically identical cropped decoded output pictures when invoking the decoding process associated with that profile for a bitstream conforming to that profile and level. Any decoding process that produces identical cropped decoded output pictures to those produced by the process described herein (with the correct output order or output timing, as specified) conforms to the decoding process requirements of this Specification.

At the beginning of decoding a CVS, after activating the SPS RBSP that is active for the entire CVS and before decoding any VCL NAL units of the CVS, clause 8.1.2 is invoked with the CVS as input.

    - CVS decoding process

Input to this process is a CVS. Output of this process is a list of decoded pictures.

The variable HighestTid, which identifies the highest temporal sub-layer to be decoded, is specified as follows:

–	If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

–	Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1.

The sub-bitstream extraction process as specified in clause XX is applied with the CVS and HighestTid as inputs, and the output is assigned to a bitstream referred to as BitstreamToDecode.

Clause 8.1.3 is repeatedly invoked for each coded picture in BitstreamToDecode in decoding order.

    - Decoding process for a coded picture

The decoding processes specified in this clause apply to each coded picture, referred to as the current picture and denoted by the variable CurrPic, in BitstreamToDecode.

Depending on the value of chroma_format_idc, the number of sample arrays of the current picture is as follows:

–	If chroma_format_idc is equal to 0, the current picture consists of 1 sample array SL.

–	Otherwise (chroma_format_idc is not equal to 0), the current picture consists of 3 sample arrays SL, SCb, SCr.

The decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause 7. When interpreting the semantics of each syntax element in each NAL unit, the term "the bitstream" (or part thereof, e.g., a CVS of the bitstream) refers to BitstreamToDecode (or part thereof).

When the current picture is an IRAP picture, the following applies:

–	If the current picture is an IDR picture, the first picture in the bitstream in decoding order, or the first picture that follows an end of sequence NAL unit in decoding order, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsCvsStartFlag to a value for the current picture, the variable HandleCraAsCvsStartFlag is set equal to the value provided by the external means and the variable NoRaslOutputFlag is set equal to HandleCraAsCvsStartFlag.

–	Otherwise, the variable HandleCraAsCvsStartFlag is set equal to 0 and the variable NoRaslOutputFlag is set equal to 0.

Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to one of the 3 sample arrays of the current picture, with the NAL units with colour_plane_id equal to 0, 1 and 2 being assigned to SL, SCb and SCr, respectively.

NOTE 1 – The variable ChromaArrayType is derived as equal to 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures (when chroma_format_idc is equal to 0).

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in clause TBD.

- The processes in clauses TBD specify decoding processes using syntax elements in in the tile group layer and above.

  - When the current picture is a CRA picture with NoRaslOutputFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 0.1.1 is invoked, which needs to be invoked only for the first tile group of a picture.

  - When the current picture is a GRA picture with GraFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 0.1.1 is invoked, which needs to be invoked only for the first tile group of a picture.

- The processes in clauses TBD specify decoding processes using syntax elements in all syntax structure layers.

### Decoding process for generating unavailable reference pictures

#### General decoding process for generating unavailable reference pictures

This process is invoked once per coded picture when the current picture is a CRA picture with NoRaslOutputFlag equal to 1.

When this process is invoked, the following applies:

- For each 	RefPicList[ i ][ j ], with i in the range of 0 to 1, inclusive, for j in the range of 0 to num_ref_entries[ i ][ RplsIdx[ i ] ] − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 0.1.1.2 (Generation of one unavailable picture)  and the following applies:

- If st_ref_pic_flag[ i ][ RplsIdx[ i ] ] [ j ] is equal to 1, the value of PicOrderCntVal for the generated picture is set equal to RefPicPocList[ i ][ j ].

Otherwise (i.e. if st_ref_pic_flag[ i ][ RplsIdx[ i ] ] [ j ] is equal to 0), the value of PicOrderCntVal for the generated picture is set equal to PicOrderCntVal − poc_lsb_lt[ i ][ RplsIdx[ i ] ][ j ] – (delta_poc_msb_present_flag[ i ][ j ]? (DeltaPocMsbCycleLt[ i ][ j ] * MaxPicOrderCntLsb):0)

- The value of PicOutputFlag for the generated picture is set equal to 0.

- If st_ref_pic_flag[ i ][ RplsIdx[ i ] ] [ j ] is equal to 1 (i.e. it it an STRP entry), the generated picture is marked as "used for short-term reference".

- Otherwise (i.e. if st_ref_pic_flag[ i ][ RplsIdx[ i ] ] [ j ] is equal to 0) (i.e. it it an LTRP entry), the generated picture is marked as "used for long-term reference".

- RefPicList[ i ][ j ] is set to be the generated reference picture.

#### Generation of one unavailable picture

When this process is invoked, an unavailable picture is generated as follows:

–	The value of each element in the sample array SL for the picture is set equal to 1  <<  ( BitDepthY − 1 ).

–	When ChromaArrayType is not equal to 0, the value of each element in the sample arrays SCb and SCr for the picture is set equal to 1  <<  ( BitDepthC − 1 ).

–	The prediction mode CuPredMode[ x ][ y ] is set equal to MODE_INTRA for x = 0..pic_width_in_luma_samples − 1, y = 0..pic_height_in_luma_samples − 1.

# Conclusion

This document proposes gradual random access signalling. Additionally decoding process for generation of unavailable reference pictures is proposed and it is proposed to invoke this process for CRA and GRA picture decoding. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-M1001-v6, Versatile Video Coding (Draft 4), Marrakech, March 2019