| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0104 |
| --- | --- |

| Title: | CE3-3.5: Explicitly signal non angular modes in intra mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Yao, J.-Q. Zhu, W.-T. Cai, K. Kazui 601,Yongda International Tower, 2277 Longyang Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5809 yaojie@cn.fujitsu.com zhujianqing@cn.fujitsu.com caiwenting@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Source: | Fujitsu |  |  |

_____________________________

- Abstract

This contribution presents the results of explicitly signal non angular modes on top of VTM-4.0. Harmonization was made to deal with new element of ISP [1] introduced in VTM-4.0.

The test CE3-3.5.1.x evaluates the proposed method with 6 MPM while test CE3-3.5.2.x reduced to 4 MPM. Test CE3-3.5.x.b is the method implemented on another codebase with encoder modification that evaluates all the candidates in the MPM list during the R-D mode decision. The results reportedly show an average luma BD rate performance of -0.09%, -0.12%, -0.05%, -0.07% (AI), 0.xx%, 0.xx%, 0.xx%, 0.xx% (RA) for CE3-3.5.1.a, 3.5.1.b, 3.5.2.a and 3.5.2.b respectively. It shows 6 MPM slightly outperforms 4 MPM. And the performance are consistent between 2 codebase.

Introduction

Unifying MPM derivation process by explicitly signal non angular modes (Planar, DC) was first proposed in JVET-M0210 [2] and tested in VTM-3.0. It propose to signal a syntax (intra_luma_non_ang_flag) at first for intra block to indicate whether IntraMode is angular or non angular. Figure 1 shows the coding process of the method. For non_angular mode, a syntax (intra_luma_planar_flag) is signaled to indicate whether IntraMode is Planar or DC. For angular modes, MPM list is used and is derived only using angular modes which is the same as the one currently being used for non-zero reference line index in VTM. Figure 2 shows details of the MPM list generation process.

VTM-4.0 introduces a new tool, Intra Sub-Partitions(ISP). The method in CE3-3.5.1 and CE3-3.5.1 also consider this new item.

VTM-3.0

parse intra_luma_ref_idx

if (intra_luma_ref_idx == 0)

parse intra_luma_mpm_flag

MPM without Planar&DC

else

MPM with Planar&DC

JVET-M0210

parse intra_luma_non_ang_flag

if(intra_luma_non_ang_flag == 1)

parse intra_luma_planar_flag

else

parse intra_luma_mpm_flag

if (intra_luma_mpm_flag)

parse intra_luma_ref_idx

MPM without Planar&DC

Figure 1. coding process of explicitly signal non angular modes

maxAB = max (leftIntraDir, aboveIntraDir)

minAB = min (leftIntraDir, aboveIntraDir)

if maxAB != minAB, minAB > DC

candModeList[ 0 ] = leftIntraDir

candModeList[ 1 ] = aboveIntraDir

if maxAB − minAB == 1

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )

candModeList[ 4 ] = 2 + ( ( minAB + 60 ) % 64 )

candModeList[ 5 ] = 2 + ( maxAB % 64 )

else if maxAB − minAB == 2

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )

candModeList[ 3 ] = 2 + ( ( minAB + 61 ) % 64 )

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )

candModeList[ 5 ] = 2 + ( ( minAB + 60 ) % 64 )

else if maxAB − minAB > 61

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )

candModeList[ 4 ] = 2 + ( minAB % 64 )

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )

else

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )

candModeList[ 3 ] = 2 + ( ( minAB − 1 ) % 64 )

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )

else if (maxAB == minAB, maxAB > DC) or (maxAB != minAB , maxAB > DC)

candModeList[ 0 ] = maxAB

candModeList[ 1 ] = 2 + ( (  maxAB  + 61 ) % 64 )

candModeList[ 2 ] = 2 + ( ( maxAB − 1 ) % 64 )

candModeList[ 3 ] = 2 + ( (  maxAB  + 60 ) % 64 )

candModeList[ 4 ] = 2 + ( maxAB % 64 )

candModeList[ 5 ] = 2 + ( ( maxAB + 59 ) % 64 )

else

candModeList[ 0 ] = VER

candModeList[ 1 ] = HOR

candModeList[ 2 ] = 2

candModeList[ 3 ] = DIA

candModeList[ 4 ] = 66

candModeList[ 5 ] = 26

Figure 2. MPM list generation process for angular modes

Harmonization with IPS

Interaction with Intra Sub-Partitions(ISP)

The coding process of the proposed method is shown in Figure 3. For non-angular modes. If the intra_luma_planar_flag == 0 (in another word intra mode is DC), then the intra_subpartitions_mode_flag will be inferred to be 0 and therefore it will not be sent to the decoder. For angular modes. MPM flag (intra_luma_mpm_flag), reference line index (intra_luma_ref_idx), and ISP mode (intra_subpartitions_mode_flag) are sequentially coded. The intra_subpartitions_mode_flag will also be inferred to be 0 and will not be sent to the decoder in the following 2 conditions: intra_luma_mpm_flag == 0; intra_luma_mpm_flag == 1 and intra_luma_ref_idx == 1.

VTM-4.0

parse intra_luma_ref_idx

if (intra_luma_ref_idx == 0)

parse intra_subpartitions_mode_flag

if (intra_subpartitions_mode_flag == 0)

parse intra_luma_mpm_flag

MPM with Planar&DC

else

MPM without DC

else

MPM without Planar&DC

Proposed

parse intra_luma_non_ang_flag

if(intra_luma_non_ang_flag == 1)

// no MPM

parse intra_luma_planar_flag

if (intra_luma_planar_flag)

parse intra_subpartitions_mode_flag

else

parse intra_luma_mpm_flag

if (intra_luma_mpm_flag)

parse intra_luma_ref_idx

if (intra_luma_ref_idx == 0)

parse intra_subpartitions_mode_flag

MPM without Planar&DC

Figure 3. coding process modifications of the proposed method

Syntax and semantics modifications

The detailed syntax and semantics modifications to VVC WD [3] can be found in attached document.

CE Results

According to the CE3 description in JVET-M1023 [4] and the common test conditions in JVET-M1010 [5], the proposed method is evaluated for all-intra and random-access configuration with VTM 4.0. For tests 3.5.x.a, the anchor is VTM-4.0. For tests 3.5.x.b, the anchor is the modified VTM-4.0(VTM-4.0_all MPM) with bellowing SW changes which includes all MPM modes in full encoder RD check:

#if ALL_MPM_RD

for (int j = 0; j < NUM_MOST_PROBABLE_MODES; j++)

#else

for (int j = 0; j < numCand; j++)

#endif

The following table is an overview over the scheduled CE tests 3.5.1 and 3.5.2.

| Test # | Description |
| --- | --- |
| CE3-3.5.1.a | Explicitly signal Planar&DC MPM list and intra_luma_ref_idx signalling only for angular mode |
| CE3-3.5.1.b | Test 3.5.1.a, always include all MPMs(6 MPMs in VTM-4.0) for full encoder R-D checking |
| CE3-3.5.2.a | Test 3.5.1.a, use 4MPMs described in JVET-M0494 |
| CE3-3.5.2.b | Test 3.5.2.a, always include all MPMs(6 MPMs in VTM-4.0) for full encoder R-D checking |

CE3-3.5.1.a

Anchor is VTM-4.0. Encoding time for RA is not reliable.

Table 1. Experiment results of CE3-3.5.1.a

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.28% | -0.19% | 94% | 98% |
| Class A2 | -0.07% | 0.05% | -0.09% | 93% | 99% |
| Class B | -0.10% | -0.05% | -0.04% | 95% | 98% |
| Class C | 0.02% | -0.02% | 0.04% | 107% | 99% |
| Class E | -0.03% | 0.02% | -0.11% | 110% | 96% |
| Overall | -0.09% | -0.06% | -0.07% | 100% | 98% |
| Class D | 0.01% | -0.09% | -0.25% | 104% | 97% |
| Class F | 0.12% | -0.08% | 0.00% | 104% | 97% |

CE3-3.5.1.b

Anchor is VTM-4.0_all MPM. Encoding time for RA is not reliable.

Table 2. Experiment results of CE3-3.5.1.b

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 + all MPM |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | -0.24% | -0.14% | 100% | 100% |
| Class A2 | -0.08% | -0.12% | -0.08% | 89% | 99% |
| Class B | -0.14% | -0.11% | -0.09% | 98% | 98% |
| Class C | -0.07% | -0.07% | 0.05% | 107% | 99% |
| Class E | -0.04% | -0.05% | 0.11% | 105% | 102% |
| Overall | -0.12% | -0.12% | -0.03% | 100% | 99% |
| Class D | -0.06% | 0.06% | -0.01% | 122% | 98% |
| Class F | 0.06% | -0.06% | -0.07% | 89% | 96% |

CE3-3.5.2.a

Anchor is VTM-4.0. Encoding time for RA is not reliable.

Table 3. Experiment results of CE3-3.5.2.a

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.27% | -0.22% | 101% | 98% |
| Class A2 | -0.03% | 0.04% | -0.07% | 96% | 97% |
| Class B | -0.07% | -0.07% | 0.00% | 97% | 96% |
| Class C | 0.09% | 0.00% | 0.08% | 101% | 100% |
| Class E | 0.05% | 0.03% | 0.07% | 105% | 97% |
| Overall | -0.05% | -0.05% | -0.02% | 100% | 98% |
| Class D | 0.06% | -0.03% | -0.07% | 113% | 102% |
| Class F | 0.18% | 0.09% | 0.11% | 102% | 96% |

CE3-3.5.2.b

Anchor is VTM-4.0_all MPM. Encoding time for RA is not reliable.

Table 4. Experiment results of CE3-3.5.2.b

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 + all MPM |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.32% | -0.17% | 96% | 100% |
| Class A2 | -0.04% | -0.07% | -0.03% | 88% | 100% |
| Class B | -0.09% | -0.10% | -0.09% | 95% | 99% |
| Class C | -0.01% | -0.03% | -0.07% | 109% | 99% |
| Class E | 0.05% | 0.01% | 0.12% | 116% | 102% |
| Overall | -0.07% | -0.10% | -0.05% | 100% | 100% |
| Class D | -0.02% | 0.06% | 0.09% | 136% | 99% |
| Class F | 0.13% | -0.02% | -0.02% | 89% | 98% |

References

- Santiago De-Luxán-Hernández, Valeri George, and etc, “CE3: Intra Sub-Partitions Coding Mode (Tests 1.1.1 and 1.1.2),” JVET-M0102, Jan 2019.

- Jie Yao, Jianqing Zhu, Wenting Cai and Kimihiko Kazui, “Non-CE3: Intra prediction information coding,” JVET-M0210, Jan 2019.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” JVET-M1001, Jan 2019.

- Geert Van der Auwera, Ling Li, Alexey Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-M1023, Jan 2019.

- F. Bossen, J. Boyce, and etc, “JVET common test conditions and software reference configurations for SDR video,” JVET document, JVET-M1010, Jan 2019.

Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).