Modification to VVC WD are highlighted in yellow below:

      - Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( tile_group_type != I ) { |  |
| if( treeType != DUAL_TREE_CHROMA ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( sps_pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| intra_luma_non_ang_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_non_ang_flag[ x0 ][ y0 ] ) |  |
| intra_luma_planar_flag[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] && ( y0 % CtbSizeY ) > 0 ) |  |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( ( ( intra_luma_mpm_flag[ x0 ][ y0 ] && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) ¦¦ intra_luma_planar_flag[ x0 ][ y0 ] ) && ( cbWidth <= MaxTbSizeY ¦ ¦ cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY ¦ ¦ cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY )) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_non_ang_flag[ x0 ][ y0 ] == 0 ) |  |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_data( x0, y0, cbWidth, cbHeight ) |  |
| } else { |  |
| if( tile_group_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ] = 0 |  |
| } else { |  |
| mvd_coding( x0, y0, 1, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 1, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 1, 2 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( ( sps_amvr_enabled_flag && inter_affine_flag = = 0 && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 1 ] != 0 ) ) ¦ ¦ ( sps_affine_amvr_enabled_flag && inter_affine_flag[ x0 ][ y0 ] = = 1 && ( MvdCpL0[ x0 ][ y0 ][ 0 ] [ 0 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 0 ] [ 1 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 0 ] [ 0 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 0 ] [ 1 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 1 ] [ 0 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 1 ] [ 1 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 1 ] [ 0 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 1 ] [ 1 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 2 ] [ 0 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 2 ] [ 1 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 2 ] [ 0 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 2 ] [ 1 ] != 0 ) ) { |  |
| if( !sps_cpr_enabled_flag ¦ ¦ !( inter_pred_idc[ x0 ][ y0 ] = = PRED_L0 && ref_idx_l0[ x0 ][ y0 ] = = num_ref_idx_l0_active_minus1 ) ) |  |
| amvr_flag[ x0 ][ y0 ] | ae(v) |
| if( amvr_flag[ x0 ][ y0 ] ) |  |
| amvr_precision_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( sps_gbi_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && cbWidth * cbHeight >= 256 ) |  |
| gbi_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && merge_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] ) { |  |
| if( cbWidth <= MaxSbtSize && cbHeight <= MaxSbtSize ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ¦ ¦ allowSbtVerQ ¦ ¦ allowSbtHorQ ) |  |
| cu_sbt_flag | ae(v) |
| } |  |
| if( cu_sbt_flag ) { |  |
| if( ( allowSbtVerH ¦ ¦ allowSbtHorH ) && ( allowSbtVerQ ¦ ¦ allowSbtHorQ) ) |  |
| cu_sbt_quad_flag | ae(v) |
| if( ( cu_sbt_quad_flag && allowSbtVerQ && allowSbtHorQ ) ¦ ¦ ( !cu_sbt_quad_flag && allowSbtVerH && allowSbtHorH ) ) |  |
| cu_sbt_horizontal_flag | ae(v) |
| cu_sbt_pos_flag | ae(v) |
| } |  |
| } |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |
| } |  |
| } |  |

- 7.4.5.6 Coding unit semantics

pcm_flag[ x0 ][ y0 ] equal to 1 specifies that the pcm_sample( ) syntax structure is present and the transform_tree( ) syntax structure is not present in the coding unit including the luma coding block at the location ( x0, y0 ). pcm_flag[ x0 ][ y0 ] equal to 0 specifies that pcm_sample( ) syntax structure is not present. When pcm_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

The value of pcm_flag[ x0 + i ][ y0 + j ] with i = 1..cbWidth − 1, j = 1..cbHeight − 1 is inferred to be equal to pcm_flag[ x0 ][ y0 ].

pcm_alignment_zero_bit is a bit equal to 0.

intra_luma_non_ang_flag[ x0 ][ y0 ] equal to  1 specifies that the intra prediction mode for luma samples is not angular. intra_luma_non_ang_flag[ x0 ][ y0 ] equal to 0 specifies that the intra prediction mode for luma samples is angular. When intra_luma_non_ang_flag[ x0 ][ y0 ] is not present it is inferred to be equal to 0.

When intra_luma_non_ang_flag[ x0 ][ y0 ] is equal to 1, intra_luma_planar_flag[ x0 ][ y0 ] specify intra prediction mode for luma samples is PLANAR or DC. intra_luma_planar_flag[ x0 ][ y0 ] equal to 1 specifies that the intra prediction mode for luma samples is PLANAR. intra_luma_planar_flag[ x0 ][ y0 ] equal to  0 specifies that the intra prediction mode for luma samples is DC. When intra_luma_planar_flag[ x0 ][ y0 ] is not present it is inferred to be equal to 0.

When intra_luma_mpm_flag[ x0 ][ y0 ] is equal to 1, intra_luma_ref_idx[ x0 ][ y0 ] specifies the intra prediction reference line index IntraLumaRefLineIdx[ x ][ y ] for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1 as specified in Table 7-6.

When intra_luma_ref_idx[ x0 ][ y0 ] is not present it is inferred to be equal to 0.

Table 7-6 – Specification of IntraLumaRefLineIdx[ x ][ y ] based on intra_luma_ref_idx[ x0 ][ y0 ].

| intra_luma_ref_idx[ x0 ][ y0 ] | IntraLumaRefLineIdx[ x ][ y ] x = x0..x0 + cbWidth − 1 y = y0..y0 + cbHeight − 1 |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 3 |

When intra_luma_mpm_flag[ x0 ][ y0 ] is equal to 1 and intra_luma_ref_idx[ x0 ][ y0 ] is equal to 0, or intra_luma_planar_flag[ x0 ][ y0 ] is equal to 1, intra_subpartitions_mode_flag[ x0 ][ y0 ] equal to 1 specifies that the current intra coding unit is partitioned into NumIntraSubPartitions[ x0 ][ y0 ] rectangular transform block subpartitions. intra_subpartitions_mode_flag[ x0 ][ y0 ] equal to 0 specifies that the current intra coding unit is not partitioned into rectangular transform block subpartitions.

When intra_subpartitions_mode_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

intra_subpartitions_split_flag[ x0 ][ y0 ] specifies whether the intra subpartitions split type is horizontal or vertical. When intra_subpartitions_split_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If cbHeight is greater than MaxTbSizeY, intra_subpartitions_split_flag[ x0 ][ y0 ] is inferred to be equal to 0.

- Otherwise (cbWidth is greater than MaxTbSizeY), intra_subpartitions_split_flag[ x0 ][ y0 ] is inferred to be equal to 1.

The variable IntraSubPartitionsSplitType specifies the type of split used for the current luma coding block as illustrated in Table 7-7. IntraSubPartitionsSplitType is derived as follows:

- If intra_subpartitions_mode_flag[ x0 ][ y0 ] is equal to 0, IntraSubPartitionsSplitType is set equal to 0.

- Otherwise, the IntraSubPartitionsSplitType is set equal to 1 + intra_subpartitions_split_flag[ x0 ][ y0 ].

Table 7-7 – Name association to IntraSubPartitionsSplitType

| IntraSubPartitionsSplitType | Name of IntraSubPartitionsSplitType |
| --- | --- |
| 0 | ISP_NO_SPLIT |
| 1 | ISP_HOR_SPLIT |
| 2 | ISP_VER_SPLIT |

The variable NumIntraSubPartitions specifies the number of transform block subpartitions an intra luma coding block is divided into. NumIntraSubPartitions is derived as follows:

- If IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, NumIntraSubPartitions is set equal to 1.

- Otherwise, if one of the following conditions is true, NumIntraSubPartitions is set equal to 2:

  - cbWidth is equal to 4 and cbHeight is equal to 8,

  - cbWidth is equal to 8 and cbHeight is equal to 4.

- Otherwise, NumIntraSubPartitions is set equal to 4.

When intra_luma_non_ang_flag[ x0 ][ y0 ] is equal to 0, the syntax elements intra_luma_mpm_flag[ x0 ][ y0 ], intra_luma_mpm_idx[ x0 ][ y0 ] and intra_luma_mpm_remainder[ x0 ][ y0 ] specify the angular intra prediction mode for luma samples. The array indices x0, y0 specify the location ( x0 , y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When intra_luma_mpm_flag[ x0 ][ y0 ] is equal to 1, the intra prediction mode is inferred from a neighbouring intra-predicted coding unit according to clause 8.3.2.

When intra_luma_mpm_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 10.

intra_chroma_pred_mode[ x0 ][ y0 ] specifies the intra prediction mode for chroma samples. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

mvp_l0_flag[ x0 ][ y0 ] specifies the motion vector predictor index of list 0 where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When mvp_l0_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

mvp_l1_flag[ x0 ][ y0 ] has the same semantics as mvp_l0_flag, with l0 and list 0 replaced by l1 and list 1, respectively.

inter_pred_idc[ x0 ][ y0 ] specifies whether list0, list1, or bi-prediction is used for the current coding unit according to Table 7-8. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

Table 7-8 – Name association to inter prediction mode

| inter_pred_idc | Name of inter_pred_idc |  |
| --- | --- | --- |
|  | ( cbWidth + cbHeight ) != 8 | ( cbWidth + cbHeight ) = = 8 |
| 0 | PRED_L0 | PRED_L0 |
| 1 | PRED_L1 | PRED_L1 |
| 2 | PRED_BI | n.a. |

When inter_pred_idc[ x0 ][ y0 ] is not present, it is inferred to be equal to PRED_L0.

ref_idx_l0[ x0 ][ y0 ] specifies the list 0 reference picture index for the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When ref_idx_l0[ x0 ][ y0 ] is not present it is inferred to be equal to 0.

It is a requirement of bitstream conformance that when the reference picture corresponding to ref_idx_l0[ x0 ][ y0 ] for the current coding unit is the current decoded picture, inter_pred_idc[ x0 ][ y0 ] shall be equal to 0.

ref_idx_l1[ x0 ][ y0 ] has the same semantics as ref_idx_l0, with l0 and list 0 replaced by l1 and list 1, respectively.

inter_affine_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, when decoding a P or B tile group, affine model based motion compensation is used to generate the prediction samples of the current coding unit. inter_affine_flag[ x0 ][ y0 ] equal to 0 specifies that the coding unit is not predicted by affine model based motion compensation. When inter_affine_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

cu_affine_type_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, when decoding a P or B tile group, 6-parameter affine model based motion compensation is used to generate the prediction samples of the current coding unit. cu_affine_type_flag[ x0 ][ y0 ] equal to 0 specifies that 4-parameter affine model based motion compensation is used to generate the prediction samples of the current coding unit.

MotionModelIdc[ x ][ y ] represents motion model of a coding unit as illustrated in Table 7-9. The array indices x, y specify the luma sample location ( x, y ) relative to the top-left luma sample of the picture.

The variable MotionModelIdc[ x ][ y ] is derived as follows for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

- If merge_flag[ x0 ][ y0 ] is equal to 1, the following applies:

MotionModelIdc[ x ][ y ] = merge_subblock_flag[ x0 ][ y0 ]	(7-78)

- Otherwise (merge_flag[ x0 ][ y0 ] is equal to 0), the following applies:

MotionModelIdc[ x ][ y ] = inter_affine_flag[ x0 ][ y0 ] + cu_affine_type_flag[ x0 ][ y0 ]	(7-79)

Table 7-9 – Interpretation of MotionModelIdc[ x0 ][ y0 ]

| MotionModelIdc[ x ][ y ] | Motion model for motion compensation |
| --- | --- |
| 0 | Translational motion |
| 1 | 4-parameter affine motion |
| 2 | 6-parameter affine motion |

It is a requirement of bitstream conformance that when the reference picture corresponding to ref_idx_l0[ x0 ][ y0 ] for the current coding unit is the current decoded picture, MotionModelIdc[ x ][ y ] shall be equal to 0.

amvr_flag[ x0 ][ y0 ] specifies the resolution of motion vector difference. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. amvr_flag[ x0 ][ y0 ] equal to 0 specifies that the resolution of the motion vector difference is 1/4 of a luma sample. amvr_flag[ x0 ][ y0 ] equal to 1 specifies that the resolution of the motion vector difference is further specified by amvr_precision_flag[ x0 ][ y0 ].

When amvr_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If sps_cpr_enabled_flag is equal to 1, amvr_flag[ x0 ][ y0 ] is inferred to be equal to 1.

- Otherwise ( sps_cpr_enabled_flag is equal to 0 ), amvr_flag[ x0 ][ y0 ] is inferred to be equal to 0.

amvr_precision_flag[ x0 ][ y0 ] equal to 0 specifies that the resolution of the motion vector difference is one integer luma sample if inter_affine_flag[ x0 ][ y0 ] is equal to 0, and 1/16 of a luma sample otherwise. amvr_precision_flag[ x0 ][ y0 ] equal to 1 specifies that the resolution of the motion vector difference is four luma samples if inter_affine_flag[ x0 ][ y0 ] is equal to 0, and one integer luma sample otherwise. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When amvr_precision_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

The motion vector differences are modified as follows:

- If inter_affine_flag[ x0 ][ y0 ] is equal to 0, the variable MvShift is derived and the variables MvdL0[ x0 ][ y0 ][ 0 ], MvdL0[ x0 ][ y0 ][ 1 ], MvdL1[ x0 ][ y0 ][ 0 ], MvdL1[ x0 ][ y0 ][ 1 ] are modified as follows:

MvShift = ( amvr_flag[ x0 ][ y0 ] + amvr_precision_flag[ x0 ][ y0 ] ) << 1	(7-80)

MvdL0[ x0 ][ y0 ][ 0 ] = MvdL0[ x0 ][ y0 ][ 0 ]  << ( MvShift + 2 )	(7-81)

MvdL0[ x0 ][ y0 ][ 1 ] = MvdL0[ x0 ][ y0 ][ 1 ]  <<  ( MvShift + 2 )	(7-82)

MvdL1[ x0 ][ y0 ][ 0 ] = MvdL1[ x0 ][ y0 ][ 0 ]<<( MvShift + 2 )	(7-83)

MvdL1[ x0 ][ y0 ][ 1 ] = MvdL1[ x0 ][ y0 ][ 1 ]  << ( MvShift + 2 )	(7-84)

- Otherwise (inter_affine_flag[ x0 ][ y0 ] is equal to 1), the variable MvShift is derived and the variables MvdCpL0[ x0 ][ y0 ][ 0 ][ 0 ], MvdCpL0[ x0 ][ y0 ][ 0 ][ 1 ], MvdCpL0[ x0 ][ y0 ][ 1 ][ 0 ], MvdCpL0[ x0 ][ y0 ][ 1 ][ 1 ], MvdCpL0[ x0 ][ y0 ][ 2 ][ 0 ] and MvdCpL0[ x0 ][ y0 ][ 2 ][ 1 ] are modified as follows:

MvShift = amvr_coarse_precisoin_flag[ x0 ][ y0 ]  ? 
				 ( amvr_coarse_precisoin_flag[ x0 ][ y0 ]  <<  1 )  :  ( − (amvr_flag[ x0 ][ y0 ]  <<  1) ) )	(7-85)

MvdCpL0[ x0 ][ y0 ][ 0 ][ 0 ] = MvdCpL0[ x0 ][ y0 ][ 0 ][ 0 ]  << ( MvShift + 2 )	(7-86)

MvdCpL1[ x0 ][ y0 ] [ 0 ][ 1 ] = MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ]  <<  ( MvShift + 2 )	(7-87)

MvdCpL0[ x0 ][ y0 ][ 1 ][ 0 ] = MvdCpL0[ x0 ][ y0 ][ 1 ][ 0 ]  << ( MvShift + 2 )	(7-88)

MvdCpL1[ x0 ][ y0 ] [ 1 ][ 1 ] = MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ]  <<  ( MvShift + 2 )	(7-89)

MvdCpL0[ x0 ][ y0 ][ 2 ][ 0 ] = MvdCpL0[ x0 ][ y0 ][ 2 ][ 0 ]  << ( MvShift + 2 )	(7-90)

MvdCpL1[ x0 ][ y0 ] [ 2 ][ 1 ] = MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ]  <<  ( MvShift + 2 )	(7-91)

gbi_idx[ x0 ][ y0 ] specifies the weight index of bi-prediction with CU weights. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When gbi_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

cu_cbf equal to 1 specifies that the transform_tree( ) syntax structure is present for the current coding unit. cu_cbf equal to 0 specifies that the transform_tree( ) syntax structure is not present for the current coding unit.

When cu_cbf is not present, it is inferred as follows:

- If cu_skip_flag[ x0 ][ y0 ] is equal to 1, cu_cbf is inferred to be equal to 0.

- Otherwise, cu_cbf is inferred to be equal to 1.

cu_sbt_flag equal to 1 specifies that for the current coding unit, subblock transform is used. cu_sbt_flag equal to 0 specifies that for the current coding unit, subblock transform is not used.

When cu_sbt_flag is not present, its value is inferred to be equal to 0.

NOTE – : When subblock transform is used, a coding unit is split into two transform units; one transform unit has residual data, the other does not have residual data.

cu_sbt_quad_flag equal to 1 specifies that for the current coding unit, the subblock transform includes a transform unit of 1/4 size of the current coding unit. cu_sbt_quad_flag equal to 0 specifies that for the current coding unit the subblock transform includes a transform unit of 1/2 size of the current coding unit.

When cu_sbt_quad_flag is not present, its value is inferred to be equal to 0.

cu_sbt_horizontal_flag equal to 1 specifies that the current coding unit is split horizontally into 2 transform units. cu_sbt_horizontal_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is split vertically into 2 transform units.

When cu_sbt_horizontal_flag is not present, its value is derived as follows:

- If cu_sbt_quad_flag is equal to 1, cu_sbt_horizontal_flag is set to be equal to allowSbtHorQ.

- Otherwise (cu_sbt_quad_flag is equal to 0), cu_sbt_horizontal_flag is set to be equal to allowSbtHorH.

cu_sbt_pos_flag equal to 1 specifies that the tu_cbf_luma, tu_cbf_cb and tu_cbf_cr of the first transform unit in the current coding unit are not present in the bitstream. cu_sbt_pos_flag equal to 0 specifies that the tu_cbf_luma, tu_cbf_cb and tu_cbf_cr of the second transform unit in the current coding unit are not present in the bitstream.

The variable SbtNumFourthsTb0 is derived as follows:

sbtMinNumFourths = cu_sbt_quad_flag  ?  1  :  2		(7-92)

SbtNumFourthsTb0 = cu_sbt_pos_flag  ?  ( 4 − sbtMinNumFourths )  :  sbtMinNumFourths	(7-93)

### 8.3.2 Derivation process for luma intra prediction mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

Table 8-1 specifies the value for the intra prediction mode IntraPredModeY[ xCb ][ yCb ] and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |
| 81..83 | INTRA_LT_CCLM, INTRA_L_CCLM, INTRA_T_CCLM |

NOTE – : The intra prediction modes INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM are only applicable to chroma components.

IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb + cbHeight − 1 ) and ( xCb + cbWidth − 1, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If one or more of the following conditions are true, candIntraPredModeX is set equal to INTRA_PLANAR.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA and mh_intra_flag[ xNbX ][ yNbX ] is not equal to 1.

- pcm_flag[ xNbX ][ yNbX ] is equal to 1.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The variables ispDefaultMode1 and ispDefaultMode2 are defined as follows:

- If IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT, ispDefaultMode1 is set equal to INTRA_ANGULAR18 and ispDefaultMode2 is set equal to INTRA_ANGULAR5.

- Otherwise, ispDefaultMode1 is set equal to INTRA_ANGULAR50 and ispDefaultMode2 is set equal to INTRA_ANGULAR63.

- The candModeList[ x ] with x = 0..5 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA and candIntraPredModeA is greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-8)

candModeList[ 1 ] = INTRA_PLANAR	(8-9)

candModeList[ 2 ] = INTRA_DC		(8-10)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-11)

candModeList[ 4 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-12)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-13)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0 or IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT), the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-14)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-15)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-16)

  - If one of the following conditions is true,

  - IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT and candIntraPredModeA  is less than INTRA_ANGULAR34,

  - IntraSubPartitionsSplitType is equal to ISP_VER_SPLIT and candIntraPredModeA  is greater than or equal to INTRA_ANGULAR34,

  - IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0,

the following applies:

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-17)

candModeList[ 4 ] = 2 + ( candIntraPredModeA % 64 )	(8-18)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 59 ) % 64 )	(8-19)

  - Otherwise, the following applies:

candModeList[ 3 ] = ispDefaultMode1	(8-20)

candModeList[ 4 ] = ispDefaultMode2	(8-21)

candModeList[ 5 ] = INTRA_PLANAR	(8-22)

- Otherwise if candIntraPredModeB is not equal to candIntraPredModeA and candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC, the following applies:

  - The variables minAB and maxAB are derived as follows:

minAB = Min( candIntraPredModeA, candIntraPredModeB )	(8-23)

maxAB = Max( candIntraPredModeA, candIntraPredModeB )	(8-24)

  - If candIntraPredModeA and candIntraPredModeB are both greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-25)

candModeList[ 1 ] = candIntraPredModeB	(8-26)

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 2 ] = INTRA_PLANAR	(8-27)

candModeList[ 3 ] = INTRA_DC		(8-28)

  - If maxAB − minAB is in the range of 2 to 62, inclusive, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-29)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-30)

  - Otherwise, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-31)

candModeList[ 5 ] = 2 + ( ( maxAB ) % 64 )	(8-32)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0 or IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT), the following applies:

  - When IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT, and abs( candIntraPredModeB − ispDefaultMode1 ) is less than abs( candIntraPredModeA − ispDefaultMode1 ), the following applies:

candModeList[ 0 ] = candIntraPredModeB	(8-33)

candModeList[ 1 ] = candIntraPredModeA	(8-34)

  - If maxAB − minAB is equal to 1, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-35)

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-36)

candModeList[ 4 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-37)

candModeList[ 5 ] = 2 + ( maxAB % 64 )	(8-38)

  - Otherwise if maxAB − minAB is equal to 2, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-39)

candModeList[ 3 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-40)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-41)

candModeList[ 5 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-42)

  - Otherwise if maxAB − minAB is greater than 61, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-43)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-44)

candModeList[ 4 ] = 2 + ( minAB % 64 )	(8-45)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-46)

  - Otherwise, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-47)

candModeList[ 3 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-48)

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-49)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-50)

  - Otherwise (candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC), candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-51)

candModeList[ 1 ] = candIntraPredModeB	(8-52)

candModeList[ 2 ] = 1 − minAB		(8-53)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-54)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-55)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-56)

  - Otherwise, if IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0, the following applies:

candModeList[ 0 ] = maxAB		(8-57)

candModeList[ 1 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-58)

candModeList[ 2 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-59)

candModeList[ 3 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-60)

candModeList[ 4 ] = 2 + ( maxAB % 64 )	(8-61)

candModeList[ 5 ] = 2 + ( ( maxAB + 59 ) % 64 )	(8-62)

  - Otherwise (IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT), the following applies:

candModeList[ 0 ] = INTRA_PLANAR	(8-63)

candModeList[ 1 ] = maxAB		(8-64)

candModeList[ 2 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-65)

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-66)

candModeList[ 4 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-67)

candModeList[ 5 ] = 2 + ( maxAB % 64 )	(8-68)

- Otherwise, the following applies:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-69)

candModeList[ 1 ] = ( candModeList[0]  = =  INTRA_PLANAR )  ?  INTRA_DC  :  	(8-70) 
		INTRA_PLANAR

candModeList[ 2 ] = INTRA_ANGULAR50	(8-71)

candModeList[ 3 ] = INTRA_ANGULAR18	(8-72)

candModeList[ 4 ] = INTRA_ANGULAR46	(8-73)

candModeList[ 5 ] = INTRA_ANGULAR54	(8-74)

  - Otherwise, if IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0, the following applies:

candModeList[ 0 ] = INTRA_ANGULAR50	(8-75)

candModeList[ 1 ] = INTRA_ANGULAR18	(8-76)

candModeList[ 2 ] = INTRA_ANGULAR2	(8-77)

candModeList[ 3 ] = INTRA_ANGULAR34	(8-78)

candModeList[ 4 ] = INTRA_ANGULAR66	(8-79)

candModeList[ 5 ] = INTRA_ANGULAR26	(8-80)

  - Otherwise, if IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT, the following applies:

candModeList[ 0 ] = INTRA_PLANAR	(8-81)

candModeList[ 1 ] = INTRA_ANGULAR18	(8-82)

candModeList[ 2 ] = INTRA_ANGULAR25	(8-83)

candModeList[ 3 ] = INTRA_ANGULAR10	(8-84)

candModeList[ 4 ] = INTRA_ANGULAR65	(8-85)

candModeList[ 5 ] = INTRA_ANGULAR50	(8-86)

  - Otherwise, if IntraSubPartitionsSplitType is equal to ISP_VER_SPLIT, the following applies:

candModeList[ 0 ] = INTRA_PLANAR	(8-87)

candModeList[ 1 ] = INTRA_ANGULAR50	(8-88)

candModeList[ 2 ] = INTRA_ANGULAR43	(8-89)

candModeList[ 3 ] = INTRA_ANGULAR60	(8-90)

candModeList[ 4 ] = INTRA_ANGULAR3	(8-91)

candModeList[ 5 ] = INTRA_ANGULAR18	(8-92)

- The candModeList[ x ] with x = 0..5 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA and candIntraPredModeA is greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-10)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-11)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-12)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-13)

candModeList[ 4 ] = 2 + ( candIntraPredModeA % 64 )	(8-14)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 59 ) % 64 )	(8-15)

- Otherwise if candIntraPredModeB is not equal to candIntraPredModeA and candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC, the following applies:

  - The variables minAB and maxAB are derived as follows:

minAB = Min( candIntraPredModeA, candIntraPredModeB )	(8-16)

maxAB = Max( candIntraPredModeA, candIntraPredModeB )	(8-17)

  - If candIntraPredModeA and candIntraPredModeB are both greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-18)

candModeList[ 1 ] = candIntraPredModeB	(8-19)

  - If maxAB − minAB is equal to 1, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-26)

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-27)

candModeList[ 4 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-28)

candModeList[ 5 ] = 2 + ( maxAB % 64 )	(8-29)

  - Otherwise if maxAB − minAB is equal to 2, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-30)

candModeList[ 3 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-31)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-32)

candModeList[ 5 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-33)

  - Otherwise if maxAB − minAB is greater than 61, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-34)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-35)

candModeList[ 4 ] = 2 + ( minAB % 64 )	(8-36)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-37)

  - Otherwise, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-38)

candModeList[ 3 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-39)

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-40)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-41)

  - Otherwise (candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC), candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = maxAB							    (8-48)

candModeList[ 1 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-49)

candModeList[ 2 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-50)

candModeList[ 3 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-51)

candModeList[ 4 ] = 2 + ( maxAB % 64 )	(8-52)

candModeList[ 5 ] = 2 + ( ( maxAB + 59 ) % 64 )	(8-53)

- Otherwise, the following applies:

candModeList[ 0 ] = INTRA_ANGULAR50	(8-60)

candModeList[ 1 ] = INTRA_ANGULAR18	(8-61)

candModeList[ 2 ] = INTRA_ANGULAR2	(8-62)

candModeList[ 3 ] = INTRA_ANGULAR34	(8-63)

candModeList[ 4 ] = INTRA_ANGULAR66	(8-64)

candModeList[ 5 ] = INTRA_ANGULAR26	(8-65)

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, the IntraPredModeY[ xCb ][ yCb ] is set equal to  candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived by applying the following ordered steps:

- When candModeList[ i ] is greater than candModeList[ j ] for i = 0..4 and for each i, j = ( i + 1 )..5, both values are swapped as follows:

( candModeList[ i ], candModeList[ j ] ) = Swap( candModeList[ i ], candModeList[ j ] )	(8-66)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to ( intra_luma_mpm_remainder[ xCb ][ yCb ] + 2 ).

  - For i equal to 0 to 5, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

The variable IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].

      - General

Input to this process is a request for a syntax element.

Output of this process is the binarization of the syntax element.

Table 9-4 specifies the type of binarization process associated with each syntax element and corresponding inputs.

The specification of the truncated Rice (TR) binarization process, the truncated binary (TB) binarization process, the k-th order Exp-Golomb (EGk) binarization process and the fixed-length (FL) binarization process are given in clauses 9.5.3.3 through 9.5.3.6, respectively.

| Table 9-4 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| tile_group_data( ) | end_of_tile_group_flag | FL | cMax = 1 |
| coding_tree_unit( ) | alf_ctb_flag[ ][ ][ ] | FL | cMax = 1 |
| sao( ) | sao_merge_left_flag | FL | cMax = 1 |
|  | sao_merge_up_flag | FL | cMax = 1 |
|  | sao_type_idx_luma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_type_idx_chroma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_offset_abs[ ][ ][ ][ ] | TR | cMax = ( 1 << ( Min( bitDepth, 10 ) − 5 ) ) − 1, cRiceParam = 0 |
|  | sao_offset_sign[ ][ ][ ][ ] | FL | cMax = 1 |
|  | sao_band_position[ ][ ][ ] | FL | cMax = 31 |
|  | sao_eo_class_luma | FL | cMax = 3 |
|  | sao_eo_class_chroma | FL | cMax = 3 |
| coding_quadtree( ) | qt_split_cu_flag[ ][ ] | FL | cMax = 1 |
| multi_type_tree( ) | mtt_split_cu_flag | FL | cMax = 1 |
|  | mtt_split_cu_vertical_flag | FL | cMax = 1 |
|  | mtt_split_cu_binary_flag | FL | cMax = 1 |
| coding_unit( ) | cu_skip_flag[ ][ ] | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_non_ang_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_planar_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_ref_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | intra_subpartitions_mode_flag | FL | cMax = 1 |
|  | intra_subpartition_split_flag | FL | cMax = 1 |
|  | intra_luma_mpm_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_mpm_idx[ ][ ] | TR | cMax = 5, cRiceParam = 0 |
|  | intra_luma_mpm_remainder[ ][ ] | TB | cMax = 60 58 |
|  | intra_chroma_pred_mode[ ][ ] | 9.5.3.7 | - |
|  | merge_flag[ ][ ] | FL | cMax = 1 |
|  | inter_pred_idc[ x0 ][ y0 ] | 9.5.3.8 | cbWidth, cbHeight |
|  | inter_affine_flag[ ][ ] | FL | cMax = 1 |
|  | cu_affine_type_flag[ ][ ] | FL | cMax = 1 |
|  | ref_idx_l0[ ][ ] | TR | cMax = num_ref_idx_l0_active_minus1, cRiceParam = 0 |
|  | mvp_l0_flag[ ][ ] | FL | cMax = 1 |
|  | ref_idx_l1[ ][ ] | TR | cMax = num_ref_idx_l1_active_minus1, cRiceParam = 0 |
|  | mvp_l1_flag[ ][ ] | FL | cMax = 1 |
|  | avmr_flag[ ][ ] | FL | cMax = 1 |
|  | amvr_precision_flag[ ][ ] | FL | cMax = 1 |
|  | gbi_idx[ ][ ] | TR | cMax = NoBackwardPredFlag ? 4: 2 |
|  | cu_cbf | FL | cMax = 1 |
|  | cu_sbt_flag | FL | cMax = 1 |
|  | cu_sbt_quad_flag | FL | cMax = 1 |
|  | cu_sbt_horizontal_flag | FL | cMax = 1 |
|  | cu_sbt_pos_flag | FL | cMax = 1 |
| merge_data( ) | mmvd_flag[ ][ ] | FL | cMax = 1 |
|  | mmvd_merge_flag[ ][ ] | FL | cMax = 1 |
|  | mmvd_distance_idx[ ][ ] | TR | cMax = 7, cRiceParam = 0 |
|  | mmvd_direction_idx[ ][ ] | FL | cMax = 3 |
|  | ciip_flag[ ][ ] | FL | cMax = 1 |
|  | ciip_luma_mpm_flag[ ][ ] | FL | cMax = 1 |
|  | ciip_luma_mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | merge_subblock_flag[ ][ ] | FL | cMax = 1 |
|  | merge_subblock_idx[ ][ ] | TR | cMax = MaxNumSubblockMergeCand − 1, cRiceParam = 0 |
|  | merge_triangle_flag[ ][ ] | FL | cMax = 1 |
|  | merge_triangle_idx[ ][ ] | EG1 | - |
|  | merge_idx[ ][ ] | TR | cMax = MaxNumMergeCand − 1, cRiceParam = 0 |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus2[ ] | EG1 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |
| transform_unit( ) | tu_cbf_luma[ ][ ][ ] | FL | cMax = 1 |
|  | tu_cbf_cb[ ][ ][ ] | FL | cMax = 1 |
|  | tu_cbf_cr[ ][ ][ ] | FL | cMax = 1 |
|  | cu_qp_delta_abs | 9.5.3.9 | - |
|  | cu_qp_delta_sign_flag | FL | cMax = 1 |
|  | transform_skip_flag[ ][ ] | FL | cMax = 1 |
|  | tu_mts_idx[ ][ ] | TR | cMax = 4, cRiceParam = 0 |
| residual_coding( ) | last_sig_coeff_x_prefix | TR | cMax = ( log2TbWidth << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_y_prefix | TR | cMax = ( log2TbHeight << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_x_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_x_prefix >> 1 ) − 1 ) − 1 ) |
|  | last_sig_coeff_y_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_y_prefix >> 1 ) − 1 ) − 1 ) |
|  | coded_sub_block_flag[ ][ ] | FL | cMax = 1 |
|  | sig_coeff_flag[ ][ ] | FL | cMax = 1 |
|  | par_level_flag[ ] | FL | cMax = 1 |
|  | abs_level_gt1_flag[ ] | FL | cMax = 1 |
|  | abs_level_gt3_flag[ ] | FL | cMax = 1 |
|  | abs_remainder[ ] | 9.5.3.10 | cIdx, current sub-block index i, x0, y0 |
|  | dec_abs_level[ ] | 9.5.3.11 | cIdx, x0, y0, xC, yC, log2TbWidth, log2TbHeight |
|  | coeff_sign_flag[ ] | FL | cMax = 1 |

      - Derivation process for ctxTable, ctxIdx and bypassFlag

        - General

Input to this process is the position of the current bin within the bin string, binIdx.

Outputs of this process are ctxTable, ctxIdx and bypassFlag.

The values of ctxTable, ctxIdx and bypassFlag are derived as follows based on the entries for binIdx of the corresponding syntax element in Table 9-10:

- If the entry in Table 9-10 is not equal to "bypass", "terminate" or "na", the values of binIdx are decoded by invoking the DecodeDecision process as specified in clause 9.5.4.3.2 and the following applies:

- ctxTable is specified in [Ed. (BB):.tbd when init clause is incorporated]

- The variable ctxInc is specified by the corresponding entry in Table 9-10 and when more than one value is listed in Table 9-10 for a binIdx, the assignment process for ctxInc for that binIdx is further specified in the clauses given in parenthesis.

- The variable ctxIdxOffset is specified by the lowest value of ctxIdx in [Ed. (BB):.tbd when init clause is incorporated] depending on the current value of initType.

- ctxIdx is set equal to the sum of ctxInc and ctxIdxOffset.

- bypassFlag is set equal to 0.

- Otherwise, if the entry in Table 9-10 is equal to "bypass", the values of binIdx are decoded by invoking the DecodeBypass process as specified in clause 9.5.4.3.4 and the following applies:

- ctxTable is set equal to 0.

- ctxIdx is set equal to 0.

- bypassFlag is set equal to 1.a

- Otherwise, if the entry in Table 9-10 is equal to "terminate", the values of binIdx are decoded by invoking the DecodeTerminate process as specified in clause 9.5.4.3.5 and the following applies:

- ctxTable is set equal to 0.

- ctxIdx is set equal to 0.

- bypassFlag is set equal to 0.

- Otherwise (the entry in Table 9-10 is equal to "na"), the values of binIdx do not occur for the corresponding syntax element.

| Table 9-10 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |  |
|  | 0 |  | 1 | 2 | 3 | 4 | >= 5 |
| end_of_tile_group_flag | terminate |  | na | na | na | na | na |
| alf_ctb_flag[ ][ ][ ] | 0..8 (clause 9.5.4.2.2) |  | na | na | na | na | na |
| sao_merge_left_flag | 0 |  | na | na | na | na | na |
| sao_merge_up_flag | 0 |  | na | na | na | na | na |
| sao_type_idx_luma | 0 |  | bypass | na | na | na | na |
| sao_type_idx_chroma | 0 |  | bypass | na | na | na | na |
| sao_offset_abs[ ][ ][ ][ ] | bypass |  | bypass | bypass | bypass | bypass | na |
| sao_offset_sign[ ][ ][ ][ ] | bypass |  | na | na | na | na | na |
| sao_band_position[ ][ ][ ] | bypass |  | bypass | bypass | bypass | bypass | bypass |
| sao_eo_class_luma | bypass |  | bypass | na | na | na | na |
| sao_eo_class_chroma | bypass |  | bypass | na | na | na | na |
| qt_split_cu_flag[ ][ ] | 0..5 (clause 9.5.4.2.2) |  | na | na | na | na | na |
| mtt_split_cu_flag | 0..12 (clause 9.5.4.2.2) |  | na | na | na | na | na |
| mtt_split_cu_vertical_flag | ( cbWidth = = cbHeight ) ? 0 : ( (cbWidth > cbHeight ) ? 1 : 2 ) |  | na | na | na | na | na |
| mtt_split_cu_binary_flag | 0 |  | na | na | na | na | na |
| cu_skip_flag[ ][ ] | 0,1,2 (clause 9.5.4.2.2) |  | na | na | na | na | na |
| pred_mode_flag | 0 |  | na | na | na | na | na |
| pcm_flag[ ][ ] | terminate |  | na | na | na | na | na |
| intra_luma_non_ang_flag[ ][ ] | When mh_intra_flag[ ][ ] is equal to 0, ctxInc is between 0..4 and derived per clause a.b.c.d.e. When mh_intra_flag[ ][ ] is equal to 1, ctxInc is ( cbWidth > 2*cbHeight ¦¦ cbHeight > 2*cbWidth) ? 5 : 6 |  | na | na | na | na | na |
| intra_luma_planar_flag[ ][ ] | 0 |  | na | na | na | na | na |
| intra_luma_ref_idx[ ][ ] | 0 |  | 1 | 2 | na | na | na |
| intra_subpartitions_mode_flag | 0 |  | na | na | na | na | na |
| intra_subpartition_split_flag | 0 |  | na | na | na | na | na |
| intra_luma_mpm_flag[ ][ ] | 0 |  | na | na | na | na | na |
| intra_luma_mpm_idx[ ][ ] | bypass |  | bypass | bypass | bypass | bypass | na |
| intra_luma_mpm_remainder[ ][ ] | bypass |  | bypass | bypass | bypass | bypass | bypass |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 | 0 |  | bypass | bypass | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 0 | 0 |  | 1 | 2 | bypass | bypass | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 1 | 0 |  | 1 | 2 | 2 | na | na |
| merge_subblock_flag[ ][ ] | 0,1,2 (clause 9.5.4.2.2) |  | na | na | na | na | na |
| merge_subblock_idx[ ][ ] sps_sbtmvp_enabled_flag = = 0 | 0 |  | bypass | bypass | bypass | bypass | bypass |
| merge_subblock_idx[ ][ ] sps_sbtmvp_enabled_flag = = 1 | 0 |  | 1 | 2 | 3 | 4 | 4 |
| merge_flag[ ][ ] | 0 |  | na | na | na | na | na |
| mmvd_flag[ ][ ] | 0 |  | na | na | na | na | na |
| mmvd_merge_flag[ ][ ] | 0 |  | na | na | na | na | na |
| mmvd_distance_idx[ ][ ] | 0 |  | bypass | bypass | bypass | bypass | bypass |
| mmvd_direction_idx[ ][ ] | bypass |  | bypass | na | na | na | na |
| merge_triangle_flag[ ][ ] | 0,1,2 (clause 9.5.4.2.2) |  | na | na | na | na | na |
| merge_triangle_idx[ ][ ] | 0 |  | bypass | bypass | bypass | bypass | bypass |
| merge_idx[ ][ ] | 0 |  | bypass | bypass | bypass | bypass | na |
| ciip_flag[ ][ ] | 0 |  | na | na | na | na | na |
| ciip_luma_mpm_flag[ ][ ] | 0 |  | na | na | na | na | na |
| ciip_luma_mpm_idx[ ][ ] | bypass |  | bypass | na | na | na | na |
| inter_pred_idc[ x0 ][ y0 ] | ( cbWidth + cbHeight ) != 8 ? 7 − ( ( 1 + Log2( cbWidth ) + Log2( cbHeight ) ) >> 1 ) : 4 |  | 4 | na | na | na | na |
| inter_affine_flag[ ][ ] | 0,1,2 (clause 9.5.4.2.2) |  | na | na | na | na | na |
| cu_affine_type_flag[ ][ ] | 0 |  | na | na | na | na | na |
| ref_idx_l0[ ][ ] | 0 |  | 1 | bypass | bypass | bypass | bypass |
| ref_idx_l1[ ][ ] | 0 |  | 1 | bypass | bypass | bypass | bypass |
| mvp_l0_flag[ ][ ] | 0 |  | na | na | na | na | na |
| mvp_l1_flag[ ][ ] | 0 |  | na | na | na | na | na |
| amvr_flag[ ][ ] | inter_affine_flag[ ][ ] ? 3 : ( 0,1,2 (clause 9.5.4.2.2) ) |  | na | na | na | na | na |
| amvr_precision_flag[ ][ ] | 0 |  | na | na | na | na | na |
| gbi_idx[ ][ ] NoBackwardPredFlag = = 0 | 0 |  | 1 | na | na | na | na |
| gbi_idx[ ][ ] NoBackwardPredFlag = = 1 | 0 |  | 1 | 2 | 3 | na | na |
| cu_cbf | 0 |  | na | na | na | na | na |
| cu_sbt_flag | ( cbWidth * cbHeight < 256 ) ? 1 : 0 |  | na | na | na | na | na |
| cu_sbt_quad_flag | 0 |  | na | na | na | na | na |
| cu_sbt_horizontal_flag | ( cbWidth = = cbHeight ) ? 0 : ( cbWidth < cbHeight ) ? 1 : 2 |  | na | na | na | na | na |
| cu_sbt_pos_flag | 0 |  | na | na | na | na | na |
| abs_mvd_greater0_flag[ ] | 0 |  | na | na | na | na | na |
| abs_mvd_greater1_flag[ ] | 0 |  | na | na | na | na | na |
| abs_mvd_minus2[ ] | bypass |  | bypass | bypass | bypass | bypass | bypass |
| mvd_sign_flag[ ] | bypass |  | na | na | na | na | na |
| tu_cbf_luma[ ][ ][ ] | 0,1,2,3 (clause 9.5.4.2.4) |  | na | na | na | na | na |
| tu_cbf_cb[ ][ ][ ] | trDepth = = 0 ? 0 : 1 |  | na | na | na | na | na |
| tu_cbf_cr[ ][ ][ ] | tu_cbf_cb[ ][ ][ ] |  | na | na | na | na | na |
| cu_qp_delta_abs | 0 |  | 1 | 1 | 1 | 1 | bypass |
| cu_qp_delta_sign_flag | bypass |  | na | na | na | na | na |
| transform_skip_flag[ ][ ] | 0 |  | na | na | na | na | na |
| tu_mts_idx[ ][ ] | cqtDepth |  | 6 | 7 | 8 | na | na |
| last_sig_coeff_x_prefix | 0..23 (clause 9.5.4.2.3) |  |  |  |  |  |  |
| last_sig_coeff_y_prefix | 0..23 (clause 9.5.4.2.3) |  |  |  |  |  |  |
| last_sig_coeff_x_suffix | bypass |  | bypass | bypass | bypass | bypass | bypass |
| last_sig_coeff_y_suffix | bypass |  | bypass | bypass | bypass | bypass | bypass |
| coded_sub_block_flag[ ][ ] | 0..3 (clause 9.5.4.2.5) |  | na | na | na | na | na |
| sig_coeff_flag[ ][ ] | 0..89 (clause 9.5.4.2.7) |  | na | na | na | na | na |
| par_level_flag[ ] | 0..32 (clause 9.5.4.2.8) |  | na | na | na | na | na |
| abs_level_gt1_flag[ ] | 0..32 (clause 9.5.4.2.8) |  | na | na | na | na | na |
| abs_level_gt3_flag[ ] | 0..32 (clause 9.5.4.2.8) |  | na | na | na | na | na |
| abs_remainder[ ] | bypass |  | bypass | bypass | bypass | bypass | bypass |
| dec_abs_level[ ] | bypass |  | bypass | bypass | bypass | bypass | bypass |
| coeff_sign_flag[ ] |  | bypass | na | na | na | na | na |

##### a.b.c.d.e Derivation process of ctxInc using left, left-bottom, above and above-right syntax elements

Input to this process is the luma location ( x0, y0 ) specifying the top-left luma sample of the current luma block relative to the top-left sample of the current picture, the colour component cIdx, the current coding quadtree depth cqDepth, and the width and the height of the current coding block in luma samples cbWidth and cbHeight.

Output of this process is ctxInc.

The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( x0 − 1, y0 ) as inputs, and the output is assigned to availableL.

The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( x0 − 1, y0 + cbHeight − 1 ) as inputs, and the output is assigned to availableLB.

The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( x0, y0 − 1 ) as inputs, and the output is assigned to availableA.

The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( x0 + cbWidth − 1, y0 − 1 ) as inputs, and the output is assigned to availableAR.

The same CTU checking process for a block as specified in clause 6.Y.X [Same CTU checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( x0, y0 − 1 ) as inputs, and the output is assigned to sameCtuA.

The same CTU checking process for a block as specified in clause 6.Y.X [Same CTU checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( x0 + cbWidth − 1, y0 − 1 ) as inputs, and the output is assigned to sameCtuAR.

The assignment of ctxInc is specified as follows with condL, condLB, condA and condAR for the syntax elements intra_luma_angular_mode_flag[ x0 ][ y0 ] specified in Table 9-xx:

ctxInc = ( condL  &&  availableL ) + ( condLB  &&  availableLB ) + ( condA  &&  availableA && sameCtuA ) + ( condAR  &&  availableAR && sameCtuAR )	(9-yy)

Table 9-xx – Specification of ctxInc using left, left-bottom, above and above-right syntax elements

| Syntax element | condL | condLB | condA | condAR |
| --- | --- | --- | --- | --- |
| intra_luma_angular_mode_flag[ x0 ][ y0 ] | CuPredMode[ xNbL ][ yNbL ] is equal to MODE_INTRA && intra_luma_angular_mode_flag[ xNbL ][ yNbL ] is equal to 1 | CuPredMode[ xNbLB ][ yNbLB ] is equal to MODE_INTRA && intra_luma_angular_mode_flag[ xNbLB ][ yNbLB ] is equal to 1 | CuPredMode[ xNbA ][ yNbA ] is equal to MODE_INTRA && intra_luma_angular_mode_flag[ xNbA ][ yNbA ] is equal to 1 | CuPredMode[ xNbAR ][ yNbAR ] is equal to MODE_INTRA && intra_luma_angular_mode_flag[ xNbAR ][ yNbAR ] is equal to 1 |