Table 9-15 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| …… | …… | ...... | …… | …… | …… | …… |
| st_idx[ ][ ][ ] | 0,1,4,5 (clause 9.5.4.2.8) | 2,3,6,7 (clause 9.5.4.2.8) na | na | na | na | na |
| …… | …… | …… | …… | …… | …… | …… |

##### 9.5.4.2.8   Derivation process of ctxInc for the syntax element st_idx

Inputs to this process are the colour component index cIdx, the luma or chroma location ( x0, y0 ) specifying the top-left sample of the current luma or chroma coding block relative to the top-left sample of the current picture depending on cIdx, the tree type treeType, the luma intra prediction mode IntraPredModeY[ x0 ][ y0 ] as specified in clause 8.4.2, the syntax element intra_chroma_pred_mode[ x0 ][ y0 ] specifying the intra prediction mode for chroma samples as specified in clause 7.4.7.5, and the multiple transform selection index tu_mts_idx[ x0 ][ y0 ].

Output of this process is the variable ctxInc.

The variable intraModeCtx is derived as follows:

- If cIdx is equal to 0, intraModeCtx is derived as follows:

intraModeCtx = ( IntraPredModeY[ x0 ][ y0 ] <= 1 ) ? 1 : 0

- Otherwise (cIdx is greater than 0), intraModeCtx is derived as follows:

intraModeCtx = ( intra_chroma_pred_mode[ x0 ][ y0 ] >= 4 ) ? 1 : 0

The variable mtsCtx is derived as follows:

mtsCtx = ( tu_mts_idx[ x0 ][ y0 ] > 0 && treeType != SINGLE_TREE ) ? 1 : 0

The variable ctxInc is derived as follows:

ctxInc = ( binIdx << 1 ) + intraModeCtx + ( mtsCtx << 2 )