| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0106 |
| --- | --- |

| Title: | CE7-related: TU level limits on context-coded bins for coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes to limit the total number of context-coded bins for coefficient coding at TU-level. The limit is set to number of non-zero coded coefficient groups in a TU times the existing regular coded bin count limit at coefficient group level (2 bins/coefficient). The simulations results show -0.05% AI, -0.03% RA, -0.04% LB BD-Rate versus the VTM-4.0 anchor under common test conditions.

# Proposal

The current coefficient coding scheme in VVC [2] imposes limits on number of regular coded bins at coefficient group (CG) level with 2 bins/coefficient, i.e. 32 for 16-coefficient CGs and 8 for 4-coefficient CGs. Instead of imposing a limit fixed limit for each CG, a TU level limit for total number of context-coded bins is used. The limit is set to number of non-zero coded coefficient groups in a TU times the existing regular coded bin count limit at coefficient group level (32 or 8 bins depending on CG size). The last coefficient position is used to determine the number of CGs scanned starting from the last position. If a CG is not coded or skipped (transform zero-out) in the scan, then the number of regular bins allocated for a CG is discounted from the remaining number of regular coded bins. This can be described by:

lastPosSubblock = (scanPosLast >> log2CGSize) + 1

regBinTULimit = lastPosSubblock * (log2CGSize == 2 ? 8 : 32),

where scanPosLast is the last position in scan order, log2CGSize is 2 for CGs of size 4, and 4 for CGs of size 16, regBinTULimit is the initial total number of regular coded bins at TU level. If a CG is not coded or skipped in in the scan path, then

regBinTULimit -= (log2CGSize == 2 ? 8 : 32).

# Results

The proposed scheme is implemented on top of VTM-4.0 software [3].  The simulations were carried out under the JVET common test conditions [1][4].

The test results under JVET common test conditions is tabulated in Table 1.

|  |  |  |  |  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Over VTM-4.0 |  |  |  |  |
|  | Y |  | U |  | V |  | EncT | DecT |  |
| Class A1 | -0.05% |  | -0.10% |  | -0.12% |  | 102% | 104% |  |
| Class A2 | -0.08% |  | -0.01% |  | -0.08% |  | 102% | 103% |  |
| Class B | -0.04% |  | -0.06% |  | -0.04% |  | 101% | 101% |  |
| Class C | -0.03% |  | -0.05% |  | -0.12% |  | 101% | 99% |  |
| Class E | -0.04% |  | -0.10% |  | -0.12% |  | 101% | 100% |  |
| Overall | -0.05% |  | -0.06% |  | -0.09% |  | 101% | 101% |  |
| Class D | 0.00% |  | -0.11% |  | -0.36% |  | 102% | 100% |  |
| Class F | -0.12% |  | -0.18% |  | -0.25% |  | 101% | 98% |  |
|  |  |  |  |  | Random Access |  |  |  |  |
|  |  |  |  |  | Over VTM-4.0 |  |  |  |  |
|  | Y |  | U |  | V |  | EncT | DecT |  |
| Class A1 | -0.04% |  | -0.06% |  | -0.05% |  | 104% | 103% |  |
| Class A2 | -0.03% |  | -0.10% |  | -0.04% |  | 102% | 103% |  |
| Class B | -0.03% |  | -0.13% |  | -0.11% |  | 101% | 101% |  |
| Class C | -0.02% |  | -0.10% |  | -0.22% |  | 101% | 100% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.03% |  | -0.10% |  | -0.11% |  | 102% | 101% |  |
| Class D | -0.03% |  | -0.52% |  | -0.02% |  | 101% | 100% |  |
| Class F | -0.12% |  | -0.30% |  | -0.48% |  | 101% | 102% |  |
|  |  |  |  |  | Low Delay B |  |  |  |  |
|  |  |  |  |  | Over VTM-4.0 |  |  |  |  |
|  | Y |  | U |  | V |  | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.02% |  | -0.16% |  | 0.02% |  | 101% | 101% |  |
| Class C | -0.05% |  | 0.23% |  | -0.13% |  | 101% | 101% |  |
| Class E | -0.05% |  | 0.39% |  | 0.38% |  | 101% | 100% |  |
| Overall | -0.04% |  | 0.11% |  | 0.06% |  | 101% | 101% |  |
| Class D | -0.01% |  | 0.44% |  | 0.01% |  | 101% | 100% |  |
| Class F | -0.26% |  | -0.57% |  | -0.28% |  | 102% | 101% |  |

Table 1: Average Bjøntegaard delta rate savings of the proposed method relative to VTM-4.0.

Additional test results with reducing the per CG limit value to 28 for 16 sample CGs and 7 for 4 sample CGs and 28 for Luma and 20 Chroma CGs (JVET-N0091) for reducing the worst case regular coded bin count for proposed scheme is attached. By modifying the per CG limit values for luma and chroma blocks to lower values, e.g. 28 bins or lower per 16 sample blocks, the worst case context-coded bin count for TUs may be further reduced.

# Conclusions

This document described a scheme for imposing a limit on total number of context-coded bins at TU-level for coefficient coding. The limit is set to number of non-zero coded coefficient groups in a TU times the existing regular coded bin count limit at coefficient group level (2 bins/coefficient). The simulations results show 0.05% AI, 0.03% RA, 0.04% LB BD-Rate gain versus the VTM-4.0 anchor under common test conditions.

# References

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” JVET-M1027, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, January 2019.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, January 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git

- “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, January 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).