| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N0108-v1 |
| --- | --- |

| Title: | AHG12: Allowing mixed IRAP and non-IRAP NAL unit types within a picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | +1-908-803-3888 yekui.wang@huawei.com +1-858-375-8065 fnu.hendry@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a design for VVC that allows for mixing of IRAP and non-IRAP NAL unit types within a picture. It is asserted that allowing mixed IRAP and non-IRAP NAL unit types within a picture is beneficial for both MCTS-based and sub-picture-based video coding. The proposed design is summarized as follows:

- For any particular picture, either all VCL NAL units have the same NAL unit type, or some VCL NAL units have a particular IRAP NAL unit type and the rest have a particular non-IRAP VCL NAL unit type. In other words, VCL NAL units of any particular picture cannot have more than one IRAP NAL unit type and cannot have more than one non-IRAP NAL unit type.

- Each picture is associated with an indication of whether the picture contains mixed nal_unit_type values. This indication is signalled in the PPS.

- A picture is considered as an IRAP picture only if it does not contain mixed nal_unit_type values and the VCL NAL units have an IRAP NAL unit type.

- For any IRAP (including IDR) NAL unit that does not belong to an IRAP picture, POC MSB is not reset.

- Reference picture lists information is signalled and reference picture lists are derived for any type of VCL NAL units, including IDR NAL units.

- For any IRAP (including IDR) NAL unit that does not belong to an IRAP picture, DPB is not reset (i.e., marking of all reference pictures as "unused for reference" is not performed).

- TemporalId shall be equal to 0 for a picture if at least one VCL NAL unit of the picture is an IRAP NAL unit.

The proposed detailed spec text changes are provided in an attachment.

# Introduction

## Motivations for allowing mixed nal_unit_type values within a picture

JVET-M0388 Option 3 suggested to allow sub-pictures of the same coded picture to have different nal_unit_type values, where sub-pictures are as described in JVET-M0261 Option 3. JVET-M0261 Option 3 is summarized as follows:

- JVET-M0261 Option 3: Sub-picture-based partitioning and decoding. Each sub-picture may refer to its own PPS and may hence have its own tile partitioning. Sub-pictures are treated like pictures in the decoding process.

As stated in JVET-M0388, allowing different nal_unit_type values within a coded picture enables to merge sub-pictures originating from a random-access picture and sub-pictures originating from a non-random-access picture into the same coded picture without substantial difficulty (e.g., without VLC-level modifications). Actually, this benefit also holds for MCTS-based coding.

We add one more scenario wherein allowing different nal_unit_type values within a coded picture is beneficial, described as follows:

- Some areas of a 360o video content can be watched a lot more by users than other areas. In MCTS/sub-picture-based viewport-dependent 360o video delivery, to better trade-off between coding efficiency and the average comparable quality viewport switching latency, more frequent IRAP pictures can be coded for more-often-watched areas than the other areas. The comparable quality viewport switching latency is the latency experienced by the user when switching from a first viewport to a second viewport until the presentation quality of the second viewport reaches a comparable presentation quality as the first viewport.

We therefore join the authors of JVET-M0388 to propose support of mixed nal_unit_type values within a coded picture in VVC, for either sub-picture-based coding or MCTS-based coding. In particular, both scenarios described above would be satisfied if mixing of nal_unit_type values within a picture is supported in a limited fashion by just allowing mixed IRAP and non-IRAP NAL unit types.

## Aspects that should be specified for such support

JVET-M0388 Option 3 relies on the sub-picture-based coding in JVET-M0261 Option 3, particularly that aspect of "sub-pictures are treated like pictures in the decoding process" as the basis for support of mixed nal_unit_type values within a picture.

However, the following aspects need to be additionally considered:

- Firstly, as mentioned above, support of mixed nal_unit_type values within a picture should be specified in a limited fashion by just allowing mixed IRAP and non-IRAP NAL unit types. To be more accurate, for any particular picture, either all VCL NAL units have the same NAL unit type, or some VCL NAL units have a particular IRAP NAL unit type and the rest have a particular non-IRAP VCL NAL unit type. In other words, VCL NAL units of any particular picture cannot have more than one IRAP NAL unit type and cannot have more than one non-IRAP NAL unit type.

- To make it work when a picture is allowed to have mixed IRAP and non-IRAP VCL NAL unit types, the POC derivation process needs to work such that the POC value derived based on any of the VCL NAL units of the picture is the same.

- Similarly, to make it work when a picture is allowed to have mixed IRAP and non-IRAP VCL NAL unit types, the reference picture management (RPM) process needs to work such that the same set of STRPs and the same set of LTRPs are kept in the DPB after reference picture marking based on any of the VCL NAL units of the picture.

- In addition, to make it work when a picture is allowed to have mixed IRAP and non-IRAP VCL NAL unit types, the mechanism of POC derivation process and the RPM process needs to be designed such that, when an MCTS/sub-picture sequence is extractable out of the original bitstream to form a new bitstream, the same mechanism needs to work the same way before and after the sub-bitstream extraction, and needs to produce the same decoding result for any particular sample before and after the sub-bitstream extraction.

The last three aspects above are unfortunately at odds with the current POC and RPM mechanism that is summarized as follows:

- In POC signalling and derivation, for IRAP pictures starting a CVS, the POC MSB needs to be (re)set to 0, whereas for other picture types the POC MSB is derived from the previous non-RASL, non-RADL picture with TemporalId equal to 0.

- In RPM, for IRAP pictures starting a CVS, no RPM information needs to be signalled, because for such a coded picture, the decoder simply marks all reference pictures in the DPB as "unused for reference", whereas on the other hand for other types of pictures, RPM information needs to be signalled and the corresponding RPM operations (RPL construction and reference picture marking) need to be performed.

There is one more important aspect that needs to be additionally considered. In the current VVC design, because mixing of nal_unit_type values within a picture is not allowed, although the derivation of whether a picture is an IRAP picture etc. and the derivation of the variable NoRaslOutputFlag etc. are described at picture level, the decoder can perform these derivations after receiving the first VCL NAL unit of any picture. However, when mixing of nal_unit_type values within a picture is allowed, care needs to be taken to make sure that any part of the decoding process description does not imply that the process of a VCL NAL unit of a picture needs to wait for the arrival of other VCL NAL units of the picture.

# Proposal

We proposal a design that allows for mixing of NAL unit type values within a picture, summarized as follows:

- Mixing of nal_unit_type values within a picture is allowed in a limited fashion by just allowing mixed IRAP and non-IRAP NAL unit types. For any particular picture, either all VCL NAL units have the same NAL unit type, or some VCL NAL units have a particular IRAP NAL unit type and the rest have a particular non-IRAP VCL NAL unit type. In other words, VCL NAL units of any particular picture cannot have more than one IRAP NAL unit type and cannot have more than one non-IRAP NAL unit type.

- Each picture is associated with an indication of whether the picture contains mixed nal_unit_type values. This indication is signalled in the PPS.

  - Note that the key role of this indication is to determine whether to reset POC MSB and to reset the DPB (i.e., mark all reference pictures as "unused for reference"). It is also possible to signal the indication in the VCL NAL unit, e.g., in the tile group header, but then the value would need to be changed in bitstream extraction or merging involving a change to the status of whether the IDR NAL units are mixed with non-IRAP NAL units in a picture. When the indication is signalled in the PPS, under the same circumstance, change of the value in the PPS would be needed, however that's acceptable as PPSs need to be rewritten and replaced anyway in such bitstream extraction or merging.

- A picture is considered as an IRAP picture only if it does not contain mixed nal_unit_type values and the VCL NAL units have an IRAP NAL unit type.

- For any IRAP (including IDR) NAL unit that does not belong to an IRAP picture, POC MSB is not reset.

- Reference picture lists information is signalled and reference picture lists are derived for any type of VCL NAL units, including IDR NAL units.

  - Note that if reference picture lists information is not signalled for VCL NAL units of IDR pictures, although the decoding process would work OK, removal/addition of reference picture lists information from/to the tile group headers would be needed in bitstream extraction or merging involving a change to the status of whether the IDR NAL units are mixed with non-IRAP NAL units in a picture.

- For any IRAP (including IDR) NAL unit that does not belong to an IRAP picture, DPB is not reset (i.e., marking of all reference pictures as "unused for reference" is not performed).

- TemporalId shall be equal to 0 for a picture if at least one VCL NAL unit of the picture is an IRAP NAL unit.

The proposed detailed spec text changes are provided in an attachment.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).