| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N0111-v1 |
| --- | --- |

| Title: | AHG12: Flexible tiling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry 10180 Telesis Court San Diego, CA, 92121, USA Maxim Sychev Building 1/7, Altufievskoe Shosse, Moscow 127106, Russia | Tel: Email: | +1-908-803-3888 yekui.wang@huawei.com +1-858-375-8065 fnu.hendry@huawei.com +7-925-008-1124 sychev.maxim@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution is a resubmission of JVET-M0129. It is asserted that, if the group adopts a sub-picture-based video coding design, e.g., JVET-N0107, that allows different sub-pictures to have different tile configurations, then there would be no need to additionally have a flexible tiling scheme that allows to partition a picture into tiles in a manner that is more flexible than the current tiling scheme, where tiles are always in tile rows and tile columns.

This contribution firstly analyzes scenarios that benefit from a flexible tiling scheme. In this contribution, flexible tiling refers to partitioning of picture into tiles in a manner that is more flexible than the current tiling scheme for VVC, which is the same as in HEVC, where tiles are always in tile rows and tile columns.

An observation from the analysis was that some tiles resulted from the current tiling scheme need to be further split into multiple tile rows. Therefore, it was asserted that, for flexible tiling support in VVC, it would be sufficient to just allow further splitting some tiles resulted from the current tiling into some second-level tile rows and tile columns.

The contribution then proposes a flexible tiling scheme that allows further splitting some tiles resulted from the current tiling scheme into some second-level tile rows and tile columns. As a result of the proposed scheme, tiles of a picture are not necessarily in tile rows and tile columns.

# Introduction

Per the current tiling scheme for VVC, which is the same as in HEVC, tiles are always in tile rows and tile columns. Tile column runs through from the top of a picture to the bottom of the picture. Likewise, tile row runs through from the left of the picture to the right of the picture.

There are scenarios that would benefit from a more flexible tiling scheme. Such us cases that we are aware of are described and analyzed as follows.

The first such scenario is the one that is documented in subclause D.6.2 of the 1st edition of the ISO/IEC 23090-2, Omnidirectional MediA Format (OMAF).

The scheme is illustrated with Figure 1, and explained in the following:

- Encoding: The content is encoded at two spatial resolutions, such as 5120×2560 and 2560×1280, and with 4×2 tile grid. An MCTS is coded for each tile position.

- Encapsulation of coded data: Each coded MCTS sequence is stored as a sub-picture track or a tile track.

- Viewport-adaptive MCTS selection: Each range of viewing orientations that causes a different selection of high- and low-resolution MCTSs is considered. Four MCTSs of the high-resolution bitstream are selected the same way as in the example above. Moreover, the four low-resolution MCTSs that are not overlapping with the selected high-resolution MCTSs are also selected.

- Creation of extractor tracks: An extractor track is created for each distinct viewport-adaptive MCTS selection. In each sample of the extractor track, one extractor is created for each MCTS, extracting data from the tile or sub-picture track that contains the selected high- or low-resolution MCTSs. Each tile encapsulating content from the low-resolution bitstream contains two slices. RegionWisePackingBox is included in the extractor track to create a mapping between the packed picture and a projected picture of the ERP format. In the presented example, the bitstreams resolved from the extractor tracks have resolution 3200×2560. Consequently, a 4K-capable decoder could decode content where the viewport is extracted from a coded bitstream with 5K (5120×2560) resolution.

Figure 1 – Example of extractor track combining MCTSs from bitstreams of different resolution

In this use case, for pictures to be delivered from the content provider to the receiver, as shown by the right-hand side of Figure 1, tiles at the last column would be better to have four rows, instead of only two rows. However, since the other columns have only two rows, per the current tiling scheme, the last columns can only have two rows of tiles. This limitation forces the content generator to use a trick in which it needs to merge two tiles into one tile with two slices.

Another solution that does not require using such trick as described above is to divide the other columns into four rows instead of two rows. However, this would result in compression penalty, which grows as the number of columns grows.

The second such scenario is video conferencing applications. For video conferencing applications, having a more flexible tile structure than what current tile structure would enable the system to have a bitstream consisting of all participants but in different sizes as illustrated in Figure 2.

Figure 2 – Example of a picture with areas showing video conferencing participants

Without a flexible tile structure, in this example, the area for the speaker would have to be divided into three tile rows, which consequently has compression penalty that grows as the number of participants grows.

We are not aware of any other such scenarios. If there are more, we'd be happy to learn.

An observation from the scenarios where more flexible tile structures are needed is as follows. Basically, based on the current tiling scheme, some tiles (e.g., the tile or tiles of the right-most tile column in Figure 1 and Figure 2) need to be further split into multiple tile rows. Depending on how to put these tiles around the other tiles, e.g., in Figure 2, participants 2 to 4 may be put below participant 1 in the picture, a tile may need to be further split into tile columns instead of tile rows. Therefore, to satisfy these scenarios, it would be sufficient to just allow further splitting tiles resulted from the current tiling scheme into some second-level tile rows and tile columns.

We therefore propose a flexible tiling scheme for VVC just to allow that.

# Proposed flexible tiling scheme

The proposed flexile tiling works as follows:

- Firstly, the current tiling scheme is applied to partition a picture into a set of tile columns and a set of tile rows. Each of the tiles resulted from this step is referred to as a first-level tile.

- For each first-level tile, a flag is signalled to specify whether or not the tile is further divided into one or more tile columns and one or more tile rows. The new tiles resulted from the division of first-level tiles are referred to as second-level tiles.

  - For simplicity, no further splitting of a second-level tile is allowed, by syntax design.

  - Also for simplicity, when a first-level tile is divided into second-level tiles, the division always uses uniform-sized second-level tile columns and second-level tile rows, by syntax design.

As a result, a picture would contain zero or more first-level tiles that were not further split and zero or more second-level tiles. A first-level tile that has been further split would only conceptually exist, would not physically exist, and would not be counted in the total number of tiles in the picture.

- For simplicity, it is constrained that a tile group shall contain only a number of complete first-level tiles or only a consecutive sequence of complete second-level tiles of one single first-level tile.

- The scan order is specified as follows (for simplicity, it is constrained that a tile group shall exclusively consist of either a number of complete first-level tiles or a true subset of a first-level tile) as shown in the example illustrated in Figure 3:

  - First-level tiles are ordered the same as in the current tiling scheme, i.e., tile raster scan of a picture. For a first-level tile that has been further split, when it is referred to, the set of second-level tiles resulted from its splitting is collectively referred to.

  - The second-level tiles of any first-level tile tileA precedes the second-level tiles of another first-level tile that succeeds tileA.

  - For all the second-level tiles of a first-level tile, their order is the second-level tile raster scan of the first-level tile.

  - As before, CTUs within any physically-existing tile are in the order of CTU raster scan of the tile.

Figure 3 – Example of scan order in flexible tile

The specification text, relative to the basis text in JVET-L0686-v2, is provided below, where the texts in the basis text that are not mentioned below apply as they are, and modified texts relative to the basis text are highlighted in yellow.

### CTB raster and tile scanning process

The list ColWidth[ i ] for i ranging from 0 to num_level1_tile_columns_minus1, inclusive, specifying the width of the i-th first-level tile column in units of CTBs, is derived as follows:

if( uniform_level1_tile_spacing_flag )
	for( i = 0; i  <=  num_level1_tile_columns_minus1; i++ )
		ColWidth[ i ] = ( ( i + 1 ) * PicWidthInCtbsY ) / ( num_level1_tile_columns_minus1 + 1 ) − 
						( i * PicWidthInCtbsY ) / ( num_level1_tile_columns_minus1 + 1 )
else {
	ColWidth[ num_level1_tile_columns_minus1 ] = PicWidthInCtbsY	(6-1)
	for( i = 0; i < num_level1_tile_columns_minus1; i++ ) {
		ColWidth[ i ] = tile_level1_column_width_minus1[ i ] + 1
		ColWidth[ num_tile_level1_columns_minus1 ]  −=  ColWidth[ i ]
	}
}

The list RowHeight[ j ] for j ranging from 0 to num_level1_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of CTBs, is derived as follows:

if( uniform_level1_tile_spacing_flag )
	for( j = 0; j  <=  num_level1_tile_rows_minus1; j++ )
		RowHeight[ j ] = ( ( j + 1 ) * PicHeightInCtbsY ) / ( num_level1_tile_rows_minus1 + 1 ) − 
						( j * PicHeightInCtbsY ) / ( num_level1_tile_rows_minus1 + 1 )
else {
	RowHeight[ num_level1_tile_rows_minus1 ] = PicHeightInCtbsY	(6-2)
	for( j = 0; j < num_level1_tile_rows_minus1; j++ ) {
		RowHeight[ j ] = tile_level1_row_height_minus1[ j ] + 1
		RowHeight[ num_level1_tile_rows_minus1 ]  −=  RowHeight[ j ]
	}
}

The list colBd[ i ] for i ranging from 0 to num_level1_tile_columns_minus1 + 1, inclusive, specifying the location of the i-th tile column boundary in units of CTBs, is derived as follows:

for( colBd[ 0 ] = 0, i = 0; i  <=  num_level1_tile_columns_minus1; i++ )
		colBd[ i + 1 ] = colBd[ i ] + ColWidth[ i ]		(6-3)

The list rowBd[ j ] for j ranging from 0 to num_level1_tile_rows_minus1 + 1, inclusive, specifying the location of the j-th tile row boundary in units of CTBs, is derived as follows:

for( rowBd[ 0 ] = 0, j = 0; j  <=  num_level1_tile_rows_minus1; j++ )
	rowBd[ j + 1 ] = rowBd[ j ] + RowHeight[ j ]		(6-4)

The variable NumTilesInPic, specifying the number of tiles in a picture referring to the PPS, and the lists TileColBd[ i ], TileRowBd[ i ], TileWidth[ i ], and TileHeight[ i ] for i ranging from 0 to NumTilesInPic − 1, inclusive, specifying the location of the i-th tile column boundary in units of CTBs, the location of the i-th tile row boundary in units of CTBs, the width of the i-th tile column in units of CTBs, and the height of the i-th tile column in units of CTBs, are derived as follows:

for ( tileIdx = 0, i = 0; i < NumLevel1Tiles; i++ ) {
	tileX = i % ( num_level1_tile_columns_minus1 + 1 )
	tileY = i / ( num_level1_tile_columns_minus1 + 1 )
	if ( !level2_tile_split_flag[ i ] ) {		 (6-5)
		TileColBd[ tileIdx ] = colBd[ tileX ]
		TileRowBd[ tileIdx ] = rowBd[ tileY ]
		TileWidth[ tileIdx ] = ColWidth[ tileX ]
		TileHeight[ tileIdx ] = RowHeight[ tileY ] 		(6-5)
		tileIdx++
	} else {
		for( k = 0; k  <=  num_level2_tile_columns_minus1[ i ]; k++ )
			colWidth2[ k ] = ( ( k + 1 ) * ColWidth[ tileX ] ) / ( num_level2_tile_columns_minus1[ i ] + 1 ) −
					( k * ColWidth[ tileX ] ) / ( num_level2_tile_columns_minus1[ i ] + 1 )
		for( k = 0; k  <=  num_level2_tile_rows_minus1[ i ]; k++ )
			rowHeight2[ k ] = ( ( k + 1 ) * RowHeight[ tileY ] ) / ( num_level2_tile_rows_minus1[ i ] + 1 ) −
				( k * RowHeight[ tileY ] ) / ( num_level2_tile_rows_minus1[ i ] + 1 )
		for( colBd2[ 0 ] = 0, k = 0; k  <=  num_level2_tile_columns_minus1[ i ]; k++ )
			colBd2[ k + 1 ] = colBd2[ k ] + colWidth2[ k ]
		for( rowBd2[ 0 ] = 0, k = 0; k  <=  num_level2_tile_rows_minus1[ i ]; k++ )
			rowBd2[ k + 1 ] = rowBd2[ k ] + rowHeight2[ k ]
		numSplitTiles = (num_level2_tile_columns_minus1[ i ] + 1) *
					(num_level2_tile_rows_minus1[ i ] + 1)
		for( k = 0; k < numSplitTiles; k++ ) {
			tileX2 = k % (num_level2_tile_columns_minus1[ i ] + 1)
			tileY2 = k / (num_level2_tile_columns_minus1[ i ] + 1)
			TileColBd[ tileIdx ] = colBd[ tileX ] + colBd2[ tileX2 ]
			TileRowBd[ tileIdx ] = rowBd[ tileY ] + rowBd2[ tileY2 ]
			TileWidth[ tileIdx ] = colWidth2[ tileX2 ]
			TileHeight[ tileIdx ] = rowHeight2[ tileY2 ]
			tileIdx++
		}
	}
}
NumTilesInPic = tileIdx

The list CtbAddrRsToTs[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in CTB raster scan of a picture to a CTB address in tile scan, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ ) {
	tbX = ctbAddrRs % PicWidthInCtbsY
	tbY = ctbAddrRs / PicWidthInCtbsY	tileFound = FALSE
	for( tileIdx = NumTilesInPic − 1, i = 0; i < NumTilesInPic − 1  &&  !tileFound; i++ ) {	(6-6)
		tileFound = tbX  <  (TileColBd[ i ] + TileWidth[ i ] )  &&  tbY  <  ( TileRowBd[ i ] + TileHeight[ i ] )
		if (tileFound)
			tileIdx = i
	}
	CtbAddrRsToTs[ ctbAddrRs ] = 0
	for( i = 0; i < tileIdx; i++ )
		CtbAddrRsToTs[ ctbAddrRs ]  +=  TileHeight[ i ] * TileWidth[ i ]
	CtbAddrRsToTs[ ctbAddrRs ]  +=
				( tbY − TileRowBd[ tileIdx ] ) * TileWidth[ tileIdx ] + tbX − TileColBd[ tileIdx ]
}

The list CtbAddrTsToRs[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a CTB address in CTB raster scan of a picture, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ )	(6-7)
	CtbAddrTsToRs[ CtbAddrRsToTs[ ctbAddrRs ] ] = ctbAddrRs

The list TileId[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile ID, is derived as follows:

for( i = 0, tileIdx = 0; i  <=  NumTilesInPic; i++, tileIdx++ )
	for( y = TileRowBd[ i ]; y < TileRowBd[ i + 1 ]; y++ ) 	(6-8)
		for( x = TileColBd[ i ]; x < TileColBd[ i + 1 ]; x++ )
			TileId[ CtbAddrRsToTs[ y * PicWidthInCtbsY+ x ] ] = tileIdx

The list NumCtusInTile[ tileIdx ] for tileIdx ranging from 0 to NumTilesInPic− 1, inclusive, specifying the conversion from a tile index to the number of CTUs in the tile, is derived as follows:

for( i = 0,tileIdx = 0; i < NumTilesInPic; i++, tileIdx++ )	(6-9)
	NumCtusInTile[ tileIdx ] = TileColWidth[ tileIdx ] * TileRowHeight[ tileIdx ]

### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_level1_tile_columns_minus1 | ue(v) |
| num_level1_tile_rows_minus1 | ue(v) |
| uniform_level1_tile_spacing_flag | u(1) |
| if( !uniform_level1_tile_spacing_flag ) { |  |
| for( i = 0; i < num_level1_tile_columns_minus1; i++ ) |  |
| level1_tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_level1_tile_rows_minus1; i++ ) |  |
| level1_tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| level2_tile_present_flag | u(1) |
| for( i = 0; level2_tile_present_flag && i < NumLevel1Tiles; i++ ) { |  |
| level2_tile_split_flag[ i ] | u(1) |
| if( level2_tile_split_flag ) { |  |
| num_level2_tile_columns_minus1[ i ] | ue(v) |
| num_level2_tile_rows_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### Picture parameter set RBSP semantics

...

num_level1_tile_columns_minus1 plus 1 specifies the number of level 1 tile columns partitioning the picture. num_level1_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When not present, the value of num_level1_tile_columns_minus1 is inferred to be equal to 0.

num_level1_tile_rows_minus1 plus 1 specifies the number of level 1 tile rows partitioning the picture. num_level1_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of num_level1_tile_rows_minus1 is inferred to be equal to 0.

The variable NumLevel1Tiles is set equal to ( num_level1_tile_columns_minus1 + 1 ) * ( num_level1_tile_rows_minus1 + 1 ).

When single_tile_in_pic_flag is equal to 0, NumTilesInPic shall be greater than 1.

uniform_level1_tile_spacing_flag equal to 1 specifies that level 1 tile column boundaries and likewise level 1 tile row boundaries are distributed uniformly across the picture. uniform_level1_tile_spacing_flag equal to 0 specifies that level 1 tile column boundaries and likewise level 1 tile row boundaries are not distributed uniformly across the picture but signalled explicitly using the syntax elements level1_tile_column_width_minus1[ i ] and level1_tile_row_height_minus1[ i ]. When not present, the value of uniform_level1_tile_spacing_flag is inferred to be equal to 1.

level1_tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th level 1 tile column in units of CTBs.

level1_tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile level 1 row in units of CTBs.

level2_tile_present_flag specifies that one or more level 1 tiles are divided into level 2 tiles.

level2_tile_split_flag[ i ] specifies that the i-th level 1 tile is divided into two or more tiles.

num_level2_tile_columns_minus1[ i ] plus 1 specifies the number of tile columns partitioning the i-th tile. num_level2_tile_columns_minus1[ i ] shall be in the range of 0 to ColWidth[ i ], inclusive. When not present, the value of num_level2_tile_columns_minus1[ i ] is inferred to be equal to 0.

num_level2_tile_rows_minus1[ i ] plus 1 specifies the number of tile rows partitioning the i-th tile. num_level2_tile_rows_minus1[ i ] shall be in the range of 0 to RowHeight[ i ], inclusive. When not present, the value of num_level2_tile_rows_minus1[ i ] is inferred to be equal to 0.

When level2_tile_split_flag[ i ] is equal to 1, the value of ( num_level2_tile_columns_minus1[ i ] + 1 ) * ( num_level2_tile_rows_minus1[ i ] + 1 ) shall be greater than 1.

A picture contains zero or more level 1 tiles that have level2_tile_split_flag[ i ] equal to 0 and zero or more level 2 tiles. A level 1 tile that has level2_tile_split_flag[ i ] equal to 0 is not counted in the total number of tiles in a picture, and when it is referred to in the text, the set of level 2 tiles resulted from its splitting is collectively referred to.

The following variables are derived by invoking the CTB raster and tile scanning conversion process as specified in clause 6.5.1:

–	The list ColWidth[ i ] for i ranging from 0 to num_level1_tile_columns_minus1, inclusive, specifying the width of the i-th level 1 tile column in units of CTBs,

–	the list RowHeight[ j ] for j ranging from 0 to num_level1_tile_rows_minus1, inclusive, specifying the height of the j-th level 1 tile row in units of CTBs,

–	the variable NumTilesInPic, specifying the number of tiles in a picture referring to the PPS,

–	The list TileWidth[ i ] for i ranging from 0 to NumTilesInPic, inclusive, specifying the width of the i-th tile in units of CTBs,

–	the list TileHeight[ i ] for i ranging from 0 to NumTilesInPic, inclusive, specifying the height of the i-th tile in units of CTBs,

–	the list TileColBd[ i ] for i ranging from 0 to NumTilesInPic, inclusive, specifying the location of the i-th tile column boundary in units of CTBs,

–	the list TileRowBd[ i ] for j ranging from 0 to NumTilesInPic, inclusive, specifying the location of the i-th tile row boundary in units of CTBs,

–	the list CtbAddrRsToTs[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in the CTB raster scan of a picture to a CTB address in the tile scan,

–	the list CtbAddrTsToRs[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in the tile scan to a CTB address in the CTB raster scan of a picture,

–	the list TileId[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile ID,

–	the list NumCtusInTile[ tileIdx ] for tileIdx ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a tile index to the number of CTUs in the tile,

–	the list FirstCtbAddrTs[ tileIdx ] for tileIdx ranging from 0 to NumTilesInPic − 1, inclusive, specifying the conversion from a tile ID to the CTB address in tile scan of the first CTB in the tile,

...

### Tile group header semantics

...

A tile group shall contain only a number of complete level 1 tiles or only a consecutive sequence of complete level 2 tiles of one single level 1 tile.

tile_group_address specifies the tile address of the first tile in the tile group, where the tile address is equal to TileId[ firstCtbAddrTs ] as specified by Equation 6-8 with firstCtbAddrTs being the tile-scan CTB address of the CTBs of the first CTU in the tile group. The length of tile_group_address is Ceil( Log2 ( NumTilesInPic ) ) bits. The value of tile_group_address shall be in the range of 0 to NumTilesInPic − 1, inclusive, and the value of tile_group_address shall not be equal to the value of tile_group_address of any other coded tile group NAL unit of the same coded picture. When tile_group_address is not present it is inferred to be equal to 0.

...

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).