| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0113-v2 |
| --- | --- |

| Title: | AHG16: Subblock-based chroma residual scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VTM4.0, reconstruction of a chroma transform block (TB) has to be delayed until luma prediction of the corresponding luma TB is totally done when enabling chroma residual scaling. This contribution presents two methods to reduce the delay. In Method 1, subblock-based chroma residual scaling is applied. Each chroma TB is first converted into one or multiple M×N chroma subblocks, where M<=8 and N<=8 (M<8 when chroma TB width < 8; N<8 when chroma TB height < 8). The chroma residual inverse scale value of each chroma subblock is derived from the corresponding luma subblock. The delay of chroma reconstruction is reduced to the processing time of generating prediction samples for one luma subblock instead of one luma TB. In Method 2, the proposed subblock-based chroma residual scaling in Method 1 is further simplified. Instead of calculating the chroma residual inverse scale value for each M×N chroma subblock, one luma region of up to 256 luma samples corresponding to the chroma TB is used to derive one chroma residual inverse scale value, which is shared by the entire chroma TB. It is reported that Method 1 achieves 0.00%/0.03%/0.04% and 0.01%/0.25%/0.11% Y/Cb/Cr BD-rates with negligible encoding and decoding time changes for VTM4.0-RA and VTM4.0-LB, respectively. In Method 2, 0.01%/-0.02%/-0.01% and -0.01%/0.08%/-0.19% Y/Cb/Cr BD-rates are achieved with negligible encoding and decoding time changes for VTM4.0-RA and VTM4.0-LB, respectively.

# Introduction

## Chroma residual scaling

In VTM4.0, chroma residual scaling [1] is applied to chroma TBs to compensate for luma signal interaction with the chroma signal. The average value of luma prediction samples in the luma TB corresponding to the chroma TB is calculated and mapped to an inverse scale value (CScaleInv) for chroma residual inverse scaling at the decoder. The chroma residual inverse scaling is derived as below.

Decoder side:CRes=CResScale/CScale=CResScale*CScaleInv

In the above equation, CResScale is the reconstructed residual after inverse quantization and inverse transform, and CRes is the scaled chroma residual signal. As shown in Figure 1, the chroma residual scaling is done in the reshaped domain. Therefore, for inter TBs, a forward reshaping is applied to the luma prediction samples before calculating the average value.

Figure 1.  Intra mode and inter mode reconstruction with chroma residual scaling, where luma LF, luma DPB, and luma Motion Compensation are in the original domain and the rest are in the reshaped domain.

In case that intra subpartition (ISP) mode is applied to a coding unit (CU), the luma coding block (CB) is divided into multiple subpartitions, where each subpartition is a TB. The chroma CBs are not divided (i.e., CB = TB for chroma). The chroma residual inverse scale value is derived using the last luma TB in the CU.

## Problem definition

In VTM3.0 reconstruction process, the chroma reconstruction has no dependency on luma prediction. Chroma reconstruction can be processed right after chroma prediction (assuming chroma inverse quantization and chroma inverse transform are ready before chroma prediction).

In VTM4.0, because of the chroma residual scaling shown in Figure 1, the chroma residuals of the entire chroma TB depend on all the luma prediction samples of the entire corresponding luma TB. The chroma reconstruction process cannot start until the luma prediction samples of the entire corresponding luma TB are available. The worst case happens when the luma TB size is 64×64, where the chroma reconstruction is delayed by the processing time of generating the 64×64 luma prediction samples.

# Proposed methods

## Subblock-based chroma residual scaling (Method 1)

To reduce the delay of chroma reconstruction, the chroma residual scaling is done at subblock level. Each chroma TB is first converted into one or multiple M×N chroma subblocks, where M<=8 and N<=8 (M<8 when chroma TB width < 8; N<8 when chroma TB height < 8). The chroma residual inverse scale value of each chroma subblock is derived from the corresponding luma subblock. Therefore, the inverse scaled chroma residual of the first subblock can be derived after its corresponding luma subblock prediction samples, instead of all corresponding luma TB prediction samples, are available.

In case of ISP mode, each luma TB (i.e., subpartition) is converted into one or multiple P×Q luma subblocks (note that the subblocks here are different from subpartitions), where P<=16 and Q<=16 (P<16 when luma TB width < 16; Q<16 when luma TB height < 16). For each luma subblock, a chroma residual inverse scale value is derived.

## Simplified subblock-based chroma residual scaling (Method 2)

To reduce the computation complexity of Method 1, instead of calculating the chroma residual inverse scale value for each M×N chroma subblock, one luma region of up to 256 luma samples corresponding to the chroma TB is used to derive one chroma residual inverse scale value. The derived chroma residual inverse scale value is shared by the entire chroma TB.

The luma region is defined as follows. If the corresponding luma TB size is less than or equal to 256 luma samples, the luma region is the entire luma TB. Otherwise, if both the corresponding luma TB width and height are larger than 16, the luma region is the top-left 16×16 luma samples. Otherwise (in this case the corresponding luma TB width or height is less than 16), the luma region is the top-left 256 luma samples of the luma TB.

# Experiment results

The proposed methods have been implemented on VTM4.0 [2]. The common test conditions [3] are used for evaluation. The results of Method 1 and Method 2 are summarized in Table 1 and Table 2, respectively.

Table 1.  Results of the subblock-based chroma residual scaling (Method 1)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.00% | 101% | 102% |
| Class A2 | -0.02% | 0.00% | -0.04% | 100% | 99% |
| Class B | 0.00% | 0.00% | 0.08% | 100% | 101% |
| Class C | 0.00% | 0.15% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.04% | 100% | 101% |
| Class D | 0.01% | 0.06% | 0.02% | 101% | 100% |
| Class F | 0.01% | 0.02% | -0.05% | 100% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.18% | 0.18% | 100% | 102% |
| Class C | 0.00% | 0.24% | 0.22% | 101% | 101% |
| Class E | 0.02% | 0.36% | -0.17% | 100% | 96% |
| Overall | 0.01% | 0.25% | 0.11% | 100% | 100% |
| Class D | -0.03% | -0.15% | -0.46% | 101% | 102% |
| Class F | -0.09% | -0.16% | 0.02% | 100% | 101% |

Table 2.  Results of the simplified subblock-based chroma residual scaling (Method 2)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.09% | 0.04% | 100% | 102% |
| Class A2 | -0.01% | -0.07% | 0.00% | 100% | 101% |
| Class B | 0.01% | -0.04% | 0.02% | 100% | 101% |
| Class C | 0.02% | -0.02% | -0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.02% | -0.01% | 100% | 101% |
| Class D | 0.00% | 0.07% | 0.09% | 102% | 101% |
| Class F | -0.01% | 0.02% | -0.02% | 101% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.15% | -0.24% | 100% | 104% |
| Class C | -0.03% | 0.18% | 0.13% | 100% | 101% |
| Class E | 0.00% | 0.33% | -0.52% | 101% | 96% |
| Overall | -0.01% | 0.08% | -0.19% | 100% | 101% |
| Class D | -0.02% | 0.23% | -0.30% | 101% | 102% |
| Class F | 0.00% | 0.15% | 0.16% | 100% | 101% |

# Conclusions

This contribution presents two methods to reduce the delay of chroma reconstruction when enabling chroma residual scaling. In Method 1, the chroma residual scaling is done at chroma subblock level. Method 2 furthers simplifies Method 1 using reduced operations. Method 1 reportedly show 0.00%/0.03%/0.04% and 0.01%/0.25%/0.11% Y/Cb/Cr BD-rates with negligible encoding and decoding time changes for VTM4.0-RA and VTM4.0-LB, respectively. Method 2 reportedly achieves 0.01%/-0.02%/-0.01% and -0.01%/0.08%/-0.19% Y/Cb/Cr BD-rates with negligible encoding and decoding time changes for VTM4.0-RA and VTM4.0-LB, respectively.

# References

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, and T. Chen, “CE12: Mapping functions (test CE12-1 and CE12-2),” Document of Joint Video Experts Team, JVET-M0427.

- VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).