| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0115-v1 |
| --- | --- |

| Title: | AHG14/AHG17: GDR – Gradual decoding refresh |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Ye-Kui Wang Jianle Chen Seungwook Hong 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | fnu.hendry@huawei.com yekui.wang@huawei.com jianle.chen@huawei.com seungwook.hong@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a design for supporting gradual decoding refresh (GDR) for VVC. It is asserted that techniques for supporting GDR that are being studied in AHG14 are unnecessarily complex. It is also asserted that the use of constrained intra prediction (CIP) and encoder constraints on motion vector search causes sub-optimal coding performance.

The proposed GDR design is summarized as follows:

- A new VCL NAL unit with type, GDR_NUT, is defined. A picture with VLC NAL units of type GDR_NUT is a GDR picture and is the first picture in a GDR period.

- The access unit containing a GDR picture may start a CVS and consequently may start a bitstream.

- A flag called gdr_enabled_flag is signalled in the SPS to indicate whether GDR pictures may be present in CVSs referring to the SPS.

- When decoding starts from a GDR picture, the GDR picture is treated similarly to CRA picture treatment.

- Progressive intra refresh for GDR is performed based on intra coding of entire tile groups.

- Neither CIP nor constraining of MVs is needed. Instead, the TMVP derivation process and the interpolation processes are modified such that a boundary of the refreshed region in a reference picture is treated as if it were a picture boundary in decoding of a current block referring to that reference picture for inter prediction.

- A flag called loop_filter_across_refreshed_region_enabled_flag is define to specify the in-loop filter operations at the boundary between the refreshed region and the un-refreshed region.

Text changes are provided in the attachment of this contribution.

# Introduction

Techniques for supporting gradual decoding refresh (GDR), which is also called progressive intra refresh (PIR), for VVC have been proposed since the 11th JVET meeting by contributions [1] − [4] and are currently being studied by AHG14. The discussions on this topic have been mostly in regards to the following questions:

- What are the problem(s) and which use case(s) need this feature?

- Are there any other known techniques that can be used instead of using GDR as it has been shown that the technique costs coding performance significantly? For example, can DU based system be used to address the claimed problem(s)?

- Can the current techniques for performing GDR that is being studied in AHG14 be improved?

It was asserted that GDR is useful to reduce the end-to-end delay [2][3]. JVET-N0114 provides an in-depth analysis showing that indeed GDR is useful for reducing the end-to-end delay.

JVET-M0529 [5] proposed that the indication of the recovery point is specified in VVC more "normatively" than using SEI messages.

# Analysis and discussions

## Existing techniques for GDR

Techniques for supporting GDR that are currently being studied in AHG14 are mainly based on contributions [1] − [4]. The techniques can be briefly summarized as follows:

- Intra prediction mode forced on coding unit on column basis.

- Constrained Intra Prediction (CIP) is enabled to ensure reconstruction of intra CU.

- Encoder constraint are applied to constraint motion vectors to point within the refreshed area while taking into account an additional margin to avoid filters error spreading (6 pixels for instance).

- Former reference pictures are removed when re-looping the intra column.

- Disabling of Adaptive Loop Filter (ALF) at CTU level for CTU including Intra refresh pixels.

Using the above techniques, the reported performance degradation by [4] is about 21.29% for luma in average and by [3] is about 28.66% for luma in average against a low-delay B reference with a comparable Intra period of 32. Note that the performance loss difference between [4] and [3] is probably due the referencing structure that is used for the tests.

Some normative changes were proposed in the effort to reduce the performance loss. The proposed normative changes reportedly provide improvement of 5.26% against the encoder-only modification (i.e., against the reported performance loss of 21.29%). These normative changes include:

- Avoid the transmission of pred_mode_flag for Coding Unit with at least one pixel within the Intra Refresh Area.

- Enable the Constrained Intra Prediction (CIP) tool adaptively depending on the position and direction of the Intra Refresh Area.

- Disabling of Deblocking Filter at the Intra Refresh boundary.

In our observation, the existing techniques have at least the following issues:

- Forcing intra prediction mode for CU on column basis may cause coding sub-optimality as it interferes with coding block splitting algorithm. Furthermore, it seems to cause complicated implementation since one needs to trace the refresh direction at CU granularity.

- The refreshed region that is coded only with intra-prediction is not CTU size, instead it can go smaller than CTU size, down to minimum CU size. This makes the implementation unnecessary complicated as it may need indication at block level.

- The use of CIP is harmful to intra blocks in the un-refreshed region. Although it is necessary to code refreshed region that is coded with intra-prediction with some constraints to prevent any samples from un-refreshed region to be used for spatial reference, when CIP is used, it is picture based, meaning that all intra blocks in the picture must also be coded as CIP intra blocks. This consequently causes performance degradation. The proposed normative change such that CIP is used adaptively depending on the position and direction of the intra refresh area requires additional complexity as it is necessary to trace the intra refresh direction.

- The use of encoder constraint to limit motion search prevents encoder to choose the best motion vector if samples of the reference block associated with the motion vector are not completely within the refreshed region in the reference picture. Figure 1 illustrates the reason for non-optimality in motion search when using encoder restriction for supporting GDR. During motion search process, the encoder is constrained to not select any motion vector which resulted in some of samples of the reference block located outside the refreshed region, even if that reference block is the best reference block in rate-distortion cost criteria.

Figure 1 – Problem with encoder constraints for motion search.

## Normative indication for GDR

Normative indication proposed by [5] can briefly be summarized as follows:

- A new non-VCL NAL unit with NAL unit type Recovery Point Indication (RPI_NUT) is defined. The payload of the NAL unit contains syntax element to specify information which can be used to derive the POC value of the last picture in the GDR period.

- The access unit that contains the non-VCL NAL unit with type RPI_NUT is called as Recovery Point Begin (RBP) access unit and the picture in the RBP access unit is called RBP picture.

- Decoding process can start from RBP AU.

- When decoding starts from RBP AU, all pictures in the GDR period, except the last picture, are not outputted.

In our observation, the proposed indication and how it works to enable GDR picture to start a CVS have the following issues:

- For indicating the first the last pictures in the GDR period, a new non-VCL NAL unit is needed. This is a redundancy as the information contained in the RPI NAL unit can simply be included in the tile group header of the first picture in the GDR period. It was argued that by doing so it would be a problem if the GDR picture is lost; however, such problem has also been the problem for IRAP pictures.

- The proposed method is not able to describe which region in the pictures in the GDR period are refreshed region and un-refreshed region.

- The proposed method does not describe how GDR is performed, instead, it only provide some signalling for indicating the first and the last pictures in the GDR period. It is desirable that the high level indication for supporting GDR can work together with method for performing GDR.

# Proposal

Based on our analysis and discussions above, we believe that the technique for supporting GDR and its indication should be aligned.

The proposed GDR design is summarized as follows:

- A VCL NAL unit type, GDR_NUT, is defined.

- A coded video sequence (CVS) may start with a GDR access unit (and consequently a GDR access unit may start a bitstream).

- A flag called gdr_enabled_flag for specifying whether GDR is enabled is signalled in the SPS.

- Information that can be used to derive the POC value of the last picture in the GDR period is signalled in tile group header of GDR picture. It can be signalled similarly to how recover_point_cnt syntax element was defined in recovery point SEI message in HEVC.

- A flag called refreshed_region_flag to specify whether a tile group is part of refreshed region or not is signalled in the tile group header. All tile groups having refreshed_region_flag equal to 1 cover a connected region. A tile group with refreshed_region_flag can be of type I- (i.e., intra tile group) or B- or P- (i.e., inter tile group). Each picture starting from the GDR picture to the last picture in the GDR period contains at least one tile group with refreshed_region_flag equal to 1. A GDR picture contains at least one tile group with refreshed_region_flag equal to 1 that is intra coded.

- When gdr_enabled_flag is equal to 1, information of rectangular tile groups, i.e., the number of tile groups and their addresses, is allowed to be signalled in either the PPS or in tile group header.

- A flag called loop_filter_across_refreshed_region_enabled_flag is signalled to specify whether in-loop filtering operation across the boundaries between refreshed region and un-refreshed region in a picture is allowed. When this flag is equal to 1 for a tile group of the refreshed region, the following applies:

- Deblocking of the edge at the border of the tile group is not performed if the neighbor tile group that share the edge is an un-refreshed tile group.

- The SAO process for a block at the boundary of the tile group does not use any samples from outside the refreshed region boundary.

- The ALF process for a block at the boundary of the tile group does not use any samples from outside the refreshed region boundary.

- The boundaries of the refreshed region in a picture are derived based on the values of the refreshed_region_flag in the tile groups of the picture.

- When the current block in the current picture is in a tile group with refreshed_region_flag equal to 1 and the reference block is in a reference picture that contains un-refreshed region, a boundary of the refreshed region in a reference picture is treated as if it were a picture boundary in decoding of the current block referring to that reference picture for inter prediction, by applying the modified TMVP derivation and interpolation processes.

Text changes relative to JVET-M1001-v5 are provided in the attachment of this contribution.

# References

- J.-M. Thiesse and D. Nicholson, "Improved Cyclic Intra Refresh", JVET-K0212, July 2018.

- J.-M. Thiesse, D. Nicholson, and D. Gommelet, "AHG14: Intra Refresh Test conditions and Anchors generation Proposal", JVET-L0160, October 2018.

- K. Kazui, "AHG14: Software for ultra low-latency encoding", JVET-M0197, January 2019.

- J.-M. Thiesse, D. Nicholson, and D. Gommelet, "AHG14: Updates on Intra Refresh Proposal", JVET-M0387, January 2019.

- M. Petterson, R. Sjöberg, and M. Damghanian, "AHG14: Normative Recovery Point Indication", JVET-M0529, January 2019.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).