| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0118-v2 |
| --- | --- |

| Title: | AHG19: Adaptive resolution change (ARC) support in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seungwook Hong Ye-Kui Wang Jianle Chen Yu-Chen Sun Tsui-Shan Chang Jian Lou | Tel: Email: | fnu.hendry@huawei.com seungwook.hong@huawei.comyekui.wang@huawei.com jianle.chen@huawei.com yuchen.s@alibaba-inc.com tsuishan.cts@alibaba-inc.com j.lou@alibaba-inc.com |
| Source: | Huawei Technologies Co., Ltd. and Alibaba Group |  |  |

_____________________________

# Abstract

This contribution reports a software implementation effort for supporting the so-called adaptive resolution change (ARC) feature on top of the VTM-4.0.1 reference software. Not all aspects of the design currently used in the implementation are intended to be the final design of ARC supported in VVC, rather, the implementation is intended as a testbed to help the study of related aspects of ARC. As the development of ARC progresses and more ARC design aspects are agreed by the JVET experts, the implementation is expected to be updated accordingly. It is recommended that the software is used as the reference software for the ARC work in AHG19.

In addition to the reference software effort, this contribution also discusses and proposes several aspects of ARC design.

The following aspects are proposed:

- A list of picture resolutions is signalled in the SPS, and an index to the list is signalled in the PPS to specify the size of an individual picture.

- For any picture that is to be outputted, the decoded picture before resampling is cropped (as necessary) and outputted, i.e., a resampled picture is not for output, only for inter prediction reference.

- Support 1.5x and 2x resampling ratios. No support of arbitrary resampling ratio. Further study the need of one or two more other resampling ratios.

- Between picture-level resampling and block-level resampling, the proponents prefer block-level resampling.

  - However, if picture-level resampling is chosen, the following aspects are proposed:

    - When a reference picture is resampled, both the resampled version and the original, resampled version of the reference picture are stored in the DPB, and thus both would affect the DPB fullness.

    - A resampled reference picture is marked as "unused for reference" when the corresponding un-resampled reference picture is marked as "unused for reference".

    - The RPL signalling syntax is kept unchanged, while the RPL construction process is modified as follows: When a reference picture needs to be included into a RPL entry, and a version of that reference picture with the same resolution as the current picture is not in the DPB, the picture resampling process is invoked and the resampled version of that reference picture is included into the RPL entry.

    - The number of resampled reference pictures that may be present in the DPB should be limited, e.g., to be less than or equal to 2.

  - Otherwise (block-level resampling is chosen), the following are suggested:

    - To limit the worst-case decoder complexity, it is proposed that bi-prediction of a block from a reference picture with a different resolution than the current picture is disallowed.

    - Another option is that, when resampling and quarter-pel interpolation need to be done, the two filters are combined and the operation is applied at once.

- Regardless of which of the picture-based and block-based resampling approaches is chosen, it is proposed that temporal motion vector scaling is applied as needed.

- Start a CE on ARC resampling filters.

In the first revision of this contribution (-v2), an updated spreadsheet of simulation results for test provided in -v1 is added.

# Introduction

At the end of the 13th JVET meeting in Marrakech, AHG19 was established with one of its mandates being to study adaptive resolution change (ARC) approaches for real-time communications, adaptive streaming, and 360-degree viewpoint-dependent streaming, including filters for resampling, reference picture management, and related scope and signalling. Motivations for ARC support can be found in JVET-M0135 [1] and JVET-M0259 [2].

To facilitate the ARC study in AHG19, we have started an effort to add the support of ARC into the VTM reference software, including reference picture resampling and use of a reference picture having a different spatial resolution than the current picture. Although the current ARC features in the implementation may not be based on the best design, the implementation can hopefully help the ARC study. For example, the software can be used as the basis for implementing tools that need to be modified and tested for supporting ARC. The description of the ARC software implementation and simulation results are provided in section 2.

A discussion and a proposed for the ARC design are provided in section 3.

# Software implementation

## Overview

The ARC software was implemented on top of VTM-4.0.1, with the following changes:

- A list of supported resolutions is signalled in SPS.

- The spatial resolution signalling was moved from SPS to PPS.

- A picture-based resampling scheme was implemented for resampling reference pictures. After a picture is decoded, the reconstructed picture may be resampled to a different spatial resolution. The original reconstructed picture and the resampled reconstructed picture are both stored in the DPB and are available for reference by future pictures in decoding order.

- The implemented resampling filters are based on the filters tested in [3], as follows:

  - The up-sampling filter: 4-tap +/- quarter-phase DCTIF with taps (-4, 54, 16, -2)/64

  - The down-sampling filter: the h11 filter with taps (1, 0, -3, 0, 10, 16, 10, 0, -3, 0, 1)/32

- When constructing the reference picture lists of the current picture (i.e., L0 and L1), only the reference pictures with the same resolution as the current picture are used. Note that the reference pictures may be available in both their original sizes or the resampled sizes.

- TMVP and ATVMP may be enabled; however, when the original coding resolutions of the current picture and a reference picture are different, TMVP and ATMVP are disabled for that reference picture.

- For convenience and simplicity of the starting-point software implementation, when outputting a picture, the decoder outputs the highest available resolution.

## Simulations and analysis

### Network bandwidth adaptation use cases

For studying this type of use cases, including video telephony and conferencing applications in which the bit rate may need to be reduced during certain period to adapt to the changes in network bandwidth, we did a simulation as follows:

- For the anchor, we ran VTM-4.0.1 with the low-delay configuration and inserted an IDR picture whenever a resolution change is needed.

- For ARC, we ran the modified software with an IDR picture only at the beginning of the bitstream, while an inter-coded picture referring to a reference picture with a different spatial resolution was inserted when a resolution change is needed.

- To simulate the network bandwidth change, the spatial resolution of coded pictures was reduced by half for both for the picture width and the picture height at the 3rd second and restored back to its original width and height at the 7th second.

Table 1 tabulates the partial results showing the performance of the implemented ARC software for the simulation setup above. It is expected that the performance of ARC software would increase as more mature design of ARC is agreed and implemented.

Table 1 Performance of implemented ARC software 
over LD configuration with IDR when resolution changes is needed.

| Low delay B Main10 |  |  |  |
| --- | --- | --- | --- |
|  | Anchor |  |  |
|  | Y | U | V |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C | -2.06% | -3.44% | -2.23% |
| Class E | -12.01% | -27.62% | -24.89% |
| Overall | 0.00% | 0.00% | 0.00% |
| Class D | -1.88% | -5.33% | -3.35% |
| Class F (optional) |  |  |  |

Figure 1 and Figure 2 show the bit-count fluctuation between anchor bitstream and test bitstream. As expected, the anchor, which is coded by introducing IDR picture for changing resolution (i.e., at picture 30 and picture 30) exhibits bit-count spike at the beginning of resolution change. On the other hand, the bitstream coded with ARC software does not exhibit any bit-count spike at resolution changing points.

Figure 1 – Coded bit-count fluctuation for RacehorseC sequence coded with VTM-4.0.1

Figure 2 – Coded bit-count fluctuation for RacehorseC sequence coded with ARC software

# Discussions and proposals

## On signalling of picture sizes and picture output

- On the list of spatial resolutions of coded pictures in the bitstream

Currently all coded pictures in a CVS have the same resolution. Thus it is straightforward to signal just one resolution (i.e., picture width and height) in the SPS. With ARC support, instead of one resolution, a list of picture resolutions needs to be signalled, and we propose that this list is signalled in the SPS, and an index to the list is signalled in the PPS to specify the size of an individual picture.

- On picture output

We propose that, for any picture that is to be outputted, the decoded picture before resampling is cropped (as necessary) and outputted, i.e., a resampled picture is not for output, only for inter prediction reference. The ARC resampling filters should be designed to optimize the use of the resampled pictures for inter prediction, and such filters may not be optimal for picture outputting/displaying purpose, while video terminal devices usually have optimized output zooming/scaling functionalities already implemented.

## On resampling

Resampling of a decoded picture can be either picture-based or block-based. For the final ARC design in VVC, we prefer block-based resampling over picture-based resampling. We recommend that these two approaches are discussed and the JVET makes a decision on which of these two should be specified for ARC support in VVC.

### Picture-based resampling

In picture-based resampling for ARC, a picture is resampled only once for a particular resolution, which is then stored in the DPB, while the un-resampled version of the same picture is also kept in the DPB.

Employing picture-based resampling for ARC has two issues: 1) additional DPB buffer is required for storing resampled reference pictures, and 2) additional memory bandwidth is required since due to increased operations of reading reference picture data from the DPB and writing reference picture data into the DPB.

Keeping only one version of a reference picture in the DPB would not a good idea for picture-based resampling. If we keep only the un-resampled version, a reference picture may need to be resampled multiple times since multiple pictures may refer to the same reference picture, and . On the other hand, if a reference picture is resampled and we only keep the resampled version, then we need to apply inverse resampling when the reference picture needs to be outputted, since it's better to output un-resampled pictures, as discussed above. This is a problem since resampling process is not a lossless operation. Take a picture A and downsample it then upsample it to get A' with the same resolution as A, A and A' would not be the same; A' would contain less information than A since some high frequency information has been lost during the downsampling and upsampling processes.

To deal with the issues of additional DPB buffer and memory bandwidth, we proposed that, if the ARC design in VVC uses picture-based resampling, the following applies:

- When a reference picture is resampled, both the resampled version and the original, resampled version of the reference picture are stored in the DPB, and thus both would affect the DPB fullness.

- A resampled reference picture is marked as "unused for reference" when the corresponding un-resampled reference picture is marked as "unused for reference".

- The reference picture lists (RPLs) of each tile group contain reference pictures that have the same resolution as the current picture. While there is no need for a change to the RPL signalling syntax, the RPL construction process is modified to ensure what is said in the previous sentence, as follows: When a reference picture needs to be included into a RPL entry while a version of that reference picture with the same resolution as the current picture is not yet available, the picture resampling process is invoked and the resampled version of that reference picture is included.

- The number of resampled reference pictures that may be present in the DPB should be limited, e.g., to be less than or equal to 2.

Furthermore, to enable temporal MV usage (e.g. merge mode and ATMVP) for the case that the temporal MV comes from the reference frame with different resolution than the current one, we propose that, scaling temporal MV to the current resolution as needed.

### Block-based ARC resampling

In block-based resampling for ARC, a reference block is resampled whenever needed, and no resampled picture is stored in the DPB.

The main issue here is the additional decoder complexity. This is because a block in a reference picture may be referred to multiple times by multiple blocks in another picture and by blocks in multiple pictures.

When a block in a reference picture is referred to by a block in the current picture and the resolutions of the reference picture and the current picture are different, the reference block is resampled by invocation of the interpolation filter such that the reference block has the integer-pel resolution. When the motion vector is in quarter-pel, the interpolation process is invoked again to obtain the resampled reference block in the quarter-pel resolution. Therefore, for each motion compensation operation for the current block from a reference block involving different resolutions, up to two, instead of one, interpolation filtering operations are needed. Without ARC support, up to only one interpolation filter operation (i.e., for generation of the reference block in the quarter-pel resolution) is needed.

To limit the worst-case complexity, we propose that, if the ARC design in VVC uses block-based resampling, the following apply:

- Bi-prediction of a block from a reference picture with a different resolution than the current picture is disallowed.

- To be more more precise, the constraint is as follow: For the current block blkA in the current picture picA refers to a reference block blkB in a reference picture picB, when picA and picB have different resolutions, block blkA shall be a uni-predicted block.

With this constraint, the worst-case number of interpolation operations needed to decode a block is limited to two. If a block refers to a block from a different-resolution picture, the number of interpolation operations needed is two as discussed above. This is the same as in the case when the block refers to a reference block from a same-resolution picture and coded as a bi-predicted block since the number of interpolation operations is also two (i.e., one for getting the quarter-pel resolution for each reference block).

To simplify the implementation, we propose another variation that if the ARC design in VVC uses block-based resampling, the following apply:

- If the reference frame and current frame have different resolutions, the corresponding positions of every pixel of predictors are calculated first, and then the interpolation is applied only one time. That is, two interpolation operations (i.e. one for resampling and one for quarter-pel interpolation) are combined into only one interpolation operation. The sub-pel interpolation filters in the current VVC can be reused, but, in this case, the granularity of interpolation should be enlarged but the interpolation operation times are reduced from two to one.

- To enable temporal MV usage (e.g. merge mode and ATMVP) for the case that the temporal MV comes from the reference frame with different resolution than the current one, we propose that, scaling temporal MV to the current resolution as needed.

## Resampling ratios

In JVET-M0135 [1], to start the discussion on ARC, we proposed that for the starting point of ARC, consider only the resampling ratio of 2x (meaning 2 x 2 for upsampling and 1/2 x 1/2 for downsampling). From further discussion on this topic after the Marrakech meeting, we learned that supporting only the resampling ratio of 2x is very limited, as in some cases a smaller difference between resampled and un-resampled resolutions would be more beneficial.

Although support of arbitrary resampling ratio may be desirable, it support seemed difficult. This is because to support arbitrary resampling ratio, the number of resampling filters that have to be defined and implemented seemed to be too many and to impose a significant burden on decoder implementations.

We propose that more than one but a small number of resampling ratios should be supported, but at least 1.5x and 2x resampling ratios, and arbitrary resampling ratio is not supported.

## Max DPB buffer size and buffer fullness

With ARC, the DPB may contains decoded pictures of different spatial resolutions within the same CVS. For DPB management and related aspects, counting DPB size and fullness in units decoded picture no longer works.

Below are discussions of some specific aspects that need to be addressed and possible solutions in the final VVC specification if ARC is supported (we are not proposing to adopt the possible solutions at this meeting):

- Rather than using the value of PicSizeInSamplesY (i.e., PicSizeInSamplesY = pic_width_in_luma_samples * pic_height_in_luma_samples) for deriving MaxDpbSize (i.e., maximum number of reference picture that may present in the DPB), derivation of MaxDpbSize is based on the value of MinPicSizeInSamplesY. MinPicSizeInSampleY defined as follows:

MinPicSizeInSampleY = (Width of the smallest picture resolution in the bitstream) * 
	 	 	 	 	 	 (Height of the smallest resolution in the bitstream)

The derivation of MaxDpbSize is modified as follows (based on the HEVC equation):

if( MinPicSizeInSamplesY  <=  ( MaxLumaPs  >>  2 ) )
	MaxDpbSize = Min( 4 * maxDpbPicBuf, 16 )
else if( MinPicSizeInSamplesY  <=  ( MaxLumaPs  >>  1 ) )
	MaxDpbSize = Min( 2 * maxDpbPicBuf, 16 ) 
else if( MinPicSizeInSamplesY  <=  ( ( 3 * MaxLumaPs )  >>  2 ) )
	MaxDpbSize = Min( ( 4 * maxDpbPicBuf ) / 3, 16 )
else
	MaxDpbSize = maxDpbPicBuf

- Each decoded picture is associated with a value called PictureSizeUnit. PictureSizeUnit is an integer value that specifies how big a decoded picture size is relative to the MinPicSizeInSampleY. The definition of PictureSizeUnit depends on what resampling ratios are supported for ARC in VVC.

For example, if ARC supports only the resampling ratio of 2, the PictureSizeUnit is defined as follows:

- Decoded pictures having the smallest resolution in the bitstream are associated with PictureSizeUnit of 1.

- Decoded pictures having the resolution that is 2 by 2 of the smallest resolution in the bitstream is associated with PictureSizeUnit of 4 (i.e., 1 * 4).

For another example, if ARC supports both the resampling ratios of 1.5 and 2, the PictureSizeUnit is defined as follows:

- Decoded pictures having the smallest resolution in the bitstream is associated with PictureSizeUnit of 4.

- Decoded pictures having the resolution that is 1.5 by 1.5 of the smallest resolution in the bitstream is associated with PictureSizeUnit of 9 (i.e., 2.25 * 4).

- Decoded pictures having the resolution that is 2 by 2 of the smallest resolution in the bitstream is associated with PictureSizeUnit of 16 (i.e., 4 * 4).

For other resampling rations supported by ARC, the same principle as given by the examples above should be used to determine the value of PictureSizeUnit for each picture size.

- Let the variable MinPictureSizeUnit be the smallest possible value of PictureSizeUnit. That is, if ARC supports only resampling ratio of 2, MinPictureSizeUnit is 1; if ARC supports resampling ratios of 1.5 and 2, MinPictureSizeUnit is 4; likewise, the same principle is used to determine the value of MinPictureSizeUnit.

- The value range of sps_max_dec_pic_buffering_minus1[ i ] is specified to range from 0 to (MinPictureSizeUnit * (MaxDpbSize – 1)). The variable MinPictureSizeUnit is the smallest possible value of PictureSizeUnit.

- The DPB fullness operation is specified based on PictureSizeUnit as follows:

- The HRD is initialized at decoding unit 0, with both the CPB and the DPB being set to be empty (the DPB fullness is set equal to 0).

- When the DPB is flushed (i.e., all pictures are removed from the DPB), the DPB fullness is set equal to 0.

- When a picture is removed from the DPB, the DPB fullness is decrement by the value of PictureSizeUnit associated with the removed picture.

- When a picture is inserted into the DPB, the DPB fullness is increment by the value of PictureSizeUnit associated with the inserted picture.

## Resampling filters

In the software implementation, the implemented resampling filters were simply taken from previously available filters described in JCTVC-H0234 [3]. Other resampling filters should be tested and used if they provide better performance and/or lower complexity. We propose that various resampling filters to be tested to strike a trade-off between complexity and performance. Such tests can be done in a CE.

## Miscellaneous necessary modifications to existing tools

To support ARC, some modifications and / or additional operations may be needed to some of the existing coding tools. For example, in the ARC software implementation picture-based resampling, for simplicity we disabled TMVP and ATMVP when the original coding resolutions of current picture and reference picture are different.

We recommend to study the impact of ARC on existing coding tools, at least for TMVP and ATMVP, and whether any modification or constraint on certain existing tools is needed for supporting ARC in VVC. Such study can be part of the AHG19 activity.

# References

- Hendry, Y.-K. Wang, J. Chen, T. Davies, A. Fuldseth, Y.-C. Sun, T.-S Chang, and J. Lou, "On adaptive resolution change (ARC) for VVC", JVET-M0135, January 2019.

- M. M. Hannuksela and A. Aminlou, " Use cases and proposed design choices for adaptive resolution changing (ARC)", JVET-M0259, January 2019.

- T. Davies, "AHG18: Design considerations for Adaptive Resolution Coding (ARC)", JCTVC-H0234, February 2012.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. and Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).