| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N0119-v1 |
| --- | --- |

| Title: | AHG17: EDR – external decoding refresh |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | +1-908-803-3888 yekui.wang@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a design for support random accessing a bitstream from inter-coded pictures without a recovery period. Such random accessing needs a small number of reference pictures to be provided by an external means, thus named external decoding refresh (EDR).

The proposed design is summarized as follows:

- An EDR picture is a random accessible picture for which one or more external pictures are needed when random accessing from the picture.

- An EDR picture may start a CVS (hence may also start a bitstream).

- External pictures are provided in the form of coded pictures instead of decoded pictures.

- There is no change to RPL syntax, semantics, and derivation process.

- A new NAL unit type (EDR_NUT) is defined for EDR pictures.

- For each EDR picture, the POC MSB values of the external pictures and the MSB values are signalled in the tile group header.

- A flag is signalled in the SPS to specifies whether EDR pictures may be present in the CVS.

The proposed detailed spec text changes are provided in an attachment.

# Introduction

## Cross RAP referencing (CRR) based video coding

In AVC, HEVC, and the current design of VVC, an IRAP picture is not only coded as an intra picture, but also the decoding of an IRAP picture when random accessing from the IRAP picture would flush the DPB, and consequently pictures earlier than the IRAP picture would not be available for inter-predication reference by pictures following the IRAP picture in decoding.

At the previous JVET meeting, a so-called cross RAP referencing (CRR) based video coding approach was proposed in JVET-M0360, to circumvent the unavailability of the earlier pictures than an IRAP picture. The approach in JVET-M0360 employs a trick by providing a limited number of the earlier pictures, typically representing different scenes of the video content, through a separate video bitstream, which can be referred to as an external means. Such earlier pictures are referred to as the library pictures in JVET-M0360. Consequently, each library picture can be used for inter prediction referencing by pictures across IRAP pictures that start a CVS.

As reported in JVET-M0360, the approach can provide significant video compression gain of up to about 37% for certain video content.

Below is a brief description of signalling, decoding process, and random access operations of the CRR approach included in JVET-M0360.

The library pictures that can be used for inter prediction cross IRAP pictures are carried in a separate bitstream from the main bitstream. Library pictures are not output. Pictures for output are carried in the main bitstream. In the specification, the decoded library pictures are specified as provided by external means, similarly as in SHVC for the base layer pictures when provided by external means.

Again similarly as in SHVC for the base layer pictures when provided by external means, the decoded pictures of the library pictures are stored in a separate DPB. The separate DPB is accessibly the decoder of the main bitstream. The syntax of the main bitstream is changed to enable one or more reference indices of a reference picture list of a main-bitstream picture pointing to one or more of the library pictures. Aside that the reference picture list construction, the decoding process for CTUs remains the same.

When performing random access at a particular RAP picture, the library pictures corresponding to main-bitstream pictures earlier than the RAP picture in decoding order are made available to the main-bitstream decoder. As mentioned above, library pictures are provided to the main-bitstream decoder by external means. In practical systems, e.g., in the ISOBMFF and DASH environment, the library pictures are carried in a separate track/representation than the track(s)/representation(s) carrying the main-bitstream pictures, and the tracks/representations are timely synchronized through presentation times, and the file format and DASH metadata (e.g., in the file format sample entries and the DASH MPD) includes association of the tracks. When a streaming client requests a main-bitstream segment, the library segment(s) that contains the required library pictures are also requested, preferably a bit before the main-bitstream segment.

## Discussion

The JVET meeting minutes noted under JVET-M0360 are as follows:

This contribution proposes an approach to allow inter prediction referencing across random access points (RAPs), referred to as cross-RAP referencing (CRR). Simulation results reportedly show BD coding gains in the range from 11.21% to 25.91% based on VTM3.0 in VVC CTC and from 19.6% to 37.38% based on HM16.15 in HEVC CTC. A brief description of signalling, decoding process, and random access operations is provided. It is proposed to start a CE on this.

The idea of a “library picture” provided by external (unicast) means and referenced across what are otherwise random access points was described.

This proposed scheme does not have any “drift”.

It was noted that the DRAP concept as specified in an HEVC SEI message (see JCTVC-S0095) has a close relationship to this.

It was asked what impact on syntax this would have. It was remarked that, with system support for managing the library, this seems possible with the current syntax (e.g., using long-term reference pictures – possibly more than one of them).

System support would be required no matter what, as the system needs to deliver the library pictures.

Possibly some property indicator could be used to assist – e.g., an SEI message that just indicates that no following pictures in decoding and/or output order will refer to any previous pictures in decoding that are not in the RPS of the current picture.

Further study in an AHG was encouraged to study the properties of such an approach and determine whether this or similar functionality would need to have some impact on the syntax or decoding process. Such an impact does not seem strictly necessary for basically achieving this usage, given appropriate systems support, but may be determined helpful in some way in further study. It was suggested for systems experts to also study this and determine whether file-format / DASH / other system support should be specified for this sort of usage.

The following conclusions were drawn from a study of whether changes to syntax (e.g., RPL syntax) and decoding process are needed for enabling such video coding approach:

- If the necessary reference pictures are provided by an external means in the form of coded pictures instead of decoded pictures, and such random accessible pictures, referred to as external decoding refresh (EDR) pictures for convince) are not coded with IRAP NAL unit types, the functionality can be provided without changing the RPL syntax.

- For each EDR picture, the POC MSBs of the external pictures and the POC MSB of the EDR picture itself need to be signalled to be able to recover the POC relationships among the external pictures, the current picture, and subsequent pictures in decoding order.

- Potentially, it is possible to just signal the POC MSBs using an SEI message, and describe the required decoding process in an informative way, in the same nature as the decoding process description in the semantics of the recovery point SEI message for random accessing from a GDR picture.

- However, to be able to support random accessing from EDR pictures in a conforming way, some changes to the general decoding process are needed to allow such a random accessible picture to start a CVS (and consequently may also start a bitstream).

This contribution proposes a design that allow an EDR picture to start a CVS.

# Proposal

The proposed design is summarized as follows:

- An EDR picture is a random accessible picture for which one or more external pictures are needed when random accessing from the picture.

- It is specified that an EDR picture may start a CVS (hence may also start a bitstream).

- External pictures are provided in the form of coded pictures instead of decoded pictures. External pictures are not output by the decoder; they are for inter prediction referencing only.

- There is no change to RPL syntax, semantics, and derivation process.

- A new NAL unit type (EDR_NUT) is defined for EDR pictures.

  - When random accessing from an EDR picture, provided the necessary parameter sets and external pictures are available when they need to be activated or referred to in inter prediction, the EDR picture and all subsequent non-RASL pictures in decoding order in the current CVS can be correctly decoded.

  - For an EDR picture in a CVS that is not the first picture of the CVS, the external pictures are pictures in the CVS that precede the EDR picture in decoding order. Therefore, when decoding starts from an IRAP or EDR picture preceding the EDR picture in decoding order, no external pictures need to be provided for correct decoding of the EDR picture and all subsequent pictures in decoding order in the CVS.

- For each EDR picture, the POC MSB values of the external pictures and the MSB values are signalled in the tile group header.

- When random accessing from an EDR picture, or when continuous decoding of an EDR picture at a splicing point, in both cases the EDR picture would have the variable NoIncorrectPicOutputFlag equal to 1, the following ordered steps apply:

  - The syntax elements num_external_pics_minus1 and external_pic_poc_msb_delta[ i ] for i ranging from 0 to num_external_pics_minus1, inclusive, are parsed from the tile group header of any tile group of the current picture, and the values of the variables NumExtPics, ExtPicPocMsb[ i ] for i ranging from 0 to num_external_pics_minus1, and CurrPicPocMsb are derived.

  - NumExtPics access units each containing an external picture provided for the current picture by an external means not specified in this Specification are placed at the beginning of the CVS starting from the EDR picture.

  - The external pictures, in increasing order of the external picture indices, are then decoded, followed by the EDR pictures and subsequent pictures in decoding order.

    - When decoding the i-th external picture, its PicOrderCntMsb value is set equal to ExtPicPocMsb[ i ] in the POC derivation process.

    - When decoding the EDR picture, its PicOrderCntMsb value is set equal to CurrPicPocMsb in the POC derivation process.

    - When decoding the 0-th external picture, all reference pictures in the DPB (if any) are marked as "unused for reference".

    - When decoding the EDR picture, marking of all reference pictures in the DPB (if any) are marked as "unused for reference" is not performed.

- RADL pictures and RASL pictures for allowed to be associated with an EDR picture.

- A flag is signalled in the SPS to specifies whether EDR pictures may be present in the CVS.

The proposed detailed spec text changes are provided in an attachment.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).