| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N0119-v2 |
| --- | --- |

| Title: | AHG17: EDR – external decoding refresh |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang, Haitao Yang (Huawei) Hualong Yu, Lu Yu (ZJU) | Tel: Email: | yekui.wang@huawei.com ylonge37@zju.edu.cn |
| Source: | Huawei Technologies Co., Ltd., Zhejiang University |  |  |

_____________________________

# Abstract

This contribution proposes a design for support random accessing a bitstream from inter-coded pictures without a recovery period. Such random accessing needs a small number of reference pictures to be provided by an external means, thus named external decoding refresh (EDR).

The proposed design is summarized as follows:

- An EDR picture is a random accessible picture for which one or more external pictures are needed when random accessing from the picture.

- An EDR picture may start a CVS (hence may also start a bitstream).

- External pictures are provided in the form of coded pictures instead of decoded pictures.

- There is no change to RPL syntax, semantics, and derivation process.

- A new NAL unit type (EDR_NUT) is defined for EDR pictures.

- For each EDR picture, the POC MSB values of the external pictures and the MSB values are signalled in the tile group header.

- A flag is signalled in the SPS to specifies whether EDR pictures may be present in the CVS.

A coded EDR sequence (CES) is defined as a sequence of access units that consists, in decoding order, of an EDR access unit, followed by zero or more access units that are not an EDR access unit and not an IRAP access unit, including all subsequent access units up to but not including any subsequent access unit that is an EDR access unit or IRAP access unit.

In v1 of this contribution, the external pictures associated with each EDR picture are the external pictures needed for decoding of the EDR picture and all the following non-RASL pictures in the CVS in decoding order.

In v2 of this document, an asserted optimization was included, such that the external pictures associated with each EDR picture are the external pictures needed for decoding of the EDR picture and all the following non-RASL pictures in the CES in decoding order.

The proposed detailed spec text changes are provided in an attachment.

# Introduction

## Cross RAP referencing (CRR) based video coding

In AVC, HEVC, and the current design of VVC, an IRAP picture is not only coded as an intra picture, but also the decoding of an IRAP picture when random accessing from the IRAP picture would flush the DPB, and consequently pictures earlier than the IRAP picture would not be available for inter-predication reference by pictures following the IRAP picture in decoding.

At the previous JVET meeting, a so-called cross RAP referencing (CRR) based video coding approach was proposed in JVET-M0360, to circumvent the unavailability of the earlier pictures than an IRAP picture. The approach in JVET-M0360 employs a trick by providing a limited number of the earlier pictures, typically representing different scenes of the video content, through a separate video bitstream, which can be referred to as an external means. Such earlier pictures are referred to as the library pictures in JVET-M0360. Consequently, each library picture can be used for inter prediction referencing by pictures across IRAP pictures that start a CVS.

As reported in JVET-M0360, the approach can provide significant video compression gain of up to about 37% for certain video content.

Below is a brief description of signalling, decoding process, and random access operations of the CRR approach included in JVET-M0360.

The library pictures that can be used for inter prediction cross IRAP pictures are carried in a separate bitstream from the main bitstream. Library pictures are not output. Pictures for output are carried in the main bitstream. In the specification, the decoded library pictures are specified as provided by external means, similarly as in SHVC for the base layer pictures when provided by external means.

Again similarly as in SHVC for the base layer pictures when provided by external means, the decoded pictures of the library pictures are stored in a separate DPB. The separate DPB is accessibly the decoder of the main bitstream. The syntax of the main bitstream is changed to enable one or more reference indices of a reference picture list of a main-bitstream picture pointing to one or more of the library pictures. Aside that the reference picture list construction, the decoding process for CTUs remains the same.

When performing random access at a particular RAP picture, the library pictures corresponding to main-bitstream pictures earlier than the RAP picture in decoding order are made available to the main-bitstream decoder. As mentioned above, library pictures are provided to the main-bitstream decoder by external means. In practical systems, e.g., in the ISOBMFF and DASH environment, the library pictures are carried in a separate track/representation than the track(s)/representation(s) carrying the main-bitstream pictures, and the tracks/representations are timely synchronized through presentation times, and the file format and DASH metadata (e.g., in the file format sample entries and the DASH MPD) includes association of the tracks. When a streaming client requests a main-bitstream segment, the library segment(s) that contains the required library pictures are also requested, preferably a bit before the main-bitstream segment.

## Discussion

The JVET meeting minutes noted under JVET-M0360 are as follows:

This contribution proposes an approach to allow inter prediction referencing across random access points (RAPs), referred to as cross-RAP referencing (CRR). Simulation results reportedly show BD coding gains in the range from 11.21% to 25.91% based on VTM3.0 in VVC CTC and from 19.6% to 37.38% based on HM16.15 in HEVC CTC. A brief description of signalling, decoding process, and random access operations is provided. It is proposed to start a CE on this.

The idea of a “library picture” provided by external (unicast) means and referenced across what are otherwise random access points was described.

This proposed scheme does not have any “drift”.

It was noted that the DRAP concept as specified in an HEVC SEI message (see JCTVC-S0095) has a close relationship to this.

It was asked what impact on syntax this would have. It was remarked that, with system support for managing the library, this seems possible with the current syntax (e.g., using long-term reference pictures – possibly more than one of them).

System support would be required no matter what, as the system needs to deliver the library pictures.

Possibly some property indicator could be used to assist – e.g., an SEI message that just indicates that no following pictures in decoding and/or output order will refer to any previous pictures in decoding that are not in the RPS of the current picture.

Further study in an AHG was encouraged to study the properties of such an approach and determine whether this or similar functionality would need to have some impact on the syntax or decoding process. Such an impact does not seem strictly necessary for basically achieving this usage, given appropriate systems support, but may be determined helpful in some way in further study. It was suggested for systems experts to also study this and determine whether file-format / DASH / other system support should be specified for this sort of usage.

The following conclusions were drawn from a study of whether changes to syntax (e.g., RPL syntax) and decoding process are needed for enabling such video coding approach:

- If the necessary reference pictures are provided by an external means in the form of coded pictures instead of decoded pictures, and such random accessible pictures, referred to as external decoding refresh (EDR) pictures for convince) are not coded with IRAP NAL unit types, the functionality can be provided without changing the RPL syntax.

- For each EDR picture, the POC MSBs of the external pictures and the POC MSB of the EDR picture itself need to be signalled to be able to recover the POC relationships among the external pictures, the current picture, and subsequent pictures in decoding order.

- Potentially, it is possible to just signal the POC MSBs using an SEI message, and describe the required decoding process in an informative way, in the same nature as the decoding process description in the semantics of the recovery point SEI message for random accessing from a GDR picture.

- However, to be able to support random accessing from EDR pictures in a conforming way, some changes to the general decoding process are needed to allow such a random accessible picture to start a CVS (and consequently may also start a bitstream).

This contribution proposes a design that allow an EDR picture to start a CVS.

# Proposal

The proposed design is summarized as follows:

- An EDR picture is a random accessible picture for which one or more external pictures are needed when random accessing from the picture.

- It is specified that an EDR picture may start a CVS (hence may also start a bitstream).

- External pictures are provided in the form of coded pictures instead of decoded pictures. External pictures are not output by the decoder; they are for inter prediction referencing only.

- There is no change to RPL syntax, semantics, and derivation process.

- A new NAL unit type (EDR_NUT) is defined for EDR pictures.

  - A coded EDR sequence (CES) is defined as a sequence of access units that consists, in decoding order, of an EDR access unit, followed by zero or more access units that are not an EDR access unit and not an IRAP access unit, including all subsequent access units up to but not including any subsequent access unit that is an EDR access unit or IRAP access unit.

  - When random accessing from an EDR picture, provided the necessary parameter sets and external pictures are available when they need to be activated or referred to in inter prediction, the EDR picture and all subsequent non-RASL pictures in decoding order in the current CES can be correctly decoded.

  - For a particular EDR picture in a CVS that is not the first picture of the CVS, when decoding starts from or before another EDR picture preceding the EDR picture in decoding order, only those external pictures for the particular EDR picture that have not been provided for earlier decoded EDR pictures need to be provided for correct decoding of the EDR picture and all subsequent pictures in decoding order in the CES.

- For each EDR picture, the POC LSB and MSB values of the external pictures and the MSB value of the EDR picture itself are signalled in the tile group header.

- At the beginning of decoding of each CES CesToDecode, the variable ExternalPicsProvidedFlag is set equal to 0.

- When decoding an EDR picture and ExternalPicsProvidedFlag is set equal to 0, the following ordered steps apply:

  - When the EDR picture starts a CVS (i.e., NoIncorrectPicOutputFlag is equal to 1), all reference pictures currently in the DPB (if any) are marked as "unused for reference".

  - The syntax element num_external_pics_minus1 and the syntax elements external_pic_poc_lsb[ i ] and external_pic_poc_msb_delta[ i ] for i ranging from 0 to num_external_pics_minus1, inclusive, are parsed from the tile group header of any tile group of the current picture, and the values of the variable NumExtPics and the variables ExtPicPocMsb[ i ] and ExtPicFullPoc[ i ] for i ranging from 0 to num_external_pics_minus1 are derived.

  - Among the NumExtPics external pictures for the EDR picture, those that are not currently in the DPB (i.e., for which there is no decoded picture in the DPB that has PicOrderCntVal equal to ExtPicFullPoc[ i ]) are provided by an external means not specified in this Specification. The access units containing these external pictures are inserted into CesToDecode immediately before the EDR access unit in decoding order.

  - The decoding order of the external pictures in CesToDecode shall be in increasing order of the external picture indices. Each external picture shall be intra coded, i.e., it shall not refer to any pictures other than itself for inter prediction in its decoding process.

  - ExternalPicsProvidedFlag is set equal to 1.

- RADL pictures and RASL pictures for allowed to be associated with an EDR picture.

- A flag is signalled in the SPS to specifies whether EDR pictures may be present in the CVS.

The proposed detailed spec text changes are provided in an attachment.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. and Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).