| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N0120-v1 |
| --- | --- |

| Title: | AHG17: Misc. HLS clean-ups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | +1-908-803-3888 yekui.wang@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes the following aspects:

- To add some constraints on IRAP, RASL, and RADL pictures, which are asserted to be technically identical as in HEVC for IRAP, RASL, and RADL pictures.

- To add text to the empty clause on order of VCL NAL units and association to coded pictures, such that decoding order of VCL NAL units within a coded picture is clearly specified.

- To change the description of the general decoding process, such that the term "the bitstream" in the specification does not refer to just one CVS.

# Proposals

## Some constraints on IRAP, RASL, and RADL pictures

As part of the semantics, add the following constraints, which are technically identical as in HEVC for IRAP, RASL, and RADL pictures:

Each picture, other than the first picture in the bitstream in decoding order, is considered to be associated with the previous IRAP picture in decoding order.

When a picture is a leading picture of an IRAP picture, it shall be a RADL or RASL picture.

When a picture is a trailing picture of an IRAP picture, it shall not be a RADL or RASL picture.

When a picture is a leading picture of an IRAP picture, it shall precede, in decoding order, all trailing pictures that are associated with the same IRAP picture.

No RASL pictures shall be present in the bitstream that are associated with an IDR picture.

No RADL pictures shall be present in the bitstream that are associated with with an IDR picture having nal_unit_type equal to IDR_N_LP.

NOTE – It is possible to perform random access at the position of an IRAP access unit by discarding all access units before the IRAP access unit (and to correctly decode the IRAP picture and all the subsequent non-RASL pictures in decoding order), provided each parameter set is available (either in the bitstream or by external means not specified in this Specification) when it needs to be activated.

Any picture that precedes an IRAP picture in decoding order shall precede the IRAP picture in output order and shall precede any RADL picture associated with the IRAP picture in output order.

Any RASL picture associated with a CRA picture shall precede any RADL picture associated with the CRA picture in output order.

Any RASL picture associated with a CRA picture shall follow, in output order, any IRAP picture that precedes the CRA picture in decoding order.

## Order of VCL NAL units and association to coded pictures

This clause is currently empty in the latest VVC WD. The following text is proposed to added:

##### 7.4.2.4.5 Order of VCL NAL units and association to coded pictures

This clause specifies the order of VCL NAL units and association to coded pictures.

Each VCL NAL unit is part of a coded picture.

The order of the VCL NAL units within a coded picture is constrained as follows:

–	Let tileGroupAddrA and tileGroupAddrB be the tile_group_address values of any two tile group NAL units A and B within the same coded picture. When tileGroupAddrA is less than tileGroupAddrB, A shall precede B.

## On the general decoding process

Note that this issue also exists in the latest HEVC specification.

The general decoding process is specified in a CVS by CVS manner, because the sub-bitstream extraction process relies on the SPS syntax element sps_max_sub_layers_minus1, while the active SPS can change across CVSs.

For each CVS of the input bitstream, the sub-bitstream extraction process is applied first with the output being BitstreamToDecode. Therefore, BitstreamToDecode contains one CVS. After that, the decoding process for a coded picture specified in clause 8.1.3 is repeatedly invoked for each coded picture in BitstreamToDecode.

The following is stated in clause 8.1.3:

The decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause 7. When interpreting the semantics of each syntax element in each NAL unit, the term "the bitstream" (or part thereof, e.g., a CVS of the bitstream) refers to BitstreamToDecode (or part thereof).

From the yellow-highlighted wording, it is clear that the term "the bitstream" mentioned above was intended to mean something that can contain more than one CVS.

Furthermore, there are many places related to "interpreting the semantics of each syntax element in each NAL unit" that have the wording "the first picture in the bitstream", particularly when used to determine whether a CRA picture starts a CVS. In my understanding, in those places, the intent was that the term "the bitstream" mentioned above was intended to mean something that can contain more than one CVS.

Lastly, at the beginning of the general decoding process for the input bitstream, strictly speaking, determining the CVSs in the input bitstream would not be clear before determination of the variable NoRaslOutputFlag for each IRAP picture, which currently is only part of the decoding process of a CVS.

Therefore, this seems to be a bug in the text, in both the latest VVC WD and the latest HEVC spec, and it would clearer and cleaner to rearrange the description a bit.

It is therefore proposed to change the description of the general decoding process as follows:

- Firstly the CVSs in the input bitstream are identified.

- Sub-bitstream extraction is applied for each CVS.

- The extracted results of all the CVSs are concatenated to BitstreamToDecode.

- BitstreamToDecode is decoded CVS by CVS, and each CVS picture by picture.

The text changes are provided below in Subection 2 of this document.

# Spec text change for the descriptiong of the general decoding process

## 8.1 General decoding process

    - General

Input to this process is a bitstream. Output of this process is a list of decoded pictures.

The decoding process is specified such that all decoders that conform to a specified profile and level will produce numerically identical cropped decoded output pictures when invoking the decoding process associated with that profile for a bitstream conforming to that profile and level. Any decoding process that produces identical cropped decoded output pictures to those produced by the process described herein (with the correct output order or output timing, as specified) conforms to the decoding process requirements of this Specification.

For each IRAP picture in the bitstream, the following applies:

–	If the picture is an IDR picture, the first picture in the bitstream in decoding order, or the first picture that follows an end of sequence NAL unit in decoding order, the variable NoIncorrectPicOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsCvsStartFlag to a value for the picture, HandleCraAsCvsStartFlag is set equal to the value provided by the external means and NoIncorrectPicOutputFlag is set equal to HandleCraAsCvsStartFlag.

–	Otherwise, HandleCraAsCvsStartFlag and NoIncorrectPicOutputFlag are both set equal to 0.

NOTE – – The above operaiton is needed for identification of the CVSs in the bitstream.

For each CVS in the bitstream, the variable HighestTid, which identifies the highest temporal sub-layer to be decoded, is specified as follows:

–	If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

–	Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1.

For each CVS in the bitstrteam, the sub-bitstream extraction process as specified in clause 10 is applied with the CVS and HighestTid as inputs, and the output is assigned to a bitstream referred to as CvsToDecode. After that, the instances of CvsToDecode of all the CVSs are concatenated, in decoding order, and the result is assigned to the bitsream BitstreamToDecode.

Clause 8.1.2 is repeatedly invoked for each CVS in BitstreamToDecode in decoding order.

    - CVS decoding process

Input to this process is a CVS CvsToDecode. Output of this process is a list of decoded pictures.

Clause 8.1.3 is repeatedly invoked for each coded picture in BitstreamToDecode in decoding order.

    - Decoding process for a coded picture

The decoding processes specified in this clause apply to each coded picture, referred to as the current picture and denoted by the variable CurrPic, in BitstreamToDecode.

Depending on the value of chroma_format_idc, the number of sample arrays of the current picture is as follows:

–	If chroma_format_idc is equal to 0, the current picture consists of 1 sample array SL.

–	Otherwise (chroma_format_idc is not equal to 0), the current picture consists of 3 sample arrays SL, SCb, SCr.

The decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause 7. When interpreting the semantics of each syntax element in each NAL unit, and in the remaining parts of clause 8, the term "the bitstream" (or part thereof, e.g., a CVS of the bitstream) refers to BitstreamToDecode (or part thereof).

Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

...

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. does not have any current or pending patent rights relating to the technology described in this contribution (to the extent of the personal awareness of the author).