| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0121-v3 |
| --- | --- |

| Title: | Non-CE6: MTS Simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to 1) clean-up transform kernel derivation based on symmetric/anti-symmetric property between DST7 and DCT8 as an editorial change (proposal#1), and 2) replace DCT8 with FlipDST7 which has a symmetric property with DST7 as a normative change (proposal#2); Implement forward transform of FlipDST7 by DST7 with input signal reorder; Inverse transform by DST7-1 with output signal reorder. Note “reorder” can be done by “memory addressing change”.

It is asserted that proposal#1 can simplify WD by roughly 1.5-page reduction with no coding loss.

It is asserted that proposal#2 can further simplify WD by roughly 2-page reduction and reduce implementation costs with on average 0.09 %, 0.06 %, and z.zz% of bdrate changes over VTM-4.0 CTC in Class A1/A2/B/C/E for AI, RA and LB, respectively; 0.03%, 0.05%, and 0.00% of bdrate changes in Class D for AI, RA and LB, respectively; 0.05%, 0.03% and -0.16 % of bdrate changes in Class F for AI, RA and LB, respectively. Also, negligible runtime changes are observed under CTC condition.

It is recommended to adopt proposal#1 and/or proposal#2 into the next VTM and WD.

In v3, simulation results were updated.

Introduction

In VVC WD [1], transform kernels of DCT2 and associated transform process are defined using recursive property and symmetric/anti-symmetric relations of DCT2 as below:

<Recursive relation>

y[i]= j=0nonZeroS-1transMatrix[i][j*26-Log2(nTbS) ]*x[j]  with i = 0..nTbS − 1	(8-1034)

<Symmetric/Anti-symmetric relation>

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..63		(8-1036)

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..63	(8-1038)

transMatrix[ m ][ n ] = ( n & 1 ? −1 : 1 ) * transMatrixCol16to31[47 −m ][ n ]		(8-1040)
		 with m =32..47, n = 0..63

transMatrix[ m ][ n ] = ( n & 1 ? −1 : 1 ) * transMatrixCol0to15[63 −m ][ n ]		(8-1041)
		with m =48..63, n = 0..63

On the other hand, transform kernels of DST7 and DCT8 are explicitly defined in VVC/VTM[1][2] without using any relation.

Proposal

Propsoal#1: DCT8 derivation from DST-7 (Editorial Change)

Practically, there is a symmetric/anti-symmetric relation between DST7 and DCT8 as below (Note detail proof is described in Appendix):

TDCT8k,l=-1k TDST7k, N-1-l,    -(A1)

, where Tk,l represents the l-th element in the k-th basis vector (k,l=0,…,N-1).

Equation (A1) indicates that even basis vector TDCT8k,: (k%2==0) is derived from TDST7k,: by horizontally flipping (symmetric relation); odd basis vector TDCT8k,: (k%2==1) is derived from TDST7k,: by horizontally flipping and sign inversion (*(-1)) (anti-symmetric relation).

For example, 4-pt transform kernel of DST7 (called transMatrixDST7B4) and DCT8 (called transMatrixDCT8B4) are defined by (8-1042) and (8-1049), respectively [1].

<4-pt DST-7>

transMatrixDST7B4[ m ][ n ] = 	(8-1042)

{

{  29   55   74   84 }

{  74   74    0  −74 }

{  84  −29  −74   55 }

{  55  −84   74  −29 }

},

<4-pt DCT-8>

transMatrixDCT8B4[ m ][ n ] = 	(8-1049)

{

{  84   74   55   29 }

{  74    0  −74  −74 }

{  55  −74  −29   84 }

{  29  −74   84  −55 }

},

Using equation (A1), transform kernel of DCT8 is briefly defined by DST7’s as below.

transMatrix_DCT8B4[ m ][ n ] = (n & 1 ? −1 : 1) * transMatrixDST7B4 [3 −m ][ n ] 
with m = 0..3, n = 0.. 3							- (8-1049’)

In the same way, 8/16/32-pt transform kernel of DCT8 is defined by DST7’s.

Proposed specification changes are described in section 7. Also, text is available in JVET-N0121_SpecChangesForProposal1.docx.

Proposal#2: Replacing DCT8 with FlipDST7 (Normative Change)

Since proposal#1 is only an editorial change, it seems less attractive in terms of software/hardware implementation. To further simplify implementation cost, proposal#2 is 1) to replace DCT8 with FlipDST7, of which transform kernel is obtained from equation (A1) by removing sign inversion term -1k.

TFlipDST7k,l= TDST7k, N-1-l    -(A2)

, and 2) realize forward transform of FlipDST7 by DST7 with input signal reorder (equation (B1)), and inverse transform of FlipDST7 by DST7-1 with output signal reorder (equation (B2)). Hence, we need no more transform kernels of DCT8 or FlipDST7.

Note that FlipDST7 or similar concept approach is found in JVET-K0167[3], JVET-J0018[4] and JVET-J0028/JVET-K0121[5][6].

Forward Transform of FlipDST-7

Forward Transform of FlipDST-7 is defined as below:

Xt=TFlipDST7xt=TDST7Jxt=TDST7Jxt,    -(B1)

, where x is a 1xN input vector, X is 1xN output vector and J is a NxN cross-identity matrix.

Thus, Forward Transform of FlipDST-7 can be implemented by “input signal reorder” plus “DST-7”

Inverse Transform of FlipDST-7

Inverse Transform of FlipDST-7 is defined as below:

xt=TFlipDST7tXt=TDST7JtXt=JTDST7tXt,    -(B2)

, where x is a 1xN input vector, X is 1xN output vector and J is a NxN cross-identity matrix.

Thus, inverse Transform of FlipDST-7 can be implemented by “(DST-7)-1” plus “output signal reorder”

Note that “reorder” can be done by “memory addressing change”

Proposed specification changes are described in section 8. Also, text is available in JVET-N0121_SpecChangesForProposal2.docx.

Simulation Results

Following tests were carried out by using the VTM-4.0 [2] software under the common test conditions (CTC) [7]:

- Test#1: Proposal2 under CTC

Table 3-1 shows the summary of coding performances for Test#1.

Detail results are available in the attached spreadsheets

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.12% | 0.16% | 98% | 99% |
| Class A2 | 0.08% | 0.17% | 0.05% | 100% | 100% |
| Class B | 0.12% | 0.16% | 0.13% | 100% | 100% |
| Class C | 0.04% | 0.04% | 0.01% | 99% | 99% |
| Class E | 0.07% | 0.13% | 0.06% | 100% | 100% |
| Overall | 0.09% | 0.12% | 0.08% | 100% | 99% |
| Class D | 0.03% | 0.03% | -0.09% | 100% | 97% |
| Class F | 0.05% | 0.06% | 0.12% | 101% | 100% |
|  | Random Access |  |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.10% | 0.04% | 100% | 99% |
| Class A2 | 0.05% | -0.04% | 0.10% | 99% | 100% |
| Class B | 0.08% | 0.03% | -0.16% | 100% | 100% |
| Class C | 0.02% | 0.04% | -0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | -0.01% | -0.03% | 100% | 99% |
| Class D | 0.05% | -0.09% | -0.24% | 100% | 99% |
| Class F | 0.03% | -0.07% | -0.09% | 101% | 100% |
|  | Low Delay B |  |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.02% | 0.30% | 0.00% | 100% | 98% |
| Class E | 0.07% | 1.01% | -0.06% | 101% | 99% |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.53% | 0.09% | 101% | 99% |
| Class F | -0.16% | -0.44% | -0.53% | 100% | 99% |

Conclusion

This contribution proposes:

- 1) Clean-up transform kernel derivation based on symmetric/anti-symmetric property between DST7 and DCT8 as an editorial change (proposal#1)

- 2) Replace DCT8 with FlipDST7 which has a symmetric property with DST7 as a normative change (proposal#2); Implement forward transform of FlipDST7 by DST7 with input signal reorder; Inverse transform by DST7-1 with output signal reorder.

It is recommended to adopt one of proposals into the next VTM and WD.

References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” JVET-M1001, Jan. 2019

[2] VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

[3] S.-C. Lim, J. Kang, H. Lee, J. Lee, S. Cho, H. Y. Kim, N.-U. Kim, Y.-L. Lee, D.-Y. Kim, W. J. Jeong, “CE6: DST-VII with residual flipping (Test 1.4),” JVET-K0167, Jul. 2018

[4] C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei, “Description of SDR video coding technology proposal by MediaTek”, JVET-J0018, 10-20 Apr. 2018

[5] T. Suzuki, M. Ikeda, K. Sharman, “Description of SDR and HDR video coding technology proposal by Sony,” JVET-J0028, Apr. 2018

[6] T. Tsukuba, M. Ikeda, T. Suzuki, “CE6: Transform Matrix Replacement (CE6.1.8.1),” JVET-K0123, Jul. 2018

[7] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Oct. 2019

[8] B. Bross, J. Chen, S. Liu, “Algorithm description for Versatile Video Coding and Test Model 4 (VTM 4), ” JVET-M1002, Jan. 2019

Appendix: Proof of derivation of DCT8 from DST7

Transform kernels of DST7 and DCT8 are defined [8] as below:

TDST7k,l=42N+1sin2k+1(2l+2) π4N+2=βN sin2l+2k+12π2N+1,      -(A2)

TDCT8k,l=42N+1sin2k+1(2l+2) π4N+2=βN sin2l+2k+12π2N+1,    -(A3)

βN =42N+1 and Tk,l represents the l-th element in the k-th basis vector (k,l=0,…,N-1).

TDCT8k, l=βN cosπ2l+1(k+12) 2N+1      -(A3)

By replacing l with l=N-1-l’ in (A2),

⇔TDCT8k, l=βNcos2N-l'-1+1k+12π2N+1

⇔TDCT8k, l=βNcos2N+1-2l'+2k+12π2N+1

By using product and sum formulas: cosα-β=cosαcosβ+sinαsinβ

⇔TDCT8k, l=βNcosk+12πcos2l'+2k+12π2N+1 +sink+12πsin2l'+2k+12π2N+1

By taking account of sink+12π=-1k and cosk+12π=0,

⇔TDCT8k, l=βN-1ksin2l'+2k+12π2N+1=-1kTDST7k, l'

⇔TDCT8k, l=-1kTDST7k, N-1-l    -(A3')

Proposed Text Change for Proposal#1

Proposed changes are highlighted in yellow.

- 8.7.4.3 Transformation matrix derivation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of scaled transform coefficients,

- the transformation kernel type trType.

Output of this process is the transformation matrix transMatrix.

The transformation matrix transMatrix is derived based on trType and nTbs as follows:

- If trType is equal to 0, the following applies:

… skipped …

- Otherwise, if trType is equal to 1 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = transMatrix_DST7B4[ m ][ n ] with m = 0..3, n = 0..3	(8-1042)

transMatrix_DST7B4 =	(8-X1)

{

{  29   55   74   84 }

{  74   74    0  −74 }

{  84  −29  −74   55 }

{  55  −84   74  −29 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] = transMatrixDST7B8[ m ][ n ] with m = 0..7, n = 0..7	(8-1043)

transMatrixDST7B8 = 	(8-X2)

{

{  17   32   46   60   71   78   85   86 }

{  46   78   86   71   32  −17  −60  −85 }

{  71   85   32  −46  −86  −60   17   78 }

{  85   46  −60  −78   17   86   32  −71 }

{  86  −17  −85   32   78  −46  −71   60 }

{  78  −71  −17   85  −60  −32   86  −46 }

{  60  −86   71  −17  −46   85  −78   32 }

{  32  −60   78  −86   85  −71   46  −17 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] = transMatrixDST7B8[ m ][ n ] with m = 0..15, n = 0..15	(8-1044)

transMatrixDST7B8 = 	(8-X3)

{

{   8   17   25   33   40   48   55   62   68   73   77   81   85   87   88   88 }

{  25   48   68   81   88   88   81   68   48   25    0  −25  −48  −68  −81  −88 }

{  40   73   88   85   62   25  −17  −55  −81  −88  −77  −48   −8   33   68   87 }

{  55   87   81   40  −17  −68  −88  −73  −25   33   77   88   62    8  −48  −85 }

{  68   88   48  −25  −81  −81  −25   48   88   68    0  −68  −88  −48   25   81 }

{  77   77    0  −77  −77    0   77   77    0  −77  −77    0   77   77    0  −77 }

{  85   55  −48  −87   −8   81   62  −40  −88  −17   77   68  −33  −88  −25   73 }

{  88   25  −81  −48   68   68  −48  −81   25   88    0  −88  −25   81   48  −68 }

{  88   −8  −88   17   87  −25  −85   33   81  −40  −77   48   73  −55  −68   62 }

{  87  −40  −68   73   33  −88    8   85  −48  −62   77   25  −88   17   81  −55 }

{  81  −68  −25   88  −48  −48   88  −25  −68   81    0  −81   68   25  −88   48 }

{  73  −85   25   55  −88   48   33  −87   68    8  −77   81  −17  −62   88  −40 }

{  62  −88   68   −8  −55   88  −73   17   48  −87   77  −25  −40   85  −81   33 }

{  48  −81   88  −68   25   25  −68   88  −81   48    0  −48   81  −88   68  −25 }

{  33  −62   81  −88   85  −68   40   −8  −25   55  −77   88  −87   73  −48   17 }

{  17  −33   48  −62   73  −81   87  −88   88  −85   77  −68   55  −40   25   −8 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixDST7B32Col0to15[ m ][ n ] with m = 0..15, n = 0..15	(8-1045)

transMatrixDST7B32Col0to15 =	(8-1046)

{

{   4    9   13   17   21   26   30   34   38   42   46   50   53   56   60   63 }

{  13   26   38   50   60   68   77   82   86   89   90   88   85   80   74   66 }

{  21   42   60   74   84   89   89   84   74   60   42   21    0  −21  −42  −60 }

{  30   56   77   87   89   80   63   38    9  −21  −50  −72  −85  −90  −84  −68 }

{  38   68   86   88   74   46    9  −30  −63  −84  −90  −78  −53  −17   21   56 }

{  46   78   90   77   42   −4  −50  −80  −90  −74  −38    9   53   82   89   72 }

{  53   85   85   53    0  −53  −85  −85  −53    0   53   85   85   53    0  −53 }

{  60   89   74   21  −42  −84  −84  −42   21   74   89   60    0  −60  −89  −74 }

{  66   90   56  −13  −74  −87  −46   26   80   84   34  −38  −85  −78  −21   50 }

{  72   86   34  −46  −89  −63   13   78   82   21  −56  −90  −53   26   84   77 }

{  77   80    9  −72  −84  −17   66   86   26  −60  −88  −34   53   90   42  −46 }

{  80   72  −17  −86  −60   34   90   46  −50  −89  −30   63   85   13  −74  −78 }

{  84   60  −42  −89  −21   74   74  −21  −89  −42   60   84    0  −84  −60   42 }

{  86   46  −63  −78   21   90   26  −77  −66   42   87    4  −85  −50   60   80 }

{  88   30  −78  −56   60   77  −34  −87    4   89   26  −80  −53   63   74  −38 }

{  90   13  −87  −26   84   38  −78  −50   72   60  −63  −68   53   77  −42  −82 }

},

transMatrix[ m ][ n ] = transMatrixDST7B32Col16to31[ m − 16 ][ n ] with m = 16..31, n = 0..15	(8-1047)

transMatrixDST7B32Col16to31 =	(8-1048)

{

{  66   68   72   74   77   78   80   82   84   85   86   87   88   89   90   90 }

{  56   46   34   21    9   −4  −17  −30  −42  −53  −63  −72  −78  −84  −87  −90 }

{ −74  −84  −89  −89  −84  −74  −60  −42  −21    0   21   42   60   74   84   89 }

{ −46  −17   13   42   66   82   90   86   74   53   26   −4  −34  −60  −78  −88 }

{  80   90   82   60   26  −13  −50  −77  −89  −85  −66  −34    4   42   72   87 }

{  34  −13  −56  −84  −88  −68  −30   17   60   85   87   66   26  −21  −63  −86 }

{ −85  −85  −53    0   53   85   85   53    0  −53  −85  −85  −53    0   53   85 }

{ −21   42   84   84   42  −21  −74  −89  −60    0   60   89   74   21  −42  −84 }

{  88   72    9  −60  −90  −63    4   68   89   53  −17  −77  −86  −42   30   82 }

{   9  −66  −88  −42   38   87   68   −4  −74  −85  −30   50   90   60  −17  −80 }

{ −90  −50   38   89   56  −30  −87  −63   21   85   68  −13  −82  −74    4   78 }

{   4   82   68  −21  −87  −56   38   90   42  −53  −88  −26   66   84    9  −77 }

{  89   21  −74  −74   21   89   42  −60  −84    0   84   60  −42  −89  −21   74 }

{ −17  −90  −30   74   68  −38  −88   −9   84   53  −56  −82   13   89   34  −72 }

{ −86    9   90   21  −82  −50   66   72  −42  −85   13   90   17  −84  −46   68 }

{  30   86  −17  −89    4   90    9  −88  −21   85   34  −80  −46   74   56  −66 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = ( n & 1 ? −1 : 1 )  * transMatrixDST7B4[ 3 - m ][ n ] with m = 0..3, n = 0..3	(8-1049)

, where transMatrixDST7B4 is defined in (8-X1).

{

{  84   74   55   29 }

{  74    0  −74  −74 }

{  55  −74  −29   84 }

{  29  −74   84  −55 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] =  ( n & 1 ? −1 : 1 )   * transMatrixDST7B8[ 7 - m ][ n ] with m = 0..7, n = 0..7	(8-1050)

, where transMatrixDST7B8 is defined in (8-X2).

{

{  86   85   78   71   60   46   32   17 }

{  85   60   17  −32  −71  −86  −78  −46 }

{  78   17  −60  −86  −46   32   85   71 }

{  71  −32  −86  −17   78   60  −46  −85 }

{  60  −71  −46   78   32  −85  −17   86 }

{  46  −86   32   60  −85   17   71  −78 }

{  32  −78   85  −46  −17   71  −86   60 }

{  17  −46   71  −85   86  −78   60  −32 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] =  ( n & 1 ? −1 : 1 )  * transMatrixDST7B16[ 15 - m ][ n ] with m = 0..15, n = 0..15	(8-1051)

, where transMatrixDST7B16 is defined in (8-X3).

{

{  88   88   87   85   81   77   73   68   62   55   48   40   33   25   17    8 }

{  88   81   68   48   25    0  −25  −48  −68  −81  −88  −88  −81  −68  −48  −25 }

{  87   68   33   −8  −48  −77  −88  −81  −55  −17   25   62   85   88   73   40 }

{  85   48   −8  −62  −88  −77  −33   25   73   88   68   17  −40  −81  −87  −55 }

{  81   25  −48  −88  −68    0   68   88   48  −25  −81  −81  −25   48   88   68 }

{  77    0  −77  −77    0   77   77    0  −77  −77    0   77   77    0  −77  −77 }

{  73  −25  −88  −33   68   77  −17  −88  −40   62   81   −8  −87  −48   55   85 }

{  68  −48  −81   25   88    0  −88  −25   81   48  −68  −68   48   81  −25  −88 }

{  62  −68  −55   73   48  −77  −40   81   33  −85  −25   87   17  −88   −8   88 }

{  55  −81  −17   88  −25  −77   62   48  −85   −8   88  −33  −73   68   40  −87 }

{  48  −88   25   68  −81    0   81  −68  −25   88  −48  −48   88  −25  −68   81 }

{  40  −88   62   17  −81   77   −8  −68   87  −33  −48   88  −55  −25   85  −73 }

{  33  −81   85  −40  −25   77  −87   48   17  −73   88  −55   −8   68  −88   62 }

{  25  −68   88  −81   48    0  −48   81  −88   68  −25  −25   68  −88   81  −48 }

{  17  −48   73  −87   88  −77   55  −25   −8   40  −68   85  −88   81  −62   33 }

{   8  −25   40  −55   68  −77   85  −88   88  −87   81  −73   62  −48   33  −17 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = ( n & 1 ? −1 : 1 ) *transMatrixDST7B32Col16to31 Col0to15[ 15 - m ][ n ] with m = 0..15, n = 0..15	(8-1052)

, where transMatrix_DST7B32Col16to31 is defined in (8-X5).

transMatrixCol0to15 =	(8-1053)

{

{  90   90   89   88   87   86   85   84   82   80   78   77   74   72   68   66 }

{  90   87   84   78   72   63   53   42   30   17    4   −9  −21  −34  −46  −56 }

{  89   84   74   60   42   21    0  −21  −42  −60  −74  −84  −89  −89  −84  −74 }

{  88   78   60   34    4  −26  −53  −74  −86  −90  −82  −66  −42  −13   17   46 }

{  87   72   42    4  −34  −66  −85  −89  −77  −50  −13   26   60   82   90   80 }

{  86   63   21  −26  −66  −87  −85  −60  −17   30   68   88   84   56   13  −34 }

{  85   53    0  −53  −85  −85  −53    0   53   85   85   53    0  −53  −85  −85 }

{  84   42  −21  −74  −89  −60    0   60   89   74   21  −42  −84  −84  −42   21 }

{  82   30  −42  −86  −77  −17   53   89   68    4  −63  −90  −60    9   72   88 }

{  80   17  −60  −90  −50   30   85   74    4  −68  −87  −38   42   88   66   −9 }

{  78    4  −74  −82  −13   68   85   21  −63  −87  −30   56   89   38  −50  −90 }

{  77   −9  −84  −66   26   88   53  −42  −90  −38   56   87   21  −68  −82   −4 }

{  74  −21  −89  −42   60   84    0  −84  −60   42   89   21  −74  −74   21   89 }

{  72  −34  −89  −13   82   56  −53  −84    9   88   38  −68  −74   30   90   17 }

{  68  −46  −84   17   90   13  −85  −42   72   66  −50  −82   21   90    9  −86 }

{  66  −56  −74   46   80  −34  −85   21   88   −9  −90   −4   89   17  −86  −30 }

},

transMatrix[ m ][ n ] =  ( n & 1 ? −1 : 1 ) *transMatrixDST7B32Col0to15Col16to31[ 47 − m − 16 ][ n ] with m = 16..31, n = 0..15	(8-1054)

, where transMatrixDST7B32Col0to15 is defined in (8-X4).

transMatrixCol16to31 =	(8-1055)

{

{  63   60   56   53   50   46   42   38   34   30   26   21   17   13    9    4 }

{ −66  −74  −80  −85  −88  −90  −89  −86  −82  −77  −68  −60  −50  −38  −26  −13 }

{ −60  −42  −21    0   21   42   60   74   84   89   89   84   74   60   42   21 }

{  68   84   90   85   72   50   21   −9  −38  −63  −80  −89  −87  −77  −56  −30 }

{  56   21  −17  −53  −78  −90  −84  −63  −30    9   46   74   88   86   68   38 }

{ −72  −89  −82  −53   −9   38   74   90   80   50    4  −42  −77  −90  −78  −46 }

{ −53    0   53   85   85   53    0  −53  −85  −85  −53    0   53   85   85   53 }

{  74   89   60    0  −60  −89  −74  −21   42   84   84   42  −21  −74  −89  −60 }

{  50  −21  −78  −85  −38   34   84   80   26  −46  −87  −74  −13   56   90   66 }

{ −77  −84  −26   53   90   56  −21  −82  −78  −13   63   89   46  −34  −86  −72 }

{ −46   42   90   53  −34  −88  −60   26   86   66  −17  −84  −72    9   80   77 }

{  78   74  −13  −85  −63   30   89   50  −46  −90  −34   60   86   17  −72  −80 }

{  42  −60  −84    0   84   60  −42  −89  −21   74   74  −21  −89  −42   60   84 }

{ −80  −60   50   85   −4  −87  −42   66   77  −26  −90  −21   78   63  −46  −86 }

{ −38   74   63  −53  −80   26   89    4  −87  −34   77   60  −56  −78   30   88 }

{  82   42  −77  −53   68   63  −60  −72   50   78  −38  −84   26   87  −13  −90 }

},

Proposed Text Change for Proposal#2

Proposed changes are highlighted in yellow.

- 8.7.4 Transformation process for scaled transform coefficients

- 8.7.4.1 General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

… skipped…

The (nTbW)x(nTbH) array r of residual samples is derived as follows:

- When nTbH is greater than 1, each (vertical) column of scaled transform coefficients d[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nonZeroH − 1 is transformed to e[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.7.4.2 for each column x = 0..nonZeroW − 1 with the height of the transform block nTbH, the non-zero height of the scaled transform coefficients nonZeroH, the list d[ x ][ y ] with y = 0..nonZeroH − 1, and the transform type variable trType set equal to (trTypeVer&1) and the variable bFlip set equal to (trTypeVer&2)>>1 as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbH − 1.

- When nTbH and nTbW are both greater than 1, the intermediate sample values g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 are derived as follows:

g[ x ][ y ] = Clip3( CoeffMin, CoeffMax, ( e[ x ][ y ] + 64 )  >>  7 )	(8-1033)

- When nTbW is greater than 1, each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 is transformed to r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.7.4.2 for each row y = 0..nTbH − 1 with the width of the transform block nTbW, the non-zero width of the resulting array g[ x ][ y ] nonZeroW, the list g[ x ][ y ] with x = 0..nonZeroW − 1, and the transform type variable trType set equal to (trTypeHor&1) and the variable bFlip set equal to (trTypeHor&2)>>1 as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbW − 1.

… skipped …

- 8.7.4.2 Transformation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of transformed samples,

- a variable nonZeroS specifying the horizontal sample size of non-zero scaled transform coefficients,

- a list of scaled transform coefficients x[ j ] with j = 0..nonZeroS − 1,

- a transform kernel type variable trType,

- a variable bFlip specifying the direction of output transformed samples.

Output of this process is the list of transformed samples y[ i ] with i = 0..nTbS − 1.

The transformation matrix derivation process as specified in clause 8.7.4.3 in invoked with the transform size nTbS and the transform kernel Type trType as inputs, and the transformation maxtrix transMatrix as output.

Depending on the value of trType, the following applies:, the list of transformed samples y[ i ] with i = 0..nTbS − 1 is derived as follows:

- If trType is equal to 0, the following transform matrix multiplication applies:

y[i]= j=0nonZeroS-1transMatrix[i][j*26-Log2(nTbS) ]*x[j]  with i = 0..nTbS − 1	(8-1034)

- Otherwise (trType is equal to 1), the following transform matrix multiplication applies:

y[bFlip ? (nTbS-1-i) : i]= j=0nonZeroS-1transMatrix[i][j]*x[j]  with i = 0..nTbS − 1	(8-1035)

- 8.7.4.3 Transformation matrix derivation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of scaled transform coefficients,

- the transformation kernel type trType.

Output of this process is the transformation matrix transMatrix.

The transformation matrix transMatrix is derived based on trType and nTbs as follows:

- If trType is equal to 0, the following applies:

… skipped …

- Otherwise, if trType is equal to 1 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-1042)

{

{  29   55   74   84 }

{  74   74    0  −74 }

{  84  −29  −74   55 }

{  55  −84   74  −29 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] = 	(8-1043)

{

{  17   32   46   60   71   78   85   86 }

{  46   78   86   71   32  −17  −60  −85 }

{  71   85   32  −46  −86  −60   17   78 }

{  85   46  −60  −78   17   86   32  −71 }

{  86  −17  −85   32   78  −46  −71   60 }

{  78  −71  −17   85  −60  −32   86  −46 }

{  60  −86   71  −17  −46   85  −78   32 }

{  32  −60   78  −86   85  −71   46  −17 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] = 	(8-1044)

{

{   8   17   25   33   40   48   55   62   68   73   77   81   85   87   88   88 }

{  25   48   68   81   88   88   81   68   48   25    0  −25  −48  −68  −81  −88 }

{  40   73   88   85   62   25  −17  −55  −81  −88  −77  −48   −8   33   68   87 }

{  55   87   81   40  −17  −68  −88  −73  −25   33   77   88   62    8  −48  −85 }

{  68   88   48  −25  −81  −81  −25   48   88   68    0  −68  −88  −48   25   81 }

{  77   77    0  −77  −77    0   77   77    0  −77  −77    0   77   77    0  −77 }

{  85   55  −48  −87   −8   81   62  −40  −88  −17   77   68  −33  −88  −25   73 }

{  88   25  −81  −48   68   68  −48  −81   25   88    0  −88  −25   81   48  −68 }

{  88   −8  −88   17   87  −25  −85   33   81  −40  −77   48   73  −55  −68   62 }

{  87  −40  −68   73   33  −88    8   85  −48  −62   77   25  −88   17   81  −55 }

{  81  −68  −25   88  −48  −48   88  −25  −68   81    0  −81   68   25  −88   48 }

{  73  −85   25   55  −88   48   33  −87   68    8  −77   81  −17  −62   88  −40 }

{  62  −88   68   −8  −55   88  −73   17   48  −87   77  −25  −40   85  −81   33 }

{  48  −81   88  −68   25   25  −68   88  −81   48    0  −48   81  −88   68  −25 }

{  33  −62   81  −88   85  −68   40   −8  −25   55  −77   88  −87   73  −48   17 }

{  17  −33   48  −62   73  −81   87  −88   88  −85   77  −68   55  −40   25   −8 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15	(8-1045)

transMatrixCol0to15 =	(8-1046)

{

{   4    9   13   17   21   26   30   34   38   42   46   50   53   56   60   63 }

{  13   26   38   50   60   68   77   82   86   89   90   88   85   80   74   66 }

{  21   42   60   74   84   89   89   84   74   60   42   21    0  −21  −42  −60 }

{  30   56   77   87   89   80   63   38    9  −21  −50  −72  −85  −90  −84  −68 }

{  38   68   86   88   74   46    9  −30  −63  −84  −90  −78  −53  −17   21   56 }

{  46   78   90   77   42   −4  −50  −80  −90  −74  −38    9   53   82   89   72 }

{  53   85   85   53    0  −53  −85  −85  −53    0   53   85   85   53    0  −53 }

{  60   89   74   21  −42  −84  −84  −42   21   74   89   60    0  −60  −89  −74 }

{  66   90   56  −13  −74  −87  −46   26   80   84   34  −38  −85  −78  −21   50 }

{  72   86   34  −46  −89  −63   13   78   82   21  −56  −90  −53   26   84   77 }

{  77   80    9  −72  −84  −17   66   86   26  −60  −88  −34   53   90   42  −46 }

{  80   72  −17  −86  −60   34   90   46  −50  −89  −30   63   85   13  −74  −78 }

{  84   60  −42  −89  −21   74   74  −21  −89  −42   60   84    0  −84  −60   42 }

{  86   46  −63  −78   21   90   26  −77  −66   42   87    4  −85  −50   60   80 }

{  88   30  −78  −56   60   77  −34  −87    4   89   26  −80  −53   63   74  −38 }

{  90   13  −87  −26   84   38  −78  −50   72   60  −63  −68   53   77  −42  −82 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..15	(8-1047)

transMatrixCol16to31 =	(8-1048)

{

{  66   68   72   74   77   78   80   82   84   85   86   87   88   89   90   90 }

{  56   46   34   21    9   −4  −17  −30  −42  −53  −63  −72  −78  −84  −87  −90 }

{ −74  −84  −89  −89  −84  −74  −60  −42  −21    0   21   42   60   74   84   89 }

{ −46  −17   13   42   66   82   90   86   74   53   26   −4  −34  −60  −78  −88 }

{  80   90   82   60   26  −13  −50  −77  −89  −85  −66  −34    4   42   72   87 }

{  34  −13  −56  −84  −88  −68  −30   17   60   85   87   66   26  −21  −63  −86 }

{ −85  −85  −53    0   53   85   85   53    0  −53  −85  −85  −53    0   53   85 }

{ −21   42   84   84   42  −21  −74  −89  −60    0   60   89   74   21  −42  −84 }

{  88   72    9  −60  −90  −63    4   68   89   53  −17  −77  −86  −42   30   82 }

{   9  −66  −88  −42   38   87   68   −4  −74  −85  −30   50   90   60  −17  −80 }

{ −90  −50   38   89   56  −30  −87  −63   21   85   68  −13  −82  −74    4   78 }

{   4   82   68  −21  −87  −56   38   90   42  −53  −88  −26   66   84    9  −77 }

{  89   21  −74  −74   21   89   42  −60  −84    0   84   60  −42  −89  −21   74 }

{ −17  −90  −30   74   68  −38  −88   −9   84   53  −56  −82   13   89   34  −72 }

{ −86    9   90   21  −82  −50   66   72  −42  −85   13   90   17  −84  −46   68 }

{  30   86  −17  −89    4   90    9  −88  −21   85   34  −80  −46   74   56  −66 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-1049)

{

{  84   74   55   29 }

{  74    0  −74  −74 }

{  55  −74  −29   84 }

{  29  −74   84  −55 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] = 	(8-1050)

{

{  86   85   78   71   60   46   32   17 }

{  85   60   17  −32  −71  −86  −78  −46 }

{  78   17  −60  −86  −46   32   85   71 }

{  71  −32  −86  −17   78   60  −46  −85 }

{  60  −71  −46   78   32  −85  −17   86 }

{  46  −86   32   60  −85   17   71  −78 }

{  32  −78   85  −46  −17   71  −86   60 }

{  17  −46   71  −85   86  −78   60  −32 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] = 	(8-1051)

{

{  88   88   87   85   81   77   73   68   62   55   48   40   33   25   17    8 }

{  88   81   68   48   25    0  −25  −48  −68  −81  −88  −88  −81  −68  −48  −25 }

{  87   68   33   −8  −48  −77  −88  −81  −55  −17   25   62   85   88   73   40 }

{  85   48   −8  −62  −88  −77  −33   25   73   88   68   17  −40  −81  −87  −55 }

{  81   25  −48  −88  −68    0   68   88   48  −25  −81  −81  −25   48   88   68 }

{  77    0  −77  −77    0   77   77    0  −77  −77    0   77   77    0  −77  −77 }

{  73  −25  −88  −33   68   77  −17  −88  −40   62   81   −8  −87  −48   55   85 }

{  68  −48  −81   25   88    0  −88  −25   81   48  −68  −68   48   81  −25  −88 }

{  62  −68  −55   73   48  −77  −40   81   33  −85  −25   87   17  −88   −8   88 }

{  55  −81  −17   88  −25  −77   62   48  −85   −8   88  −33  −73   68   40  −87 }

{  48  −88   25   68  −81    0   81  −68  −25   88  −48  −48   88  −25  −68   81 }

{  40  −88   62   17  −81   77   −8  −68   87  −33  −48   88  −55  −25   85  −73 }

{  33  −81   85  −40  −25   77  −87   48   17  −73   88  −55   −8   68  −88   62 }

{  25  −68   88  −81   48    0  −48   81  −88   68  −25  −25   68  −88   81  −48 }

{  17  −48   73  −87   88  −77   55  −25   −8   40  −68   85  −88   81  −62   33 }

{   8  −25   40  −55   68  −77   85  −88   88  −87   81  −73   62  −48   33  −17 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15	(8-1052)

transMatrixCol0to15 =	(8-1053)

{

{  90   90   89   88   87   86   85   84   82   80   78   77   74   72   68   66 }

{  90   87   84   78   72   63   53   42   30   17    4   −9  −21  −34  −46  −56 }

{  89   84   74   60   42   21    0  −21  −42  −60  −74  −84  −89  −89  −84  −74 }

{  88   78   60   34    4  −26  −53  −74  −86  −90  −82  −66  −42  −13   17   46 }

{  87   72   42    4  −34  −66  −85  −89  −77  −50  −13   26   60   82   90   80 }

{  86   63   21  −26  −66  −87  −85  −60  −17   30   68   88   84   56   13  −34 }

{  85   53    0  −53  −85  −85  −53    0   53   85   85   53    0  −53  −85  −85 }

{  84   42  −21  −74  −89  −60    0   60   89   74   21  −42  −84  −84  −42   21 }

{  82   30  −42  −86  −77  −17   53   89   68    4  −63  −90  −60    9   72   88 }

{  80   17  −60  −90  −50   30   85   74    4  −68  −87  −38   42   88   66   −9 }

{  78    4  −74  −82  −13   68   85   21  −63  −87  −30   56   89   38  −50  −90 }

{  77   −9  −84  −66   26   88   53  −42  −90  −38   56   87   21  −68  −82   −4 }

{  74  −21  −89  −42   60   84    0  −84  −60   42   89   21  −74  −74   21   89 }

{  72  −34  −89  −13   82   56  −53  −84    9   88   38  −68  −74   30   90   17 }

{  68  −46  −84   17   90   13  −85  −42   72   66  −50  −82   21   90    9  −86 }

{  66  −56  −74   46   80  −34  −85   21   88   −9  −90   −4   89   17  −86  −30 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..15	(8-1054)

transMatrixCol16to31 =	(8-1055)

{

{  63   60   56   53   50   46   42   38   34   30   26   21   17   13    9    4 }

{ −66  −74  −80  −85  −88  −90  −89  −86  −82  −77  −68  −60  −50  −38  −26  −13 }

{ −60  −42  −21    0   21   42   60   74   84   89   89   84   74   60   42   21 }

{  68   84   90   85   72   50   21   −9  −38  −63  −80  −89  −87  −77  −56  −30 }

{  56   21  −17  −53  −78  −90  −84  −63  −30    9   46   74   88   86   68   38 }

{ −72  −89  −82  −53   −9   38   74   90   80   50    4  −42  −77  −90  −78  −46 }

{ −53    0   53   85   85   53    0  −53  −85  −85  −53    0   53   85   85   53 }

{  74   89   60    0  −60  −89  −74  −21   42   84   84   42  −21  −74  −89  −60 }

{  50  −21  −78  −85  −38   34   84   80   26  −46  −87  −74  −13   56   90   66 }

{ −77  −84  −26   53   90   56  −21  −82  −78  −13   63   89   46  −34  −86  −72 }

{ −46   42   90   53  −34  −88  −60   26   86   66  −17  −84  −72    9   80   77 }

{  78   74  −13  −85  −63   30   89   50  −46  −90  −34   60   86   17  −72  −80 }

{  42  −60  −84    0   84   60  −42  −89  −21   74   74  −21  −89  −42   60   84 }

{ −80  −60   50   85   −4  −87  −42   66   77  −26  −90  −21   78   63  −46  −86 }

{ −38   74   63  −53  −80   26   89    4  −87  −34   77   60  −56  −78   30   88 }

{  82   42  −77  −53   68   63  −60  −72   50   78  −38  −84   26   87  −13  −90 }

},

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).