| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0123-v1 |
| --- | --- |

| Title: | AHG13/Non-CE6/CE8: Chroma Transform Skip |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report, Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

For AHG13 Tool reporting procedure, this contribution reports additional tool off test results of unified transform type signaling adopt in VTM-4.0/JVET-M0464 to evaluate impacts by all adopt changes including a) unified signalling for transform skip and MTS, b) transform skip size extension, c) no chroma transform skip, and d) Fast transform type selection as non-normative change:

- Test#1 (TSM_1): Restrict transform skip size up to 4x4 on top of VTM-4.0 with JVET-M0464 enabled

- Test#2 (TSM_2): Restrict transform skip size up to 4x4 on top of VTM-4.0 with JVET-M0464 disabled (including bugfix of ticket#185)

Simulation results show that coding performance of TMS_1 drop by roughly 0.2%, 0.2%, and 0.1% of luma bdrate over TSM_2 for AI, RA and LB, respectively; roughly 0.3%, 0.2%, and 0.2% chroma bdrate overTSM_2 for AI/RA/LB, respectively

Additionally, to overcome chroma bdrate degradation observed in TSM_1, this contribution proposes to 1) support chroma transform skip, and 2) unify transform type signaling between luma and chroma; relocate transform_skip_flag and tu_mts_idx into residual_coding().

It is recommended to support chroma transform skip in the next VTM and WD, or study unified transform type signaling between luma and chroma in CE.

Introduction

At the last meeting, unified Transform Type signaling (called uniMTS) in JVET-M0464 [1] was adopt in VTM-4.0 [2]. It includes following modifications:

- Unified signalling for transform skip and MTS

- Luma transform skip size extension up to 32x32

- No chroma transform skip

- Fast transform type selection as non-normative changes: limits # of candidates for RDO to 3 for intra MTS and 4 for inter MTS

In AHG13 Tool reporting procedure [3], tool off test on item b) Luma transform skip size extension up to 32x32 was carried out.

This contribution reports additionally tool off test for all adopt aspects a), b), c), and d) of [1].

Additional tool off tests for AHG13

For AHG13 Tool reporting procedure [3], following tool off tests were carried out:

- TSM_1: Restrict transform skip size up to 4x4 on top of VTM-4.0 (JVET-M0464 enabled)

  - Encoder option: --MTS=1 --TransformSkipLog2MaxSize=2

  - Chroma Transform skip: N/A

  - Configuration for TSM_1 are found in config_TSM_1.

- TSM_2: Restrict transform skip size  up to 4x4 on top of VTM-4.0 with JVET-M0464 disabled (including ticket#185)

  - Encoder option: --EMT=1 --EMTFast=1 --TransformSkipLog2MaxSize=2 with JVET-M0464 macro disabled

  - Chroma Transform skip: 2x2/4x2/2x4/4x4 can be used

  - Configuration and source changes for TSM_2 are found in config_TSM_2.

Note this setting is used to align with TS/MTS setting in VTM-3.0 CTC because VTM-3.0 allows luma/chroma TS up to 4x4.

This test was cross-checked by Tencent.

Figure 1-1 shows the coding performance of TSM_2 over TSM_1. Followings are observed on coding performance of TSM_1 over TSM_2:

- roughly 0.2%/0.2%/0.1% luma bdrate changes (loss) for AI/RA/LB

- roughly 0.3%/0.1%/0.0% chroma bdrate changes (loss) for AI/RA/LB

Considerable reasons of coding loss are:

- No chroma transform skip

- Fast transform type selection as non-normative change: limits # of candidates for RDO: 3 for Intra MTS, and 4 for inter MTS

Detail results are found in the attached spreadsheets: JVET-N0123_TSM_1.xlsx, JVET-N0123_TSM_2.xlsx and JVET-N0123_TSM_1_over_TSM_2.

Table 2-1: Coding performance of TSM_1 over TSM_1 under CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.24% | 0.20% | 0.18% | 101% | 99% |
| Class A2 | 0.14% | 0.40% | 0.24% | 100% | 96% |
| Class B | 0.23% | 0.38% | 0.39% | 101% | 100% |
| Class C | 0.32% | 0.35% | 0.38% | 101% | 101% |
| Class E | 0.27% | 0.53% | 0.41% | 101% | 102% |
| Overall | 0.24% | 0.37% | 0.33% | 101% | 100% |
| Class D | 0.30% | 0.27% | 0.24% | 102% | 103% |
| Class F | 1.96% | 1.42% | 1.39% | 98% | 102% |
|  | Random Access |  |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.31% | -0.20% | 101% | 99% |
| Class A2 | 0.04% | 0.12% | 0.12% | 99% | 99% |
| Class B | 0.17% | -0.08% | -0.17% | 100% | 101% |
| Class C | 0.35% | -0.04% | 0.01% | 99% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.17% | -0.07% | -0.07% | 100% | 100% |
| Class D | 0.54% | -0.19% | -0.58% | 98% | 103% |
| Class F | 1.80% | 1.04% | 1.05% | 99% | 102% |
|  | Low Delay B |  |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.31% | -0.01% | 99% | 100% |
| Class C | 0.37% | 0.06% | -0.21% | 98% | 102% |
| Class E | -0.23% | 0.38% | 0.13% | 100% | 102% |
| Overall | 0.09% | -0.01% | -0.04% | 99% | 101% |
| Class D | 0.53% | 0.10% | 0.32% | 96% | 103% |
| Class F | 2.03% | 0.85% | 0.78% | 98% | 100% |

Proposal

To overcome chroma bdrate degradation, it is proposed to support chroma transform skip:

- Unify TS/MTS signaling between luma and chroma: relocating transform_skip_flag and mts_idx into residual_coding()

- Add one context model for chroma transform skip

- Minimum change for mts_idx:

  - No context model change

  - No binarization change

Detail changes are found in JVET-N0123_SpecChanges_chromaTS.docx.

In addition, since in HEVC transform skip can be controlled per components Y/Cb/Cr. Thus, to inherit design spirits of HEVC, it is desirable to support transform skip control capability per components in VVC.

Proposed specification changes

- Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| …skipped… |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( transform_skip_enabled_flag && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( (( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag )) && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( !transform_skip_flag[ x0 ][ y0 ] ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |

- Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| tsAllowed = (transform_skip_enabled_flag && (1<<log2TbWidth) <= MaxTsSize && (1<<log2TbHeight) <= MaxTsSize) && (IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) |  |
| mtsAllowed = (cIdx==0) &&(( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && sps_explicit_mts_inter_enabled_flag )¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag )) && (( 1<<log2TbWidth) <= 32 ) && (( 1<<log2TbHeight) <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) |  |
| if( tsAllowed ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( mtsAllowed && !transform_skip_flag[ x0 ][ y0 ][ cIdx ]) |  |
| tu_mts_idx[ x0 ][ y0 ][ cIdx ][ cIdx ] | ae(v) |
| if( ( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbWidth > 4 ) |  |
| log2TbWidth = 4 |  |
| else |  |
| log2TbWidth = Min( log2TbWidth, 5 ) |  |
| if( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbHeight > 4 ) |  |
| log2TbHeight = 4 |  |
| else |  |
| log2TbHeight = Min( log2TbHeight, 5 ) |  |
| …skipped… |  |

- Transform unit semantics

… skipped…

transform_skip_flag[ x0 ][ y0 ] specifies whether a transform is applied to the luma transform block or not. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. transform_skip_flag[ x0 ][ y0 ] equal to 1 specifies that no transform is applied to the luma transform block. transform_skip_flag[ x0 ][ y0 ] equal to 0 specifies that the decision whether transform is applied to the luma transform block or not depends on other syntax elements. When transform_skip_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

tu_mts_idx[ x0 ][ y0 ] specifies which transform kernels are applied to the residual samples along the horizontal and vertical direction of the associated luma transform block. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture.

When tu_mts_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

- Residual coding semantics

…skipped…

transform_skip_flag[ x0 ][ y0 ][ cIdx ] specifies whether a transform is applied to the associated luma transform block or not. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. transform_skip_flag[ x0 ][ y0 ][ cIdx ] equal to 1 specifies that no transform is applied to the current luma transform block. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb and equal to 2 for Cr. transform_skip_flag[ x0 ][ y0 ][ cIdx ] equal to 0 specifies that the decision whether transform is applied to the current transform block or not depends on other syntax elements. When transform_skip_flag[ x0 ][ y0 ][ cIdx ] is not present, it is inferred to be equal to 0.

tu_mts_idx[ x0 ][ y0 ][ cIdx ] specifies which transform kernels are applied to the residual samples along the horizontal and vertical direction of the associated luma transform block. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb and equal to 2 for Cr.

When tu_mts_idx[ x0 ][ y0 ][ cIdx ] is not present, it is inferred to be equal to 0.

If ChromaArrayType is larger than 0, tu_mts_idx[ x0 ][ y0 ][ 1] and tu_mts_idx[ x0 ][ y0 ][ 2] shall be equal to 0.

..skipped…

Conclusion

To overcome chroma bdrate degradation, this contribution proposes to support chroma transform skip:

- Unify TS/MTS signaling between luma and chroma: relocating transform_skip_flag and mts_idx into residual_coding()

- Add one context model for chroma transform skip

- Minimum change for mts_idx:

  - No context model change

  - No binarization change

It is recommended to support chroma transform skip in the next VTM and WD, or study unified transform type signaling between luma and chroma in CE.

References

[1] B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, “Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip, ” JVET-M0464, Jan. 2019

[2] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” JVET-M1001, Jan. 2019

[3] W.-J. Chien, J. Boyce, “Methodology and reporting template for tool testing, ” JVET-M1005, Jan. 2019

[4] VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

[5] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Oct. 2019

[6] VTM-4.0.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0.1

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).