| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0124 |
| --- | --- |

| Title: | On Tile Group Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Modification are proposed to tile group signalling.

Following is proposed:

- Proposal 1: An inference rules is specified for single_tile_per_tile_group_flag when not present.

- Proposal 2: It is proposed to specify the bottom right tile index for the tile located at the bottom-right corner of the i-th tile group as a tile index difference with respect to the index of the tile located at the top-left corner of that i-th tile group.

- Proposal 3: It is proposed to specify the top left tile index for the tile located at the top-left corner of the i-th tile group as a tile index difference with respect to the index of the tile located at the top-left corner of the (i-1)-th tile group.

- Proposal 4: It is proposed to signal if a tile group is a motion constrained tile set.

# Introduction

VVC WD [1] includes signalling for tiles and tile groups.

Tile group information is mainly signaled in PPS and in tile group header.

We propose modifications to tile group signaling in PPS for bug-fix, bit efficiency and functionality reasons.

# Proposals

Following is proposed:

- Proposal 1: An inference rules is specified for single_tile_per_tile_group_flag when not present.

- Proposal 2: It is proposed to specify the bottom right tile index for the tile located at the bottom-right corner of the i-th tile group as a tile index difference with respect to the index of the tile located at the top-left corner of that i-th tile group (top_left_tile_idx[ i ]).

- Proposal 3: It is proposed to specify the top left tile index for the tile located at the top-left corner of the i-th tile group as a tile index difference with respect to the index of the tile located at the top-left corner of the (i-1)-th tile group.

- Proposal 4: It is proposed to signal if a tile group is a motion constrained tile set.

## Proposal 1

An inference rules is specified for single_tile_per_tile_group_flag when not present. The proposed changes are shown highlighted below.

It is asserted that currently in VVC WD [1] the value of single_tile_per_tile_group flag is used in the conditional signaling of tile_group_address in tile_group_header as shown below.

#### 7.3.4.1 General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| tile_group_address | u(v) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag ) |  |
| num_tiles_in_tile_group_minus1 | ue(v) |

However, when single_tile_in_pic_flag is equal to 1, single_tile_per_tile_group_flag is not signaled as shown below. Thus missing inference rule for single_tile_per_tile_group_flag when not present is proposed.

#### 7.3.2.2 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| single_tile_per_tile_group_flag | u(1) |
| if( !single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |
| … |  |

#### 7.4.3.2 Picture parameter set RBSP semantics

pps_pic_parameter_set_id identifies the PPS for reference by other syntax elements. The value of pps_pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

…

single_tile_per_tile_group equal to 1 specifies that each tile group that refers to this PPS includes one tile. single_tile_per_tile_group equal to 0 specifies that a tile group that refers to this PPS may include more than one tile. When not present single_tile_per_tile_group is inferred to be equal to 1.

## Proposal 2

It is proposed to specify the bottom right tile index for the tile located at the bottom-right corner of the i-th tile group as a tile index difference with respect to the index of the tile located at the top-left corner of that i-th tile group (top_left_tile_idx[ i ]). It is asserted that since bottom right tile index for a tile group is greater than or equal to top left tile index, coding in this manner is always possible and that it saves bits. The proposed changes are shown highlighted below.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| single_tile_per_tile_group_flag | u(1) |
| if( !single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx_delta[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| } |  |

#### Picture parameter set RBSP semantics

…

top_left_tile_idx[ i ] specifies the tile index of the tile located at the top-left corner of the i-th tile group. The value of top_left_tile_idx[ i ] shall not be equal to the value of top_left_tile_idx[ j ] for any i not equal to j. When not present, the value of top_left_tile_idx[ i ] is inferred to be equal to i. The length of the top_left_tile_idx[ i ] syntax element is Ceil( Log2( NumTilesInPic ) bits.

bottom_right_tile_idx_delta[ i ] specifies the difference between the tile index of the tile located at the bottom-right corner of the i-th tile group and the tile index of the tile located at the top-left corner of the i-th tile group (i.e. top_left_tile_idx[ i ]). When single_tile_per_tile_group_flag is equal to 1 bottom_right_tile_idx_delta[ i ] is inferred to be equal to 0 top_left_tile_idx[ i ]. The length of the bottom_right_tile_idx_delta[ i ] syntax element is Ceil( Log2( NumTilesInPic-top_left_tile_idx[ i ]) ) bits.

It is a requirement of bitstream conformance that any particular tile shall only be included in one tile group.

The variable NumTilesInTileGroup[ i ], which specifies the number of tiles in the i-th tile group, and related variables, are derived as follows:

deltaTileIdx = bottom_right_tile_idx[ i ] – top_left_tile_idx[ i ] 
NumTileRowsInTileGroupMinus1[ i ] = deltaTileIdx bottom_right_tile_idx_delta[ i ] / ( num_tile_columns_minus1 + 1 )
NumTileColumnsInTileGroupMinus1[ i ]  = deltaTileIdx bottom_right_tile_idx_delta[ i ] % ( num_tile_columns_minus1 + 1 )	(7-33)
NumTilesInTileGroup[ i ] = ( NumTileRowsInTileGroupMinus1[ i ] + 1 ) *
	  ( NumTileColumnsInTileGroupMinus1[ i ] + 1 )

## Proposal 3

It is proposed to specify the top left tile index for the tile located at the top-left corner of the i-th tile group as a tile index difference with respect to the index of the tile located at the top-left corner of the (i-1)-th tile group. It is asserted that this saves bits and also defines an order in which the tile groups are signalled which is asserted to be beneficial for structured signalling and for error checking. The proposed changes are shown highlighted below.

#### 7.3.2.2 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| single_tile_per_tile_group_flag | u(1) |
| if( !single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_tile_idx_delta[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| } |  |

#### 7.4.3.2 Picture parameter set RBSP semantics

…

top_left_tile_idx_delta[ i ] when i is greater than 0 specifies difference between the tile index of the tile located at the top-left corner of the i-th tile group and the tile index of the tile located at the top-left corner of the (i-1)-th tile group. top_left_tile_idx_delta[ 0 ] specifies the tile index of the tile located at the top-left corner of the 0-th tile group. The value of top_left_tile_idx_delta[ i ] shall not be equal to 0 when i is > 0 the value of top_left_tile_idx[ j ] for any i not equal to j. When not present, and when rect_tile_group_flag is equal to 1, the value of top_left_tile_idx_delta[ i ] is inferred to be equal to i 1 when i is greater than 0 and equal to 0 when i is equal to 0. The length of the top_left_tile_idx_delta[ i ] syntax element is Ceil( Log2( NumTilesInPic-TopLeftTileIdx[ i -1]) bits when i is greater than 0.

bottom_right_tile_idx[ i ] specifies the tile index of the tile located at the bottom-right corner of the i-th tile group. When single_tile_per_tile_group_flag is equal to 1 bottom_right_tile_idx[ i ] is inferred to be equal to top_left_tile_idx[ i ]. The length of the bottom_right_tile_idx[ i ] syntax element is Ceil( Log2( NumTilesInPic) ) bits.

It is a requirement of bitstream conformance that any particular tile shall only be included in one tile group.

The variable NumTilesInTileGroup[ i ], which specifies the number of tiles in the i-th tile group, and related variables, are derived as follows:

TopLeftTileIdx[ 0 ] = 0 
TopLeftTileIdx[ i ] = TopLeftTileIdx[ i–1 ] + top_left_tile_idx_delta[ i ]    
deltaTileIdx = bottom_right_tile_idx[ i ] – top_left_tile_idx TopLeftTileIdx[ i ]
NumTileRowsInTileGroupMinus1[ i ] = deltaTileIdx / ( num_tile_columns_minus1 + 1 )
NumTileColumnsInTileGroupMinus1[ i ]  = deltaTileIdx % ( num_tile_columns_minus1 + 1 )	(7-33)
NumTilesInTileGroup[ i ] = ( NumTileRowsInTileGroupMinus1[ i ] + 1 ) *
	  ( NumTileColumnsInTileGroupMinus1[ i ] + 1 )

### Tile group header semantics

#### 7.4.5.1 General tile group header semantics

…

num_tiles_in_tile_group_minus1, when present, specifies the number of tiles in the tile group minus 1. The value of num_tiles_in_tile_group_minus1 shall be in the range of 0 to NumTilesInPic − 1, inclusive. When not present, the value of num_tiles_in_tile_group_minus1 is inferred to be equal to 0.

The variable NumTilesInCurrTileGroup, which specifies the number of tiles in the current tile group, and TgTileIdx[ i ], which specifies the tile index of the i-th tile in the i-th tile group, are derived as follows:

if( rect_tile_group_flag ) {
	tileGroupIdx = 0
	while( tile_group_address  !=  tile_group_id[ tileGroupIdx ] )
		tileGroupIdx++
	NumTilesInCurrTileGroup = NumTilesInTileGroup[ tileGroupIdx ]
	tileIdx = top_left_tile_idxTopLeftTileIdx[ tileGroupIdx ]
	for( j = 0, tIdx = 0; j < NumTileRowsInTileGroupMinus1[ tileGroupIdx ] + 1;	(7-35)
					j++, tileIdx  +=  num_tile_columns_minus1 + 1 )
		for( i = 0, currTileIdx = tileIdx; i < NumTileColumnsInTileGroupMinus1[ tileGroupIdx ] + 1;
						i++, currTileIdx++, tIdx++ )
			TgTileIdx[ tIdx ] = currTileIdx
} else {
	NumTilesInCurrTileGroup = num_tiles_in_tile_group_minus1 + 1
	TgTileIdx[ 0 ] = tile_group_address
	for( i = 1; i < NumTilesInCurrTileGroup; i++ )
		TgTileIdx[ i ] = TgTileIdx[ i − 1 ] + 1
}

Proposal 2 and proposal 3 can be used together.

## Proposal 4

It is proposed to signal if a rectangular tile group is a motion constrained tile set. It is asserted that knowing this information can be helpful for 360 video bitstream rewriting scenarios as it allows knowing if tile groups from different representations can be easily rewritten together to create a merged bitstream.

#### 7.3.2.2 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| single_tile_per_tile_group_flag | u(1) |
| if( !single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| if( rect_tile_group_flag) { |  |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| is_mcts_flag[ i ] | u(1) |
| } |  |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| } |  |

is_mcts_flag[ i ] equal to 1 specifies that i-th tile group is a motion constrained tile set. is_mcts_flag[ i ]  equal to 0 specifies that i-th tile group is not a motion constrained tile set.

# Conclusion

This document proposes modifications to tile group signaling in VVC. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-M1001-v6, Versatile Video Coding (Draft 4), Marrakech, March 2019