| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N0125 |
| --- | --- |

| Title: | CE4-3.1 : MMVD binarization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungsoo Jeong Min Woo Park Kiho Choi Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | ss00.jeong@samsung.com m.w.park@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics Co.,Ltd |  |  |

_____________________________

- Abstract

This contribution proposes a method for improving the binarization of MMVD signalling which was introduced by the proposal document of JVET-M0206. This includes three tests. The first test is the binarization method change of distance list index. There is no additional CTX model and other change. Experimental results shows -0.02% luma BD-rate change in RA and 0.00% luma BD-rate change in LDB. The second test is multi-distance list method. Experimental results shows -0.01% luma BD-rate change in RA and 0.00% luma BD-rate change in LDB. The third test is the combination of first test and second test. Experimental results shows -0.02% luma BD-rate change in RA and 0.0X% luma BD-rate change in LDB.

Introduction

In the process of distance index signalling, distance index is signalled with truncated unary coding method. It is described at Table 1.

Table 1. Distance list index binarization

| Distance list index | binarization |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| 2 | 110 |
| 3 | 1110 |
| 4 | 11110 |
| 5 | 111110 |
| 6 | 1111110 |
| 7 | 1111111 |

Proposed Method

Method 1: binarization method

This binarization method is same with the binarization method which was proposed by the document of JVET-M0206.

Table 2. Proposed binarization method

| Distance list index | binarization |
| --- | --- |
| 0 | 00 |
| 1 | 01 |
| 2 | 10 |
| 3 | 110 |
| 4 | 1110 |
| 5 | 11110 |
| 6 | 111110 |
| 7 | 111111 |

Table 3. The result of method 1

Method 2: multi-distance list concept

In last meeting, the usage of new flag is adopted for disabling fraction MVD of MMVD [1]. If fractional MVD is enabled, the default distance table is used, otherwise, all elements in the default distance table are left shifted by 2.

Proposed method gives this flag more flexibility for the coverage of various sequences. In this method, encoder and decoder use one single distance list. Encoder checks the ratio of each distance index, and then, determines which shift number for a single distance list is best. The shift number is signaled with FracMMVD syntax which is signaled with UVLC coding.

previous

- 7.3.4.1 General tile group header syntax

| if ( tile_group_type != I ) { |  |
| --- | --- |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| if( ( weighted_pred_flag && tile_group_type = = P ) ¦ ¦ ( weighted_bipred_flag && tile_group = = B ) ) |  |
| pred_weight_table( ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if() |  |
| tile_group_fpel_mmvd_enabled_flag | u(1) |
| } else if ( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_merge_cand | ue(v) |

tile_group_fpel_mmvd_enabled_flag equal to 1 specifies that merge mode with motion vector difference uses integer sample precision in the current tile group. tile_group_fpel_mmvd_enabled_flag equal to 0 specifies that merge mode with motion vector difference can use fractional sample precision in the current tile group. When not present, the value of tile_group_fpel_mmvd_enabled_flag is inferred to be 0.

Table 7-11 – Specification of MmvdDistance[ x0 ][ y0 ] based on mmvd_distance_idx[ x0 ][ y0 ].

| mmvd_distance_idx[ x0 ][ y0 ] | MmvdDistance[ x0 ][ y0 ] |  |
| --- | --- | --- |
|  | tile_group_fpel_mmvd_enabled_flag = = 0 | tile_group_fpel_mmvd_enabled_flag = = 1 |
| 0 | 1 | 4 |
| 1 | 2 | 8 |
| 2 | 4 | 16 |
| 3 | 8 | 32 |
| 4 | 16 | 64 |
| 5 | 32 | 128 |
| 6 | 64 | 256 |
| 7 | 128 | 512 |

proposed

- 7.3.4.1 General tile group header syntax

| if( ( weighted_pred_flag && tile_group_type = = P ) ¦ ¦ ( weighted_bipred_flag && tile_group = = B ) ) |  |
| --- | --- |
| pred_weight_table( ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if(sps_fpel_mmvd_enabled_flag) |  |
| tile_group_mmvd_shift_value | ue(v) |
| } else if ( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_merge_cand | ue(v) |

tile_group_mmvd_shift_value not equal to 0 specifies that elements of the default distance table are left shifted by tile_group_fpel_mmvd_enabled_flag in the current tile group. tile_group_fpel_mmvd_enabled_flag equal to 0 specifies that merge mode with motion vector difference can use the default distance table in the current tile group. When not present, the value of tile_group_fpel_mmvd_enabled_flag is inferred to be 0.

Table 7-11 – Specification of MmvdDistance[ x0 ][ y0 ] based on mmvd_distance_idx[ x0 ][ y0 ].

| mmvd_distance_idx[ x0 ][ y0 ] | MmvdDistance[ x0 ][ y0 ] |  |
| --- | --- | --- |
|  | tile_group_fpel_mmvd_enabled_flag = = 0 | tile_group_fpel_mmvd_enabled_flag != 0 |
| 0 | 1 | 1 << tile_group_mmvd_shift_value |
| 1 | 2 | 2 << tile_group_mmvd_shift_value |
| 2 | 4 | 4 << tile_group_mmvd_shift_value |
| 3 | 8 | 8 << tile_group_mmvd_shift_value |
| 4 | 16 | 16 << tile_group_mmvd_shift_value |
| 5 | 32 | 32 << tile_group_mmvd_shift_value |
| 6 | 64 | 64 << tile_group_mmvd_shift_value |
| 7 | 128 | 128 << tile_group_mmvd_shift_value |

Method 3: Combination of Method 1 and Method 2

This is the combination result of both method 1 and method 2.

Conclusions

This document proposes binarization method and multi-distance list method.

Reference

- “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

Patent rights declaration(s)

Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).