| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0131-v1 |
| --- | --- |

| Title: | CE3: Offset-based reference sample reduction for CCLM (Tests 1.2.1 and 1.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo Yanzhuo Ma Shuai Wan Xinwei Li Xiaoyan Chai Jianglin Wang Fuzheng Yang YuanFang Yu Yang Liu | Email: | jyhuo@mail.xidian.edu.cn yzma@mail.xidian.edu.cn swan@nwpu.edu.cn xwli_xidian@stu.xidian.edu.cn jlwang_2@stu.xidian.edu.cn xychai@stu.xidian.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

# Abstract

This contribution presents the results of CE3 test 1.2.1 and test 1.2.2. The two tests reduce the number of reference samples to 2 or 4 for CCLM through selecting 2 or 4 samples regularly using an offset. In test 1.2.1, the number of the reference samples is 2 or 4 according to the size of the block. In test 1.2.2, the number of the reference samples is 4, which is uniform for all block sizes.

Simulation results reportedly show BD rate on Y, Cb and Cr components and encoding/decoding time comparison over VTM4.0:

- Test 1.2.1, using 2 or 4 reference samples according to the block sizes:

  - For AI configuration: -0.03%, -0.11%, and -0.21%, with 100% EncT, 100% DecT;

  - For RA configuration: -0.01%, -0.24%, and -0.27%, with 100% EncT, 100% DecT;

- Test 1.2.2, using 4 reference samples:

  - For AI configuration: -0.03%, -0.24%, and -0.31%, with 100% EncT, 99% DecT;

  - For RA configuration: -0.01%, -0.35%, and -0.46%, with 100% EncT, 100% DecT.

# Introduction

In VTM3.0, a simplified line fitting method of cross-component linear model (CCLM) was adopted, named as the MaxMin method. Two Luma samples and their corresponding chroma samples are used to obtain the linear model parameters α and β. The two luma/chorma samples used in the MaxMin method are those with the minimum and the maximum luma value among the reference samples, respectively, as Point A and B shown in Figure 1.

A

B

A

B

Figure 1 Illustration of the MaxMin method

The linear model parameters α and β are obtained according to the following equations:

β= ∑Cn-α·∑LnN

α=yB-yAxB-xA                                        （1）

β= yA-αxA                                      （2）

Table 1 shows the number of operations which are used to derive the linear model parameter α and β.

Table 1 Number of operations used in the MaxMin method

| Operation type | Number of operations |
| --- | --- |
| Multiplications | 1 |
| Addition | 3 |
| “Division” | 1 |
| Comparisons | 2N |

As for the number of reference samples N, it can be calculated as follows:

For a coding block with a square shape, N is equal to twice of the width/height;

for a non-square coding block, N is equal to twice of the smaller size in the width and height, where the reference samples of the longer boundary are firstly subsampled to the same number of samples as for the shorter boundary;

if only the left neighbouring samples or the above neighbouring samples can be used, N is equal to the number of samples in the available boundary.

Compared with linear regression method, the MaxMin method can reduce the complexity to calculate the model parameters. Meanwhile, a small coding loss is introduced because only two points are used to calculate the linear model parameter in the MaxMin method.

# Proposed method

- Reference sample selection

In the two CE tests, the reference samples used for finding max and min value are selected according to the starting offset and the sampling interval [1]. Figure 2 shows how to select 4 reference samples from a 16-pixel line.

Figure 2 The reference samples selection in proposed method

The sampling interval and starting offset are calculated as follows:

-Original reference samples number: L=16

-Reduced reference samples number: N=4

-Sampling interval: ∆=L/N=4

-Starting offset = ∆/2 =2

The proposed method chooses reference samples which are intuitively highly related to the current coding block. The proposed method can improve the robustness of the MaxMin method.

- Number of operation in the proposed methods

The number of reference samples in proposed methods is shown in Table 2. In test 1.2.1, the number of the reference samples is 2 or 4 respectively according to the size of the block. In test 1.2.2, the number of the reference samples is 4, which is uniform for all block sizes.

Table 2 The number of reference samples in the Maxmin method, Test1.2.1 and Test 1.2.2.

| Short length of chroma block | Reference samples number in the MaxMin method N | Reference samples number in test 1.2.1 N1.2.1 | Reference samples number in test 1.2.2 N1.2.2 |
| --- | --- | --- | --- |
| 2 | 4 | 2 | 4 |
| 4 | 8 | 4 |  |
| 8 | 16 |  |  |
| 16 | 32 |  |  |
| 32 | 64 |  |  |

As for the model parameter calculation in the proposed methods, the multiplication, addition and “Division” operation is the same as the Maxmin method (as shown in Table 1), while the number of “comparisons” is reduced and depicted in Table 3.

Table 3 The number of comparison in the Maxmin method, Test1.2.1 and Test 1.2.2.

|  | MaxMin method | Test 1.2.1 | Test 1.2.2 |
| --- | --- | --- | --- |
| Comparisons | 2N | 4 or 8 | 8 |

# Experimental results

The two tests are implemented based on VTM4.0. The coding performance is provided in Table 4 and Table 5.

Test 1.2.1

Anchor: VTM4.0

Tested: CE3-1.2.1

Table 4. The results of test 1.2.1

|  |  |  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V |  | EncT |  | DecT |
| Class A1 | -0.09% | -0.22% | -0.45% |  | 100% |  | 100% |
| Class A2 | 0.01% | 0.11% | 0.06% |  | 100% |  | 98% |
| Class B | -0.01% | -0.33% | -0.36% |  | 100% |  | 99% |
| Class C | -0.02% | -0.01% | -0.07% |  | 100% |  | 102% |
| Class E | -0.03% | 0.01% | -0.17% |  | 100% |  | 99% |
| Overall | -0.03% | -0.11% | -0.21% |  | 100% |  | 100% |
| Class D | -0.01% | 0.04% | 0.08% |  | 100% |  | 100% |
| Class F | 0.01% | 0.23% | -0.08% |  | 99% |  | 98% |
|  |  |  | RandomAccess |  |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT |  | DecT |  |
| Class A1 | -0.03% | -0.29% | -0.28% | 100% |  | 99% |  |
| Class A2 | -0.03% | 0.06% | 0.11% | 100% |  | 100% |  |
| Class B | 0.00% | -0.50% | -0.54% | 100% |  | 100% |  |
| Class C | 0.02% | -0.09% | -0.20% | 100% |  | 100% |  |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.24% | -0.27% | 100% |  | 100% |  |
| Class D | -0.01% | -0.17% | -0.02% | 100% |  | 101% |  |
| Class F | 0.00% | 0.02% | -0.09% | 100% |  | 99% |  |

Test 1.2.2

Anchor: VTM4.0

Tested: CE3-1.2.2

Table 5. The results of test 1.2.2

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.23% | -0.47% | 100% | 100% |
| Class A2 | -0.01% | 0.05% | -0.03% | 100% | 100% |
| Class B | -0.02% | -0.42% | -0.48% | 100% | 99% |
| Class C | -0.03% | -0.32% | -0.35% | 99% | 99% |
| Class E | -0.03% | -0.11% | -0.13% | 100% | 99% |
| Overall | -0.03% | -0.24% | -0.31% | 100% | 99% |
| Class D | -0.02% | -0.13% | -0.34% | 100% | 100% |
| Class F | -0.03% | -0.01% | -0.41% | 99% | 100% |
|  |  |  | RandomAccess |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.48% | -0.41% | 100% | 100% |
| Class A2 | -0.01% | -0.13% | -0.05% | 100% | 100% |
| Class B | 0.01% | -0.55% | -0.74% | 100% | 100% |
| Class C | 0.00% | -0.18% | -0.47% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.35% | -0.46% | 100% | 100% |
| Class D | -0.01% | -0.43% | -0.62% | 100% | 101% |
| Class F | -0.05% | -0.16% | -0.36% | 100% | 100% |

# Conclusions

This contribution presents two simplified CCLM methods to reduce the number of reference samples and design the reference samples position. The test 1.2.1 can get -0.03%, -0.11%, and -0.21% BD-bitrate gain for Y, Cb and Cr, and 100% decoding time in AI configuration and -0.01%, -0.24% and -0.27% BD-bitrate gain and 100% decoding time in RA configuration. The test 1.2.2 can get -0.03%, -0.24%, and -0.31% BD-bitrate gain for Y, Cb and Cr with 99% decoding time in AI configuration and -0.01%, -0.35% and -0.46% BD-bitrate gain with100% decoding time in RA configuration.

- References

- J.-Y. Huo, X.-W. Li, X.-Y. Chai, J.-L. Wang, Y.-Z. Ma, F.-Z. Yang, S. Wan, Y.-F. Yu, Y. Liu, JVET-M0211, “CE3-related: Fixed Reference Samples Design for CCLM”, Jan. 2019.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).