| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0132-v4 |
| --- | --- |

| Title: | Non-CE3: Simplification of DC Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo Jianglin Wang Xinwei Li Shuai Wan Yanzhuo Ma Wei Zhang Fuzheng Yang YuanFang Yu Yang Liu | Tel: Email: | jyhuo@mail.xidian.edu.cn jlwang_2@stu.xidian.edu.cn xwli_xidian@stu.xidian.edu.cn swan@nwpu.edu.cn yzma@mail.xidian.edu.cn wzhang@xidian.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

This contribution presents a simplification of DC value calculation. For square blocks, the calculation of DC value only use two reference samples, the center reference sample in the left column and top line. For non-square blocks, the center reference sample of the longer boundary is used as the DC value. The simulation results show 0.00%, 0.02%, and -0.01% BD-rates on Y, Cb, and Cr components, respectively, compared to VTM4.0 in All Intra configuration.

Introduction

In VVC draft, the mean of the neighbouring reference samples is used as the DC value. As shown in Figure 1, for a square block, the reference samples at both sides of the block are taken to calculate the DC value. For a non-square block, only the reference samples at the longer boundary are included.

Figure 1 The reference samples for calculating DC value in VVC

Proposed method

This contribution proposes a DC value calculation method with reduced reference samples. For a square block, the mean of the two reference samples at center of left column and top line is assigned as the DC value. For a non-square block, the center reference sample of the longer boundary is used as the DC value directly, as shown in Figure 2.

Figure 2 The reference samples for calculating DC value in proposed method

For square block, this method can reduce the number of addition. Assuming the block size is NxN, Table 1 shows the number of operations which are used to calculate DC value.

Table 1. The number of addition for calculating DC value

|  | Number of samples used | Number of ‘+’ operations | Number of ’>>’ operations |
| --- | --- | --- | --- |
| VTM4.0 | 2N | 2N-1 | 1 |
| The proposed method | 2 | 1 | 1 |

For non-square block, this method can reduce the number of addition and shift operations. Assuming the block size is MxN, Table2 shows the number of operations which are used.

Table 2. The number of operations for calculating DC value

|  | Number of samples used | Number of ‘+’ operations | Number of ’>>’ operations |
| --- | --- | --- | --- |
| VTM4.0 | Max(M,N) | Max(M,N)-1 | 1 |
| The proposed method | 1 | 0 | 0 |

Experimental results

The tests are implemented based on VTM4.0. The coding performance is provided in Table3.

Anchor: VTM4.0

Tested: The proposed method

Table 3. The results of the proposed method

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | -0.08% |  | 100% |
| Class A2 | 0.03% | 0.06% | 0.01% |  | 100% |
| Class B | 0.01% | 0.03% | 0.07% |  | 100% |
| Class C | 0.02% | -0.01% | 0.01% |  | 99% |
| Class E | -0.08% | -0.03% | -0.11% |  | 99% |
| Overall | 0.00% | 0.02% | -0.01% |  | 100% |
| Class D | 0.01% | 0.16% | 0.02% |  | 101% |
| Class F | -0.01% | -0.05% | -0.11% |  | 102% |
|  | Random Access |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | 0.03% | 98% | 98% |
| Class A2 | -0.03% | 0.05% | -0.06% | 97% | 98% |
| Class B | 0.00% | 0.10% | -0.19% | 99% | 99% |
| Class C | 0.00% | -0.17% | -0.27% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.14% | 99% | 98% |
| Class D | -0.01% | -0.46% | -0.17% | 100% | 102% |
| Class F | -0.04% | -0.08% | 0.05% | 99% | 99% |

Conclusions

This contribution presents a simplification of DC prediction. The proposed method could reduce computational complexity to generate DC values.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).