| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0133-v1 |
| --- | --- |

| Title: | CE13-related: In-loop filter with only CNN-based filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuai Wan Mingze Wang Yanzhuo Ma Junyan Huo Hao Gong Chengyi Zou YuanFang Yu Yang Liu | Email: | swan@nwpu.edu.cn roronoa@mail.nwpu.edu.cn yzma@mail.xidian.edu.cn jyhuo@mail.xidian.edu.cn gong_h@mail.nwpu.edu.cn 965737998@qq.com yuyuanfang@oppo.com serena@oppo.com |
| Source: | Northwestern Polytechnical University, Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

This contribution provides an in-loop filter based on convolutional neural network (CNNLF) named WSE-CNNLF(Wide-activated Squeeze-and-Excitation Convolutional Neural Network Loop Filter). In the current VVC, multiple filters, i.e., De-blocking Filter (DBF), Sample Adaptive Offset (SAO) and Adaptive Loop Filter (ALF) are used to remove artifacts or improve performance. This contribution proposes a single type of CNN-based in-loop filter, as a total solution, to replace and outperform the above-mentioned multiple filters. Simulation results report -5.72%, -8.95%, -10.77% BD-rate savings for Y, Cb and Cr components compared with VTM4.0 under AI configuration.

Introduction

Lossy compression of image and video yields visually annoying artifacts including blocking, blurring, ringing, etc., especially at low bit rates. In-loop filtering techniques can reduce these artifacts, improve the reconstructed video quality, and achieve coding gains accordingly. The convolutional neural network (CNN) is potential in this field.

CNN is a feedforward neural network, which consist of a mass of convolution operations and parameters (including weights and biases). The parameters are learned from massive amounts of training data. It is powerful in capturing the complex structures and textures of an image.

In this proposal, a deep residual CNN is proposed as a total solution, to replace all the in-loop filters in VTM4.0.

Technical description

Network inputs and outputs

The proposed network WSE-CNNLF(Wide-activated Squeeze-and-Excitation Convolutional Neural Network Loop Filter) includes six inputs: three reconstructed components and three auxiliary inputs, which are introduced as follows.

(1). Three reconstructed components

The three reconstructed components (i.e. Y/U/V components) are firstly normalized to [0, 1] by the maximum value according to the bit depth and then fed into network simultaneously. Which means, the normalized values are obtained according to equation (1).

p'x,y=p(x,y)1≪bitdepth-1 ,  x=1,…,W, y=1,…,H                (1)

Moreover, the three components are processed independently, jointly, and then independently again, with three enhanced components output simultaneously, which will be described in detail in 2.3 and shown in Fig.3. This structure can utilize the complex relationship between different components, meanwhile avoiding the repetitive implementations of complete forward computations for each components.

(2). Three auxiliary inputs

The first auxiliary input is the quantization parameter (QP). Different QPs lead to different reconstruction quality. Adding QP information as input makes the network with a single set of parameters can adapt to multi-quality- reconstructions. The quantization parameters are normalized and converted to QPmap according to equation (2).

QPmapx,y= QP51 ,  x=1,…,W, y=1,…,H                 (2)

The other two auxiliary inputs are coding unit (CU) partition information, one for luma(Y) component, one for chroma(both U and V) components. As known, the blocking artifacts are mainly caused by CU partition split. Inputting the boundaries of CUs is an attention mechanism of neural networks to process those boundaries effectively. The CU partition data are organized as CUmaps and then normalized before fed into the neural network. For example, for each CU of a frame, the positions of the boundary are filled by 2 and other positions by 1 as in Fig 1. Then the normalization factor for CUmap is 2. Then we get two CUmaps, one for Y component, the other one for both U and V components.

Fig 1. A CUmap before normalization

Processing module

Wide-activated convolution[1] is known as an effective operation in super-resolution and denoising tasks. It is consist of a wider channel 3x3 convolution activated by a ReLU function and a narrower channel 1x1 convolution.

SE (Squeeze-and-Excitation)[2] operation is known as a method of weighting each convolutional layer adaptively, it is able to exploit the complex relationship between different channels and generate a weight factor for each channel.

Given a feature map X with shape HxWxC, the SE operation includes four steps as following.

(1) Each channel is squeezed to a single numeric value using Global Average Pooling (GAP) according to equation (3)

yi=1WHm=1Wn=1Hxmni  i=1,…,C                     (3)

(2) A fully connected layer followed by a ReLU function adds the necessary nonlinearity. Its output channel complexity is also reduced by a certain ratio r.

(3) A second fully connected layer followed by a sigmoid activation gives each channel a smooth gating ratio ranged in [0,1].

(4) At last, each channel is scaled by the gating ratio.

In this proposal, we integrate the wide-activated convolution and the SE operation to form the basic processing unit named as wide-SE-block, which is depicted in Fig 2.

All the shapes of the tensors are marked in Fig 2. The parameter k denotes the wide ratio in the first wide convolution, and the parameter r denotes the reduce ratio in SE operation for computation complexity reduction. For a basic processing unit, if the number of input channels is same as that of the output channels, a skip connection will be added from the input into the output directly to learn the residual image as shown by Fig 2(a), it can also benefit fast convergence. Otherwise, there is no skip connection as shown by Fig 2(b).

(a). type A                     (b). type B

Fig 2. wide-SE-block

Network structure

As depicted in Fig 3, the process flow of the proposed WSE-CNNLF consists of three stages.

In the first stage, the Y/U/V components are separately processed by wide-SE-blocks and each is fused with its own CUmap. Before concatenated to feature maps, CUmap will additionally be pointwisely multiplied to its own component. Since three components should be of equal size to jointly processed by neural network and the width and height of U/V component have only half size of that of the Y component, we upsample the U/V components to align the matrix’s sizes.

In the second stage, different feature maps obtained by different components are concatenated and then processed by a number of wide-SE-blocks.

And in the final stage, three components are independently processed once again to generate the final residual image. Then the original input will be added to implement as a residual CNN.

In each block in Fig.3, the parameter c denotes the number of output channel of the operation, and the parameter n decides how many times the operation repeated.

Fig 3. Main structure of the proposed WSE-CNNLF

Training stage

The network settings in training stage are shown in Table 1.

Table 1. Network setting in training stage

| Mandatory | Information in Training Stage |  |  |
| --- | --- | --- | --- |
|  | Basic Settings: |  |  |
|  |  | learning rate: | 1e-3 |
|  |  | optimizer: | ADAM |
|  |  | batch size: | 100 |
|  |  | epoch: | 200 |
|  |  | loss function: | L2 |
|  |  | training GPU: | GTX 1080ti |
|  |  | training time: | 96h |
|  |  | framework: | Tensorflow |
| Optional | Other Settings: |  |  |
|  |  | patch size: | 64x64, randomly cropped |
|  |  | additional dataset (besides DIV2K): | None |
|  |  | QP set for generating training dataset: | {22,27,32,37} |

Inference stage

In inference stage, the filters in VTM4.0 including DF, SAO and ALF are all disabled. As a total solution, the proposed CNN-based filter is the only in-loop filter. The tests are conducted under the common test conditions (CTC). The test QP values are 22, 27, 32, and 37.

Table 2 gives the information of inference. Only GPU result is given.

Table 2. Information in Inference Stage

|  | Network Details |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total Conv. Layers | Total FC Layers | Frame-work | Param. Num | Param. Precision | Mem.P(MB) | Mem.T(MB) (e.g., 4K input) |
| test1 | 39 | 36 | Tensorflow | 519112 | 32(F) | 1.98 | H*W*2.68*10e-3 |
| test2 |  |  |  |  |  |  |  |
|  | All Intra Main10 Over VTM-4.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time(GPU) | Decoding Time(GPU) | Encoding Time(CPU) | Decoding Time(CPU) |
| test1 | -5.72% | -8.95% | -10.77% | 67% | 1265% |  |  |
| test2 |  |  |  |  |  |  |  |
|  | Random Access Main10 Over VTM-4.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time(GPU) | Decoding Time(GPU) | Encoding Time(CPU) | Decoding Time(CPU) |
| test1 |  |  |  |  |  |  |  |
| test2 |  |  |  |  |  |  |  |
|  | Low delay B Main10 Over VTM-4.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time(GPU) | Decoding Time(GPU) | Encoding Time(CPU) | Decoding Time(CPU) |
| test1 |  |  |  |  |  |  |  |
| test2 |  |  |  |  |  |  |  |

Experimental Results

Table 3 shows the experimental results of proposed method (DBF,SAO and ALF disabled, only WSE-CNNLF is enabled) compared to VTM-4.0 anchor.

Table 3. Experimental results for AI condition compared with VTM-4.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -3.99% | -10.41% | -10.75% | 81% | 1037% |
| Class C | -5.92% | -9.56% | -12.46% | 61% | 1285% |
| Class E | -7.26% | -6.89% | -9.10% | 60% | 1474% |
| Overall | -5.72% | -8.95% | -10.77% | 67% | 1265% |
| Class D | -6.24% | -12.31% | -16.61% | 62% | 2901% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusions

This contribution reported the performance evaluation by the convolutional neural network filter proposed. The proposed filter replaced all exist filters in VTM4 and report -5.72%,.-8.95%, -10.77% BD-rate savings for luma, and both chroma components for VTM 4.0 with AI configuration.

Reference

- J Yu, Y Fan, J Yang, N Xu, Z Wang, X Wang, T Huang. "Wide Activation for Efficient and Accurate Image Super-Resolution." arXiv preprint, arXiv:1808.08718, 2018.

- Hu, Jie, et al. "Squeeze-and-Excitation Networks." 2017.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).