| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0134 |
| --- | --- |

| Title: | CE3: Simplification of MPM derivation (CE3-3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee Hahyun Lee Sung-Chang Lim Jungwon Kang Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr hanilee@etri.re.kr sclim@etri.re.kr jungwon@etri.re.kr hykim5@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution presents test results of CE3-3.2.1 and CE3-3.2.2 described in JVET-M1023. CE3-3.2.1 presents modified MPM derivation method which is using same MPM derivation when luma reference line index is equal to zero or intra sub-partitioning mode is enabled, and MPM derivation method for non-zero luma reference line index is modified. In addition, CE3-3.2.2 changes position of Planar mode to lower MPM index. It is reported that the average luma BD-rate for AI case of CE3-3.2.1 and CE3-3.2.2 are 0.10% and -0.05% compared to VTM 4.0, respectively.

- Algorithm description

In JVET-M0239[1], a modification method of most probable mode (MPM) derivation is proposed and included in CE3[2]. In CE3-3.2, MPM derivation process for non-zero luma reference line index is simplified, and MPM derivation method for intra sub-partitioning(ISP) mode is unified with general MPM derivation method as depicted in Fig. 1. Therefore, split type dependent MPM derivation for ISP mode is removed.

In addition, CE3-3.2.2 changes a position of Planar mode in MPM list to lower index as depicted in Fig. 2. If two neighbouring modes are different and those modes are greater than DC mode, Planar mode is set to index 1. Otherwise, if one neighbouring mode is Planar or DC mode, and the other neighbouring mode is an angular mode, Planar mode is set to index 0.

Figure 1. MPM derivation process in CE3-3.2.1

Figure 2. MPM derivation process in CE3-3.2.2

Experimental results

The experiments were performed based on VTM 4.0 software [3] with VTM 4.0 configuration file under JVET common test condition [4]. In CE3-3, a codebase including all MPM modes for full encoder RD check is also used.

Table 1 and Table 2 show experimental results of the CE3-3.2.1 and CE3-3.2.2 compared to VTM 4.0, respectively. In addition, Table 3 and Table 4 show experimental results of the CE3-3.2.1 and CE3-3.2.2 compared to the codebase, respectively.

Table 1. Results of CE3-3.2.1 over VTM 4.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.05% | 0.02% | 102% | 100% |
| Class A2 | 0.07% | 0.13% | 0.03% | 102% | 100% |
| Class B | 0.11% | 0.13% | 0.21% | 102% | 100% |
| Class C | 0.11% | 0.06% | 0.10% | 101% | 100% |
| Class E | 0.14% | 0.27% | 0.08% | 101% | 100% |
| Overall | 0.10% | 0.12% | 0.10% | 102% | 100% |
| Class D | 0.03% | 0.11% | -0.16% | 102% | 100% |
| Class F | 0.14% | 0.13% | -0.08% | 101% | 100% |

Table 2. Results of CE3-3.2.2 over VTM 4.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.13% | -0.08% | 101% | 100% |
| Class A2 | -0.03% | 0.08% | -0.02% | 102% | 100% |
| Class B | -0.02% | 0.08% | 0.07% | 102% | 100% |
| Class C | 0.04% | 0.02% | 0.05% | 102% | 100% |
| Class E | -0.06% | 0.08% | -0.07% | 101% | 100% |
| Overall | -0.05% | 0.03% | 0.00% | 102% | 100% |
| Class D | -0.10% | -0.02% | -0.28% | 102% | 100% |
| Class F | 0.18% | 0.14% | 0.01% | 101% | 100% |

Table 3. Results of CE3-3.2.1 over codebase (VTM 4.0 with full RD)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0_FullRD |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.02% | 0.07% | 101% | 100% |
| Class A2 | 0.08% | 0.02% | 0.07% | 101% | 100% |
| Class B | 0.11% | 0.10% | 0.06% | 101% | 100% |
| Class C | 0.09% | 0.02% | 0.15% | 101% | 100% |
| Class E | 0.14% | 0.18% | 0.08% | 101% | 100% |
| Overall | 0.09% | 0.07% | 0.09% | 101% | 100% |
| Class D | 0.02% | 0.03% | 0.00% | 101% | 100% |
| Class F | 0.12% | 0.17% | 0.05% | 101% | 100% |

Table 4. Results of CE3-3.2.2 over codebase (VTM 4.0 with full RD)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0_FullRD |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.12% | -0.04% | 101% | 100% |
| Class A2 | 0.02% | -0.05% | 0.01% | 101% | 100% |
| Class B | 0.04% | 0.06% | 0.06% | 101% | 100% |
| Class C | 0.08% | 0.05% | 0.16% | 101% | 100% |
| Class E | 0.03% | 0.08% | 0.08% | 101% | 100% |
| Overall | 0.02% | 0.01% | 0.06% | 101% | 100% |
| Class D | -0.06% | 0.01% | 0.07% | 101% | 100% |
| Class F | 0.20% | 0.24% | 0.17% | 101% | 100% |

Conclusions

This contribution reports the experimental result of CE3-3.2.1 and CE3-3.2.2. In the tests, MPM derivation process is unified and simplified according to luma reference line index or intra sub-partitioning mode. The experimental results reportedly show that the average luma BD-rates of CE3-3.2.1 and CE3-3.2.2 are 0.10% and -0.05% for AI case compared to VTM 4.0, respectively.

Reference

- J. Lee, H. Lee, S.-C. Lim, J. Kang, and H. Y. Kim, “Non-CE3: Modification of MPM derivation,” JVET-M0239, Jan. 2019.

- G. Van der Auwera, L. Li, and A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-M1023, Jan 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Jan 2019.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).