| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0136 |
| --- | --- |

| Title: | AHG17: On reference picture list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuji Fujimoto and Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 Yuji.Fujimoto@sony.com teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In the current VVC design, there is no syntax to indicate whether the current picture is reference picture or non-reference picture. If there is no such syntax element at higher layer of syntax, it is hard to identify if the current picture can be discarded. This contribution proposes to include a flag to identify whether current picture is non-reference picture. Three methods are proposed.

Introduction

In the current VVC design, there is no syntax to indicate whether the current picture is reference picture or non-reference picture. If there is no such syntax element at higher layer of syntax, it is hard to identify if the current picture can be discarded. In case of SW decoder, when its resource is not sufficient, decoder may skip to decode non-reference picture. But in the current design, it is hard to identify if the current picture can be skipped to decode without affecting other pictures.

Other example is trick play (FF/FR). The example of FF (Fast Forward) play is shown in Figure 1.

Figure 1: Example of FF (Fast Forward) play

In AVC and HEVC, it is possible to identify by nal_ref_idc or nal_unit_type (NAL unit layer). In the current VVC design, decoder needs to reconstruct ref_pic_list of the followed pictures. Other way is to use STSA (Step-wise Temporal Sub-layer Access) picture. However, sub-layer must be used and HRD must be guaranteed for base layer and whole bitstreams. It is a burden for encoder.

Proposal

This contribution proposes to include the flag to indicate the current AU is reference picture or not.

Option 1:

One option is to include a flag to in NAL unit header and it is similar way as used in AVC.

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| nal_ref_flag | u(1) |
| nuh_reserved_zero_7bits | u(7) |
| } |  |

Table 1: Proposed syntax of NAL unit header

nal_ref_flag equal to 0 specifies that the content of the NAL unit contains a tile group of a non-reference picture.

Option 2:

Other way is to include a flag in AUD to indicate if the current picture is reference picture or not.

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| ref_pic_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 2: Proposed syntax of AUD

ref_pic_flag equal to 0 specifies that the content of the NAL unit contains a tile group of a non-reference picture.

Option 3:

Other way is to extend pic_type in AUD to indicate if the current picture is reference picture or not. In this case, there is no syntax change. The semantics of pic_type is extended.

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 2: Proposed syntax of AUD

pic_type indicates that the tile_group_type values for all tile groups of the coded pictures in the access unit containing the access unit delimiter NAL unit are members of the set listed in Table 7-2 for the given value of pic_type. The value of pic_type shall be equal to 0, 1, 2, 3, 4 or 5 in bitstreams conforming to this version of this Specification. Other values of pic_type are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore reserved values of pic_type. The value of ref_pic_flag is derived from pic_type as shown in Table 7-2.

Table 7-2 – Interpretation of pic_type

| pic_type | ref_pic_flag | Tiles_group_type values that may be present in the coded picture |
| --- | --- | --- |
| 0 | 0 | I |
| 1 | 1 | I |
| 2 | 0 | P, I |
| 3 | 1 | P, I |
| 4 | 0 | B, P, I |
| 5 | 1 | B, P, I |

Conclusions

In the current VVC design, there is no syntax to indicate whether the current picture is reference picture or non-reference picture. If there is no such syntax element at higher layer of syntax, it is hard to identify if the current picture can be discarded. This contribution proposes to include a flag to identify whether current picture is non-reference picture. Three methods are proposed. One of those methods should be adopted to WD of VVC.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).