| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0137-v1 |
| --- | --- |

| Title: | CE3-2.1: Intra chroma partitioning and prediction restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tianyang Zhou Tomohiro Ikai | Tel: Email: | +81-50-5213-6277 zhou.tianyang@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes partitioning and prediction modifications to alleviate intra chroma throughput issues. This report shows the following two tests:

Test 1(CE3-2.1.1): 2x2/2x4/4x2 restriction.

Test 2(CE3-2.1.2): CE-3.2.1.1 + DC mode only prediction in small chroma block.

The restriction size is 2x2, 4x2 and 2x4 in all cases.

In Test 1, intra chroma block partitioning in chroma dual tree chroma is restricted to address the longer intra processing pipeline in dual tree cases. Specifically, in this proposal, QT and MTT partitioning for small chroma blocks are disallowed in dual tree as a partition restriction.

In Test 2, DC mode only prediction in small chroma block is proposed to address difficulties in software cases.

- Test 1(CE3-2.1.1): the overall BD-rate loss is 0.02%/0.37%/0.49% in IO, 0.01%/0.30%/0.36% in RA, 0.00%/0.18%/ -0.55% in LB.

- Test 2(CE3-2.1.2): the overall BD-rate loss is 0.02%/0.37%/0.49% in IO, 0.10%/0.61%/0.40% in RA, 0.07%/0.66%/ -0.53% in LB.

Introduction

Since the adoption of dual tree in intra picture, it is possible to apply different partitioning in the chroma coding tree compared to the luma coding tree. However the dual tree introduces longer coding pipeline and the VVC 4.0 QTBT MinQTSizeC value range and MinBtSizeY and MinTTSizeY in chroma tree allow small chroma blocks such as 2x2, 4x2, and 2x4. It provides difficulties in practical decoder design.

Moreover, several prediction modes such as CCLM, planar and angular mode needs multiplication. This time we propose DC prediction only, which provides the simplest solutions with no particular coding loss (in 2x2).

Proposal

This contribution proposes three options (Test 1, Test2 and Test3):

|  | Dual tree (used in intra pictures in CTC) | Shared tree (used in inter pictures in CTC) |
| --- | --- | --- |
| Test1 | Disable 2x2/4x2/2x4 chroma blocks in dual tree | No change |
| Test2 | Test1 + DC only prediction in 2x2/4x2/2x4 chroma blocks | DC only prediction in 2x2/4x2/2x4 chroma blocks |

Experimental Results

Table 1 the results of test 1.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.16% | 0.11% | 100% | 100% |
| Class A2 | 0.04% | 0.39% | 0.23% | 99% | 99% |
| Class B | 0.00% | 0.18% | 0.29% | 100% | 100% |
| Class C | 0.05% | 1.06% | 1.56% | 100% | 99% |
| Class E | -0.02% | -0.05% | 0.05% | 100% | 100% |
| Overall | 0.02% | 0.37% | 0.49% | 100% | 100% |
| Class D | 0.12% | 1.80% | 2.48% | 99% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.05% | 100% | 100% |
| Class A2 | -0.01% | 0.20% | 0.16% | 100% | 101% |
| Class B | 0.01% | 0.17% | 0.23% | 101% | 101% |
| Class C | 0.03% | 0.77% | 0.90% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.30% | 0.36% | 101% | 101% |
| Class D | 0.05% | 0.88% | 1.48% | 101% | 101% |
|  | Low delay Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.17% | -0.04% | 100% | 101% |
| Class C | 0.00% | 0.11% | 0.11% | 101% | 101% |
| Class E | 0.01% | 0.86% | 0.53% | 100% | 101% |
| Overall | 0.00% | 0.18% | 0.15% | 100% | 101% |
| Class D | -0.04% | 0.54% | 0.09% | 101% | 101% |

Table 2 the results of test 2.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.16% | 0.11% | 100% | 100% |
| Class A2 | 0.04% | 0.39% | 0.23% | 99% | 99% |
| Class B | 0.00% | 0.18% | 0.29% | 100% | 100% |
| Class C | 0.05% | 1.06% | 1.56% | 100% | 99% |
| Class E | -0.02% | -0.05% | 0.05% | 100% | 101% |
| Overall | 0.02% | 0.37% | 0.49% | 100% | 100% |
| Class D | 0.12% | 1.80% | 2.48% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.07% | 0.06% | 100% | 100% |
| Class A2 | 0.05% | 0.36% | 0.18% | 100% | 101% |
| Class B | 0.05% | 0.46% | 0.25% | 101% | 101% |
| Class C | 0.23% | 1.39% | 1.01% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.61% | 0.40% | 101% | 101% |
| Class D | 0.25% | 2.49% | 1.62% | 101% | 101% |
|  | Low delay Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.04% | 0.14% | 100% | 101% |
| Class C | 0.09% | 1.01% | 1.06% | 101% | 101% |
| Class E | 0.12% | 1.37% | 0.48% | 100% | 101% |
| Overall | 0.07% | 0.66% | 0.53% | 100% | 101% |
| Class D | 0.12% | 1.73% | 1.25% | 101% | 101% |

Conclusion

This contribution proposes to restrict intra chroma block partitioning in dual tree and intra chroma prediction modes in small blocks.

・Test 2.1.1: the overall BD-rate loss is 0.02%/0.37%/0.49% in IO, 0.01%/0.30%/0.36% in RA, 0.00%/0.18%/ -0.55% in LB.

・Test 2.1.2: the overall BD-rate loss is 0.02%/0.37%/0.49% in IO, 0.10%/0.61%/0.40% in RA, 0.07%/0.66%/ -0.53% in LB.

References

[1] VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).