| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0139 |
| --- | --- |

| Title: | CE3-related: Intra Angular Prediction and Modified PDPC Based on Two Reference Lines |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dong Jiang, Jucai Lin, Feiyang Zeng and Cheng Fang | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

This contribution gives two modifications to the intra angular prediction. Firstly, intra angular prediction and PDPC based on two reference lines are proposed. The predicted value is the weighted average of two predicted values generated by two reference lines, and the PDPC value is also the weighted average of two compensated values generated by two reference lines. Secondly, the weights of predicted values and PDPC values are modified. Compared with VTM4.0, under the AI configuration, the average BD-rate gains of the first modification method are 0.04%, -0.02% and 0.03% for luminance and both chrominance components. The average BD-rate gains of the second modification method are 0.02%, 0.00% and 0.02% for luminance and both chrominance components. Combining the two modification methods, the average BD-rate gains are 0.07%, 0.01% and 0.04% for luminance and both chrominance components, while brings negligible increase of encoding and decoding complexity.

Algorithm description

In VTM4.0[1], the luminance component of intra prediction is predicted by multiple reference lines, and the best prediction mode and the corresponding reference line are selected by rdcost function. The best predicted mode is selected from 0 to 66, and the best reference line is selected from Reference Line 0, Reference Line 1 and Reference Line 3, as shown in Figure 1. Intra angular mode PDPC compensation is only used when the predicted mode is greater than 57 or less than 11, and only the pixel value of the reference line 0 is used to compensate.

Figure 1: Multiple Reference Lines in VTM4.0

Based on the existing intra angular mode prediction technology, two modification methods are proposed, as explained in JVET-M0047. The first is an intra angular mode prediction and PDPC compensation method based on two reference lines. As shown in Figure 2, the method has the following settings:

(1) In the angular prediction, when the reference samples from Reference Line 0 are used to predict, the predicted values are weighted by reference samples from Reference Line 1 and Reference Line 0, when the reference samples from Reference Line 1 and Reference Line 3 are used to predict, the predicted values are the same as VTM4.0. The PDPC compensation uses the weights of reference samples from Reference Line 1 and Reference Line 0 according to the PDPC direction to get the PDPC compensation values.

(2) When the angular prediction and the PDPC compensation are performed, the weights of Reference Line 1 and Reference Line 0 are set to 1:1.

(3) In the angular prediction, the predicted position of current prediction direction on Reference Line 1 is obtained according to the predicted position on Reference Line 0, that is, the above (or left) side of the reference sample on Reference Line 0 is selected.

(4) In the PDPC compensation, the compensated position of current PDPC direction on Reference Line1 is obtained according to the angular prediction mode.

(5) This method is only used for blocks larger than or equal to 16*16.

(6) This method is only used for the luminance component.

Figure 2: Intra Angular Prediction and PDPC Compensation

Based on Two Reference Lines, (a) and (b) Show the Range and Direction of Prediction Mode

The second is a modified PDPC method for the angular mode, except for diagonal mode (mode 2 and 66).This method mainly modifies the weight of the PDPC compensation value. In VTM4.0, the range of the predicted pixel of PDPC compensation is limited by the width and height of the current prediction block, and the range is very small. At the same time, the weight of PDPC compensation value is reduced step by step from 0.5 to 0. The mathematical relationship is as follows:

shift = ( log2( width ) – 2 + log2( height ) – 2 + 2 ) >> 2

If the PDPC compensation pixels are on the left, the PDPC compensation weights are as follows:

wL= 32 >> min(31, ( ( x<<1 ) >> shift ))

If the PDPC compensation pixels are on the top, the PDPC compensation weights are as follows:

wT= 32 >> min(31, ( ( y<<1 ) >> shift ))

Where width and height represent the width and height of the current prediction block, x and y are the coordinates of the pixels in the prediction block.

The modified method enlarges the area which applies the PDPC compensation. According to the angular prediction mode, the area of pixels applying the PDPC compensation is set to left-half of the block or above-half of the block. At the same time, the weight of PDPC compensation value is changed to nearly linearly descending from 0.5 to 0.

If the PDPC compensation pixels are on the left, the PDPC compensation weights are as follows:

wL= max(0, 32 – (( x<<6 ) >> log2( width ))

If the PDPC compensation pixels are on the top, the PDPC compensation weights are as follows:

wT= max(0, 32 – (( y<<6 ) >> log2( height ))

Where width and height represent the width and height of the current prediction block, x and y are the coordinates of the pixels in the prediction block.

In addition, the method is also used for blocks larger than or equal to 16*16, and this method is only used for the luminance component.

Experimental results

The proposed method is implemented on top of VTM4.0 software. The simulations are performed on Linux cluster using VTM AI configurations under the common test condition described in JVET-M1010 [2].

Table 1, 2, and 3 show experimental results of the first modification method, the second modification method, and combining the two modification methods compared with VTM4.0, respectively. The experimental setting of the combined method and first modification method are the same.

Table 1 Coding performance of 1st modification over VTM-4.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | 0.06% | -0.03% | 106% | 105% |
| Class A2 | -0.06% | 0.14% | -0.02% | 105% | 106% |
| Class B | -0.05% | 0.00% | -0.04% | 105% | 106% |
| Class C | 0.00% | -0.10% | -0.07% | 104% | 107% |
| Class E | -0.02% | 0.03% | 0.03% | 105% | 106% |
| Overall | -0.04% | 0.02% | -0.03% | 105% | 106% |
| Class D | -0.01% | -0.06% | -0.08% | 104% | 108% |
| Class F | 0.00% | 0.03% | -0.05% | 104% | 107% |

Table 2 Coding performance of 2t modification over VTM-4.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.00% | -0.08% | 100% | 100% |
| Class A2 | -0.02% | 0.10% | -0.08% | 100% | 100% |
| Class B | -0.01% | 0.01% | 0.03% | 100% | 100% |
| Class C | 0.01% | -0.06% | -0.03% | 100% | 100% |
| Class E | 0.01% | -0.05% | 0.00% | 100% | 100% |
| Overall | -0.02% | 0.00% | -0.02% | 100% | 100% |
| Class D | -0.01% | -0.09% | -0.05% | 101% | 100% |
| Class F | 0.00% | 0.05% | 0.10% | 100% | 100% |

Table 3: Coding performance of 1st + 2nd modifications over VTM-4.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.07% | -0.08% | 106% | 105% |
| Class A2 | -0.07% | 0.06% | -0.06% | 105% | 106% |
| Class B | -0.06% | -0.02% | -0.02% | 105% | 106% |
| Class C | 0.00% | -0.03% | -0.04% | 104% | 107% |
| Class E | -0.03% | -0.01% | -0.01% | 105% | 106% |
| Overall | -0.07% | -0.01% | -0.04% | 105% | 106% |
| Class D | -0.02% | -0.04% | -0.03% | 104% | 107% |
| Class F | 0.01% | -0.01% | 0.02% | 105% | 107% |

Conclusion

In this contribution, two modifications are proposed in intra angular prediction part. Combining the two methods can achieve 0.07%, 0.01% and 0.04% average BD-rate gains for luminance and both chrominance components with negligible increasing of encoding and decoding complexity on top of VTM4.0 under the AI configuration. It is proposed to adopt this proposal into VTM.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Dahua may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).