| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0140-v1 |
| --- | --- |

| Title: | CE3-2.1.3: Combination of CE3-2.1.1 and CE3-2.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tianyang Zhou, Tomohiro Ikai Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei | Tel: Email: | zhou.tianyang@sharp.co.jp ikai.tomohiro@sharp.co.jp {zhi-yi.lin, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | Sharp Corporation and MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution reports a result of CE3-2.1.3 (a combination of CE3-2.1.1 and CE3-2.2). It is reported that the overall BD-rate loss is 0.02 % / 0.37 % / 0.49 % in IO, 0.06 % / 0.46 % / 0.62 % in RA, 0.04 %/ 0.44 % / 0.42 % in LDB.

It is noted that the result of CE3-2.1.1 (dual tree restriction) and CE3-2.1.3 (a combination of dual tree restriction and parallel prediction) is the same in IO configuration.

Proposed Methods

Intra chroma partitioning restriction (CE3-2.1.1)

In this test, QT and MTT partitioning for small chroma blocks are disallowed in dual tree as a partition restriction.

Shared reference samples for multiple chroma intra CBs (CE3-2.2)

In this test, a method to enable parallel generation of multiple chroma intra coding blocks (CBs) within a parallel region of size larger than or equal to 16 chroma samples.

Combination test description

|  | Dual tree (used in intra pictures in CTC) | Shared tree (used in inter pictures in CTC) |
| --- | --- | --- |
| Combination test | Disable 2x2/4x2/2x4 chroma blocks in dual tree | Share reference samples for at least 16 chroma samples (2x2/4x2/2x4 chroma blocks) |

Experimental Results

Table 1 test results.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.16% | 0.11% | 101% | 101% |
| Class A2 | 0.04% | 0.39% | 0.23% | 99% | 100% |
| Class B | 0.00% | 0.18% | 0.29% | 101% | 101% |
| Class C | 0.05% | 1.06% | 1.56% | 101% | 100% |
| Class E | -0.02% | -0.05% | 0.05% | 101% | 102% |
| Overall | 0.02% | 0.37% | 0.49% | 101% | 101% |
| Class D | 0.12% | 1.80% | 2.48% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.04% | 0.25% | 101% | 101% |
| Class A2 | 0.01% | 0.23% | 0.17% | 101% | 101% |
| Class B | 0.02% | 0.40% | 0.51% | 101% | 101% |
| Class C | 0.15% | 1.01% | 1.38% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.46% | 0.62% | 101% | 101% |
| Class D | 0.17% | 1.43% | 2.11% | 102% | 101% |
|  | Low delay Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.02% | -0.14% | 101% | 101% |
| Class C | 0.02% | 0.69% | 0.88% | 101% | 101% |
| Class E | 0.13% | 0.87% | 0.75% | 100% | 101% |
| Overall | 0.04% | 0.44% | 0.42% | 101% | 101% |
| Class D | 0.08% | 1.07% | 0.27% | 102% | 101% |

Conclusion

This contribution reports the results of CE3-2.1.3(a combination of CE3-2.1.1 and CE3-2.2). The overall BD-rate loss is 0.02 % / 0.37 % / 0.49 % in IO, 0.06 % / 0.46 % / 0.62 % in RA, 0.04 % / 0.44 % / 0.42 % in LDB.

References

[1] VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

Patent rights declaration(s)

Sharp Corporation and MediaTek Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).