| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0142 |
| --- | --- |

| Title: | CE4-3.4: Diagonal direction candidate for MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto Eiichi Sasaki Tomohiro Ikai | Email: | +81-43-299-8526 tomonori.hashimoto@sharp.co.jp eiichi.sasaki@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution reports a result of CE4-3.4 diagonal direction candidate for MMVD. In CE4-3.4, two tests are conducted. Test 4-3.4.a uses diagonal direction candidate and test 4-3.4.b is a combination of diagonal direction candidate and MMVD syntax change in CE4-1.1. In test 4-3.4.a, the bd-rate is -0.07 % with 100 % encoding time under RA condition and -0.03 % with 100 % encoding time under LDB condition. In test 4-3.4.b, the bd-rate is -0.11 % with 100 % encoding time under RA condition and -0.08 % with 101 % encoding time under LDB condition. It is proposed to adopt test 4-3.4.b method into the next VVC draft / VTM.

Introduction

In merge with MVD (MMVD) [1], starting point index (index of merge candidates), motion magnitude, and motion direction are signaled. Although MMVD shows significant coding gain, it is asserted that the motion direction would not be sufficient in terms of coding efficiency.

This contribution reports a result of CE4-3.4, which tests the performance of diagonal direction candidate for MMVD. This CE also tests the performance of the combination of diagonal direction candidate for MMVD and MMVD syntax change in CE4-1.1 [2].

Proposed Methods

Diagonal direction candidates for MMVD

Diagonal direction candidates are used in addition to horizontal / vertical candidates of MMVD. The diagonal direction candidates are shown as below.

Table 1. Motion direction

| Direction IDX | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-axis | +1 | –1 | 0 | 0 | +1/2 | -1/2 | -1/2 | +1/2 |
| y-axis | 0 | 0 | +1 | –1 | +1/2 | -1/2 | +1/2 | -1/2 |

Note: the value of x-axis and y-axis of diagonal direction are half of that of horizontal and vertical direction respectively.

Fast algorithm

To reduce the fixed number of the search candidates without loss, a fast algorithm like “Hexagon Search” is applied. In this algorithm, the searching for directions is nested in the searching for distances. At first, all directions (8 directions) are searched with the smallest distance. After this first searching loop, only 3 directions, that are the best direction in previous distance and 2 neighbors, are searched. By using this algorithm, the number of search candidates in each base MV is equal to 8 + 3 * 7 = 29.

Experimental Results

The simulations were conducted according to JVET-M1024 [3]. The following Table 2 shows the results.

4-3.4.a: Diagonal direction candidate for MMVD

4-3.4.b: 4-3.4.a + 4-1.1

Table 2. All results CE4-3.4

4-3.4.a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.04% | 0.16% | 99% | 100% |
| Class A2 | -0.17% | -0.09% | -0.17% | 99% | 100% |
| Class B | -0.04% | 0.06% | 0.08% | 100% | 100% |
| Class C | -0.04% | -0.02% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | 0.00% | 0.03% | 100% | 100% |
| Class D | -0.11% | -0.02% | -0.11% | 100% | 100% |
| Class F | 0.03% | 0.10% | 0.07% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | 0.04% | 0.36% | 100% | 100% |
| Class C | -0.02% | 0.45% | 0.16% | 100% | 100% |
| Class E | -0.02% | 0.58% | 0.17% | 100% | 100% |
| Overall | -0.03% | 0.31% | 0.25% | 100% | 100% |
| Class D | -0.08% | -0.08% | -0.58% | 100% | 99% |
| Class F | 0.02% | 0.39% | -0.05% | 101% | 100% |

4-3.4.b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.06% | 0.34% | 99% | 100% |
| Class A2 | -0.22% | 0.06% | 0.06% | 99% | 100% |
| Class B | -0.06% | 0.03% | 0.10% | 100% | 101% |
| Class C | -0.11% | -0.06% | 0.01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | 0.02% | 0.12% | 100% | 101% |
| Class D | -0.04% | 0.09% | 0.25% | 100% | 101% |
| Class F | -0.04% | 0.09% | 0.03% | 99% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.14% | 0.20% | -0.12% | 101% | 100% |
| Class C | -0.06% | 0.22% | -0.01% | 100% | 100% |
| Class E | 0.00% | -0.13% | -0.37% | 101% | 101% |
| Overall | -0.08% | 0.13% | -0.14% | 101% | 100% |
| Class D | -0.02% | 0.56% | -0.19% | 101% | 100% |
| Class F | -0.04% | 0.35% | -0.01% | 101% | 100% |

Conclusion

This contribution reports a result of CE4-3.4 diagonal direction candidate for MMVD. It is proposed to adopt test 4-3.4.b method into the next VVC draft / VTM.

References

[1] S. Jeong, M. W. Park, Y. Piao, M. Park, K. Choi, “CE4 Ultimate motion vector expression (Test 4.5.4),” JVET-L0054, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

[2] E. Sasaki, T. Chujoh, T. Ikai, “Non-CE4: Syntax change of MMVD,” JVET-M0069, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

[3] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-M1024, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).