| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0147-v1 |
| --- | --- |

| Title: | CE9-related: Simplification of BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-43-299-8526 chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

BDOF (bi-directional optical flow) is a method to improve bi-directional prediction efficiency by using gradient information and 4x4 predicted pixels are generated by using 6x6 pixels in which one pixel surrounding 4x4 are overlapped. In this contribution, a simplification by changing from 6x6 to 4x4 is proposed. The less than half of processing for BDOF can be achieved by this simplification. As a result of experiments on CTC, the average of encoding time became 1% faster and the average of decoding time became 2% faster. The loss of coding efficiency was 0.06% on average.

Introduction

BDOF (bi-directional optical flow) [1] improves bi-directional prediction efficiency by using gradient information and 4x4 predicted pixels are generated by using 6x6 pixels in which one pixel surrounding 4x4 are overlapped. In this contribution, a simplification by changing from 6x6 to 4x4 is proposed. Since the overlapped pixels are removed, the padding process of prediction pixels is removed and the less than half of processing for BDOF is achieved. The experiment has been shown to reduce the encoding and decoding time with negligible loss of coding efficiency by this simplification.

Proposal changes of specification

The working draft [2] should be changed by the followings.

- 8.5.7.4	Bidirectional optical flow prediction process

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW + 2)x(nCbH + 2) luma prediction sample arrays predSamplesL0 and predSamplesL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the reference indices refIdxL0 and refIdxL1,

- the bidirectional optical flow utilization flags bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( nCbW >> 2 ) − 1, yIdx = 0..( nCbH >> 2 ) − 1.

Output of this process is the (nCbW)x(nCbH) array pbSamples of luma prediction sample values.

Variables bitDepth, shift1, shift2, shift3, shift4, offset4, and mvRefineThres are derived as follows:

- The variable bitDepth is set equal to BitDepthY.

- The variable shift1 is set to equal to Max( 2, 14 − bitDepth ).

- The variable shift2 is set to equal to Max( 7, bitDepth − 5 ).

- The variable shift3 is set to equal to Max( 4, bitDepth − 8 ).

- The variable shift4 is set equal to Max( 3, 15 − bitDepth ) and the variable offset4 is set equal to 1  <<  ( shift4 − 1 ).

- The variable mvRefineThres is set equal to Max( 2, 1  <<  ( 13 − bitDepth ) ).

For xIdx = 0..( nCbW >> 2 ) − 1 and yIdx = 0..( nCbH >> 2 ) − 1, the following applies:

- The variable xSb is set equal to ( xIdx << 2) + 1 and ySb is set equal to ( yIdx << 2 ) + 1.

- If bdofUtilizationFlag[ xSbIdx ][ yIdx ] is equal to FALSE, for x = xSb − 1..xSb + 2, y = ySb − 1.. ySb + 2, the prediction sample values of the current subblock are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset2 +	(8-852)
	predSamplesL1[ x + 1 ][ y + 1 ] )  >>  shift2 )

- Otherwise (bdofUtilizationFlag[ xSbIdx ][ yIdx ] is equal to TRUE), the prediction sample values of the current subblock are derived as follows:

  - For x =xSb..xSb + 3, y = ySb..ySb + 3, the following ordered steps apply:

- The locations ( hx, vy ) for each of the corresponding sample locations ( x, y ) inside the prediction sample arrays are derived as follows:

hx = Clip3( 1, nCbW, x )		(8-853)

vy = Clip3( 1, nCbH, y )		(8-854)

- The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientHL0[ x ][ y ]  =  (predSamplesL0[ x + 1 ][ y ] − predSampleL0[ y − 1 ][ y ] ) >> shift1	(8-855)

gradientVL0[ x ][ y ]  =   (predSampleL0[ x ][ y + 1 ] − predSampleL0[ x ][y − 1 ] ) >> shift1	(8-856)

gradientHL1[ x ][ y ]  =  (predSamplesL1[ x + 1 ][y] − predSampleL1[ x − 1 ][ y ] ) >> shift1	(8-857)

gradientVL1[ x ][ y ]  =   (predSampleL1[ x ][ y + 1 ] − predSampleL1[ x ][y − 1 ] ) >> shift1	(8-858)

- The variables temp[ x ][ y ], tempH[ x ][ y ] and tempV[ x ][ y ] are derived as follows:

diff[ x ][ y ] = (predSamplesL0[ x ][ y ] >> shift2 ) − ( predSamplesL1[ x ][ y ] >> shift2 )	(8-859)

tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] ) >> shift3	(8-860)

tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] ) >> shift3	(8-861)

  - The variables sGx2, sGy2, sGxGy, sGxdI and sGydI are derived as follows:

sGx2 = ij ( tempH[ xSb + i ][ ySb + j ] * tempH[ xSb + i ][ ySb + j ] ) with i, j = 0..3	(8-862)

sGy2 = ij(tempV[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j = 0..3	(8-863)

sGxGy = ij(tempH[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j  0..3	(8-864)

sGxdI = ij( − tempH[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = 0..3	(8-865)

sGydI = ij( − tempV[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = 0..3	(8-866)

Experimental results

The proposed method has been implemented on VTM-4.0 software. The simulation was based on VTM configuration JVET-M1010 [3]. The PC cluster was used for the simulation, but since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

As summary of experimental results, the average of encoding time became 1% faster and the average of decoding time became 2% faster. The loss of coding efficiency was 0.06% on average.

Table 1  Summary of experimental results on CTC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.05% | -0.02% | 99% | 98% |
| Class A2 | 0.08% | 0.04% | 0.02% | 99% | 98% |
| Class B | 0.08% | 0.07% | 0.03% | 100% | 98% |
| Class C | 0.05% | -0.01% | -0.08% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.04% | -0.01% | 99% | 98% |
| Class D | 0.01% | -0.05% | -0.06% | 99% | 97% |
| Class F | 0.00% | -0.03% | -0.02% | 99% | 99% |

Conclusion

In this contribution, a simplification of BDOF has been proposed. By changing from 6x6 to 4x4, it can be realized with less than half of the processing for BDOF. Finally, we would like to recommend that the working draft and reference software should support this specification.

Reference

[1]	X. Xiu, Y. He and Y. Ye, CE9-related: Complexity reduction and bit-width control for bi-directional optical flow (BIO), Joint Video Experts Team Document, JVET-L0256, Macao, Oct. 2018.

[2]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 4), Joint Video Experts Team Document, JVET-M1001-v4, Marrakesh, Mar. 2019.

[3]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-M1010, Marrakesh, Jan. 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).