| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0149 |
| --- | --- |

| Title: | AHG12: Signalling on wavefront parallel processing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Sachin Deshpande Eiichi Sasaki Takeshi Chujoh Tomoko Aono Yukinobu Yasugi | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp yasugi.yukinobu@sharp.co.jp +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Corporation, Sharp Labs of America, Inc. |  |  |

_____________________________

- Abstract

A wavefront parallel processing design in a tile group has been proposed in the JVET-M0070 where multiple tiles or wavefront parallel processing is exclusively allowed in a tile group. This proposal extends the design for rectangular tile group which specifies the tile in picture parameter set level.

In this proposal, a entropy_coding_sync_enabled_flag is signalled in picture parameter set for each rectangular tile group or signalled in a tile group. A constraint that entropy_coding_sync_enabled_flag can be enabled when tile group has only one tile (i.e. multiple tiles or wavefront processing is exclusively allowed) is applied as in JVET-M0070.

Introduction

At the last meeting discussion in HLG BoG, wavefront parallel processing is considered to be useful in VVC and the design of WPP is decided to be studied in AHG12.

In VVC 4, tile group can be specified either in PPS or tile group header. Thus it would be straight forward that entropy_coding_sync_enabled_flag is signalled in PPS (if tile structure is signalled in PPS) and is signalled in tile group header (if tile structure is signalled in tile group header). And as mentioned at the last meeting, the co-existence of multiple tiles in tile group and WPP doesn’t provide additional benefit while it arises implementation complexity. Thus we propose the exclusive usage as in JVET-M0070

Proposal

We propose a HEVC type wavefront processing where decoding process is basically the same as HEVC.

Since tile group signalling can be carried in PPS and tile group header, the entropy_coding_sync_enabled_flag which enables the wavefront (CTU row) parallel processing is signalled either in PPS or tile group header.

In PPS, a pps_entropy_coding_sync_enabled_flag per tile group is signalled when single_tile_in_pic_flag or equal to 1 or rect_tile_group_flag is equal to 1.

In tile group header, a tile_group_entropy_coding_sync_enabled_flag is signalled when single_tile_in_pic_flag and equal to 0 and rect_tile_group_flag is equal to 0.

In addition, multiple tiles or wavefront parallel processing is exclusively allowed. Figure 1 shows an example of tiles and CTU row partitioning. As the figure illustrates, tile group 0 and tile group 3 use multiple tiles while tile group 2 uses WPP. It is also allowed that a tile group has single tile without WPP as Tile 4 in tile group 1.

TileGr0

TileGr1

21

Tile

0

CTUrow0

CTUrow1

CTUrow2

CTUrow3

TileGr2

3

TileGr3

Tile

4

Tile

5

3

Tile

1

Tile

2

CTUrow4

CTUrow5

Tile

3

Figure 1: Example of tile group structure

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| single_tile_per_tile_group_flag | u(1) |
| if(!single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| if (single_tile_in_pic_flag ¦¦ rect_tile_group_flag) |  |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if (NumTilesInTileGroup[ i ] == 1) |  |
| pps_entropy_coding_sync_enabled_flag[ i ] | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

pps_entropy_coding_sync_enabled_flag[ i ] equal to 1 specifies that a specific synchronization process for context variables, and when applicable, Rice parameter initialization states variables, is invoked before decoding the CTU which includes the first CTB of a row of CTBs in i-th tile group in each picture referring to the PPS. When not present, pps_entropy_coding_sync_enabled_flag[ i ] is inferred to be equal to 0.

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| tile_group_address | u(v) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tiles_in_tile_group_minus1 | ue(v) |
| if (num_tiles_in_tile_group_minus1 == 0) |  |
| tile_group_entropy_coding_sync_enabled_flag | u(1) |
| } |  |
| tile_group_type | ue(v) |
| if ( tile_group_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus2 | u(3) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| NumTilesInCurrTileGroup = rect_tile_group_flag ? NumTilesInTileGroup[ tile_group_address] : ( num_tiles_in_tile_group_minus1 + 1) |  |
| NumEntryPoint = tile_group_entropy_coding_sync_enabled_flag ? TileGroupHeightInCtbsY-1 : NumTilesInCurrTileGroup - 1 |  |
| if(NumEntryPoint > 0) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoint; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

tile_group_entropy_coding_sync_enabled_flag equal to 1 specifies that a specific synchronization process for context variables, and when applicable, Rice parameter initialization states variables, is invoked before decoding the CTU which includes the first CTB of a row of CTBs in each tile group in each picture referring to the PPS, and a specific storage process for context variables, and when applicable, Rice parameter initialization states and palette predictor variables, is invoked after decoding the CTU which includes the second CTB of a row of CTBs in each tile group in each picture referring to the PPS. tile_group_entropy_coding_sync_enabled_flag equal to 0 specifies that no specific synchronization process for context variables, and when applicable, Rice parameter initialization states and palette predictor variables, is required to be invoked before decoding the CTU which includes the first CTB of a row of CTBs in each tile group in each picture referring to the PPS, and no specific storage process for context variables, and when applicable, Rice parameter initialization states and palette predictor variables, is required to be invoked after decoding the CTU which includes the second CTB of a row of CTBs in each tile group in each picture referring to the PPS.

When not present, tile_group_entropy_coding_sync_enabled_flag is inferred to be equal to 0 if single_tile_in_pic_flag is equal to 0 and rect_tile_group_flag is equal to 0.

When not present, tile_group_entropy_coding_sync_enabled_flag is inferred to be equal to pps_entropy_coding_sync_enabled_flag[ tileGroupIdx ] if single_tile_in_pic_flag is equal to 1 or rect_tile_group_flag is equal to 1.

The variable TileGroupHeightInCtbsY is derived as the following process.

tbX = CtbAddrInRs % PicWidthInCtbsY

tbY = CtbAddrInRs / PicWidthInCtbsY

for(j=0; j<=num_tile_rows_minus1; j++)

if(tbY>=RowBd[j])

tileY = j

TileGroupHeightInCtbsY = RowHeight[tileY]

| tile_group_data( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < NumTilesInCurrTileGroup; i++, tileIdx++ ) { |  |
| ctbAddrInTs = FirstCtbAddrTs[ TgTileIdx[ i ] ] |  |
| for( j = 0; j < NumCtusInTile[ TgTileIdx[ i ] ]; j++, ctbAddrInTs++ ) { |  |
| if( ( j % ColWidth[ TgTileIdx[ i ] ] ) = = 0 ) { |  |
| NumHmvpCand = 0 |  |
| NumHmvpIbcCand = 0 |  |
| } |  |
| CtbAddrInRs = CtbAddrTsToRs[ ctbAddrInTs ] |  |
| coding_tree_unit( ) |  |
| if(tile_group_entropy_coding_sync_enabled_flag && ( (j+1) % ColWidth[ TgTileIdx[ i ] % (num_tile_columns_minus1 + 1) ] = = 0 ) ) { |  |
| end_of_subset_one_bit /* equal to 1 */ | ae(v) |
| if( j<NumCtusInTile[ TgTileIdx[ i ] ]-1 ) |  |
| byte_alignment( ) |  |
| } |  |
| if( ! tile_group_entropy_coding_sync_enabled_flag ) { |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| if( i < NumTilesInCurrTileGroup − 1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |
| } |  |

References

[1] T. Ikai, S. Deshpande, T. Chujoh, E. Sasaki, T. Aono, “AHG12: Wavefront processing in a tile group”, JVET-M0070

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Labs of America, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).