| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0149_r2 |
| --- | --- |

| Title: | AHG12: Signalling on wavefront parallel processing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Sachin Deshpande Eiichi Sasaki Takeshi Chujoh Tomoko Aono Yukinobu Yasugi | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp yasugi.yukinobu@sharp.co.jp +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Corporation, Sharp Labs of America, Inc. |  |  |

_____________________________

- Abstract

A wavefront parallel processing design in a tile group has been proposed in the JVET-M0070 where multiple tiles or wavefront parallel processing is exclusively allowed in a tile group. This proposal extends the design for rectangular tile group which specifies the tile in picture parameter set level.

In this proposal, a entropy_coding_sync_enabled_flag is signalled in picture parameter set for each rectangular tile group or signalled in a tile group. A constraint that entropy_coding_sync_enabled_flag can be enabled when tile group has only one tile (i.e. multiple tiles or wavefront processing is exclusively allowed) is applied as in JVET-M0070.

Revision2 modified the proposal to reflect Coded Picture Region BoG (chaired by M. M. Hannuksela, 24th May, 2019 morning)

- No syntax restrictions on simultaneous use of WPP and several tiles per tile group.

- No changes in tile group header (i.e. when raster-scan-order tile groups are in use, picture-level flag in PPS applies).

- No inference of the number of entry point offsets (i.e., explicit signalling of num_entry_point_offsets in tile group header when WPP is enabled). If entry points are signalled, then entry points for all CTU rows shall be signalled.

Introduction

At the last meeting discussion in HLG BoG, wavefront parallel processing is considered to be useful in VVC and the design of WPP is decided to be studied in AHG12.

In VVC 4, tile group can be specified either in PPS or tile group header. Thus it would be straight forward that entropy_coding_sync_enabled_flag is signalled in PPS (if tile structure is signalled in PPS) and is signalled in tile group header (if tile structure is signalled in tile group header). And as mentioned at the last meeting, the co-existence of multiple tiles in tile group and WPP doesn’t provide additional benefit while it arises implementation complexity. Thus we propose the exclusive usage as in JVET-M0070

Proposal

We propose a HEVC type wavefront processing where decoding process is basically the same as HEVC.

Since tile group signalling can be carried in PPS and tile group header, the entropy_coding_sync_enabled_flag which enables the wavefront (CTU row) parallel processing is signalled either in PPS or tile group header.

In PPS, a pps_entropy_coding_sync_enabled_flag per tile group is signalled when single_tile_in_pic_flag or equal to 1 or rect_tile_group_flag is equal to 1.

In tile group header, a tile_group_entropy_coding_sync_enabled_flag is signalled when single_tile_in_pic_flag and equal to 0 and rect_tile_group_flag is equal to 0.

In addition, multiple tiles or wavefront parallel processing is exclusively allowed. Figure 1 shows an example of tiles and CTU row partitioning. As the figure illustrates, tile group 0 and tile group 3 use multiple tiles while tile group 2 uses WPP. It is also allowed that a tile group has single tile without WPP as Tile 4 in tile group 1.

TileGr0

TileGr1

21

Tile

0

CTUrow0

CTUrow1

CTUrow2

CTUrow3

TileGr2

3

TileGr3

Tile

4

Tile

5

3

Tile

1

Tile

2

CTUrow4

CTUrow5

Tile

3

Figure 1: Example of tile group structure

Note: revision 2 changes are not refrected in the above. Thus some of them is obsolete.

Revised summary of text chages

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| single_tile_per_tile_group_flag | u(1) |
| if(!single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| if (single_tile_in_pic_flag ¦¦ rect_tile_group_flag) |  |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| entropy_coding_sync_enabled_flag[ i ] | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

entropy_coding_sync_enabled_flag[ i ] equal to 1 specifies that a specific synchronization process for context variables is invoked before decoding the CTU which includes the first CTB of a row of CTBs in each tile in i-th tile group in each picture referring to the PPS. When not present, entropy_coding_sync_enabled_flag[ i ] is inferred to be equal to 0.

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| tile_group_address | u(v) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| tile_group_type | ue(v) |
| if ( tile_group_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus2 | u(3) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| NumTilesInCurrTileGroup = rect_tile_group_flag ? NumTilesInTileGroup[ tile_group_address] : ( num_tiles_in_tile_group_minus1 + 1) |  |
| if ( entropy_coding_sync_enabled_flag[ tileGroupIdx ] ) |  |
| num_entry_point_offsets | ue(v) |
| if(NumEntryPoint > 0) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoint; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

num_entry_point_offsets specifies the number of entry_point_offset_minus1[ i ] syntax elements in the tile group header. When not present, the value of num_entry_point_offsets is inferred to be equal to 0.

The variable NumEntryPoint, which specifies the number of entry point in the current tile group, is derived as follows:

NumEntryPoint = entropy_coding_sync_enabled_flag[ tileGroupIdx ] ? num_entry_point_offsets : NumTilesInCurrTileGroup − 1

It is a requirement of bitstream conformance that the value of num_entry_point_offsets shall be either 0 or the number of CTU rows minus 1 in the tile group

| tile_group_data( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < NumTilesInCurrTileGroup; i++, tileIdx++ ) { |  |
| ctbAddrInTs = FirstCtbAddrTs[ TgTileIdx[ i ] ] |  |
| for( j = 0; j < NumCtusInTile[ TgTileIdx[ i ] ]; j++, ctbAddrInTs++ ) { |  |
| if( ( j % ColWidth[ TgTileIdx[ i ] ] ) = = 0 ) { |  |
| NumHmvpCand = 0 |  |
| NumHmvpIbcCand = 0 |  |
| } |  |
| CtbAddrInRs = CtbAddrTsToRs[ ctbAddrInTs ] |  |
| coding_tree_unit( ) |  |
| if(entropy_coding_sync_enabled_flag[ tileGroupIdx ] && ( (j+1) % ColWidth[ TgTileIdx[ i ] % (num_tile_columns_minus1 + 1) ] = = 0 ) ) { |  |
| end_of_subset_one_bit /* equal to 1 */ | ae(v) |
| if( j<NumCtusInTile[ TgTileIdx[ i ] ]-1 ) |  |
| byte_alignment( ) |  |
| } |  |
| if( ! entropy_coding_sync_enabled_flag[ tileGroupIdx ] ) { |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| if( i < NumTilesInCurrTileGroup − 1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |
| } |  |

References

[1] T. Ikai, S. Deshpande, T. Chujoh, E. Sasaki, T. Aono, “AHG12: Wavefront processing in a tile group”, JVET-M0070

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Labs of America, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).