| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0151 |
| --- | --- |

| Title: | CE3-related: Modified Chroma Derived Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feiyang Zeng Dong Jiang Jucai Lin Cheng Fang | Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com zeng_feiyang@dahuatech.com fang_cheng1@dahuatech.com |
| Source: | Dahua(ZHEJIANG DAHUA TECHNOLOGY CO.,LTD) |  |  |

_____________________________

- Abstract

This contribution proposes a method to get the derived Mode (DM) by counting the intra prediction modes of the corresponding luma blocks. The simulation results show 0.01%, 0.02% coding gain in terms of average BD-rate of Cb and Cr in all intra configuration with the same encoder complexity.

Introduction

The chroma intra mode candidate list in VTM-4.0[1] includes default modes, cross-component linear mode (CCLM) and DM. The DM is gotten from the intra prediction mode of the coded luma block which is covering the CR position of the corresponding luma block, shown in Figure 1. If the DM is the same as the mode in default modes, the mode in default modes may be replaced by vdiagonal mode.

Figure 1. DM derived from CR position of the corresponding luma block

Algorithm description

With the separate tree method, a chroma block can correspond to multiple luma blocks. The proposal counts 5 intra prediction modes of the coded luma blocks which are covering 5 positions of the corresponding luma block, as explained in JVET-M0048. The 5 positions are CR, TL1, TR1, BL1 and BR1, shown in Figure 2. TL1 is the center point of the rectangle where TL is the top left point and CR is the bottom right point, the positions of TR1, BL1and BR1 are analogous. If mode M is the mode that occurs most frequently among the 5 intra prediction modes, and its occurrence number is greater than 2, select M as the DM. Otherwise, the DM would be the same as the intra prediction mode of the luma coding block which is covering the CR position of the corresponding luma block.

Figure 2. Proposed positions for DM derivation from the corresponding luma block

Experimental results

The proposed modification is implemented on VTM-4.0 reference software. The simulations are performed on Linux cluster using VTM AI configurations under the common test condition described in JVET-M1010 [2].

Table 1. Performance of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.11% | 0.02% | 100% | 101% |
| Class A2 | -0.01% | 0.05% | -0.02% | 100% | 101% |
| Class B | -0.01% | -0.01% | 0.02% | 100% | 100% |
| Class C | 0.00% | -0.06% | -0.04% | 100% | 101% |
| Class E | -0.01% | -0.09% | -0.11% | 100% | 101% |
| Overall | 0.00% | -0.01% | -0.02% | 100% | 101% |
| Class D | 0.00% | -0.03% | -0.14% | 100% | 101% |
| Class F | 0.01% | 0.11% | 0.02% | 100% | 101% |

Conclusion

The DM modification is implemented on VTM-4.0 reference software. The experimental results reportedly show that under AI configuration , it can achieve 0.01% , 0.02% coding gain in terms of the average BD-rate of Cb and Cr with 100% enc-time.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Dahua may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).