| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0159 |
| --- | --- |

| Title: | Non-CE3: Relaxation method of processing dependency in ISP for small blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyohei Unno Kei Kawamura Yoshitaka Kidani Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

In this contribution, it is proposed that a relaxation method of processing dependency for Intra Sub-Partitions (ISP) with small brock. Here, small block means a block whose sub-partitions width or height will be less than 4 pixels when ISP applied to. The proposed method changes processing unit of prediction from sub-partitions to CU for small blocks. According to the proposed method, all of processing units of intra prediction have at least 4 pixels for width and height in spite of whether ISP is applied or not. Additionally, transformation can be processed in parallel for a small block in the proposed method. Experimental results show that BD-rate for luma is 0.10%/0.04% (AI/RA) in comparison with VTM-4.0.

Introduction

Intra Sub-Partitions (ISP) was adopted to VVC Draft 4 [1] and VTM at the 13th meeting in Marrakech. There is a problem for hardware implementation aspects in ISP. Especially, it is pointed difficulty of hardware implementation in case of sub-partition width less than 4 pixels. In this contribution, it is proposed that a relaxation method of processing dependency with such a small block. The remainder of this contribution is divided into two parts;

- Proposed method, and

- Experimental results under the CTC.

Proposed Method

ISP in VVC Draft 4, predictions and transforms are processed by sub-partition. In prediction for a sub-partition, reconstructed sample values of previous sub-partition are used as shown in Figure 1 (a). Therefore, prediction and transform should be processed sequentially. Additionally, sub-partition width or height may be less than 4 pixels. These can be caused a hardware implementation issue especially for small blocks.

The proposed method switches processing unit of prediction between sub-partition (same as TU) and CU for CUs applied ISP. When predictions are processed by CU in the proposed method, an example of arrangements of reference samples is shown in Figure 1 (b). As shown in Figure 1 (b), “prediction by CU” is same as intra prediction when ISP is not applied.

(a) In case of prediction processed by sub-partition

(b) In case of prediction processed by CU

Figure 1 An example of arrangements of reference samples.

The switching conditions of processing unit for prediction are as follows.

Conditions for Test 1 (considering both of sub-partition width and height)

If a sub-partition width or height is less than 4 pixels, prediction is processed by CU. Otherwise (if both of sub-partition width and height are greater than or equal to 4 pixels), prediction is processed by sub-partition.

Conditions for Test 2 (considering only sub-partition width)

If a sub-partition width is less than 4 pixels, prediction is processed by CU. Otherwise (if the sub-partition width is greater than or equal to 4 pixels), prediction is processed by sub-partition.

Under the conditions of Test 1, all of processing units of intra prediction have at least 4 pixels for width and height in spite of whether ISP is applied or not. In other words, the smallest processing unit size of prediction is not changed by using ISP.

Additionally, transformations are processed by sub-partition as same as ISP in VTM-4.0. Therefore, when prediction is processed by CU, transformation for each sub-partition in a same CU are able to be processed in parallel.

Experimental Results

The proposed method is implemented on top of VTM-4.0. In order to verify the performance of the proposed method, we have run simulations according to the common test conditions of JVET-M1010 [2].

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Test results

Table 1 and Table 2 show the experimental results of Test 1. BD-rate for luma is 0.10%/0.04% (AI/RA) in comparison with VTM-4.0. BD-rate loss for AI is smaller than the result in Table 5 and Table 6. Table 5 shows the results in case of simply disabling ISP with sub-partition width or height less than 4 pixels. Similarly, Table 6 shows the results in case of disabling ISP with sub-partition width less than 4 pixels same as Test 1.1.2 in [3].

Table 1 Experimental results of Test 1 for AI condition.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.04% | 0.01% | 100% | 100% |
| Class A2 | 0.05% | 0.12% | 0.06% | 100% | 101% |
| Class B | 0.07% | 0.12% | 0.12% | 100% | 99% |
| Class C | 0.15% | 0.17% | 0.18% | 100% | 99% |
| Class E | 0.18% | 0.19% | 0.04% | 101% | 100% |
| Overall | 0.10% | 0.13% | 0.09% | 100% | 100% |
| Class D | 0.12% | 0.23% | 0.08% | 100% | 99% |
| Class F | 0.17% | 0.11% | 0.14% | 100% | 99% |

Table 2 Experimental results of Test 1 for RA condition.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.02% | -0.02% | 100% | 100% |
| Class A2 | 0.03% | 0.05% | 0.05% | 100% | 100% |
| Class B | 0.04% | 0.08% | -0.21% | 100% | 100% |
| Class C | 0.11% | 0.12% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.06% | -0.07% | 100% | 100% |
| Class D | 0.05% | -0.12% | -0.03% | 100% | 101% |
| Class F | 0.06% | -0.04% | -0.04% | 100% | 101% |

Table 3 and Table 4 show the experimental results of Test 2. BD-rate for luma is 0.06%/0.02% (AI/RA) in comparison with VTM-4.0. BD-rate loss for AI and RA are smaller than Test 1. BD-rate loss for AI is smaller than the result in Table 5 and Table 6.

Table 3 Experimental results of Test 2 for AI condition.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.00% | 100% | 100% |
| Class A2 | 0.02% | 0.09% | 0.00% | 100% | 100% |
| Class B | 0.03% | 0.05% | 0.06% | 100% | 99% |
| Class C | 0.11% | 0.07% | 0.01% | 100% | 99% |
| Class E | 0.14% | 0.15% | 0.16% | 100% | 100% |
| Overall | 0.06% | 0.07% | 0.05% | 100% | 100% |
| Class D | 0.08% | 0.06% | -0.18% | 100% | 99% |
| Class F | 0.09% | 0.08% | 0.10% | 100% | 100% |

Table 4 Experimental results of Test 2 for RA condition.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.14% | -0.03% | 100% | 100% |
| Class A2 | 0.01% | 0.21% | 0.00% | 100% | 100% |
| Class B | 0.00% | -0.05% | -0.08% | 100% | 99% |
| Class C | 0.05% | -0.07% | -0.18% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.02% | -0.08% | 100% | 99% |
| Class D | 0.04% | 0.01% | -0.19% | 100% | 100% |
| Class F | -0.01% | -0.12% | -0.16% | 100% | 100% |

Reference data

Table 5 shows the experimental results in case of disabling ISP with sub-partition width or height less than 4 pixels. Likewise, Table 6 shows the experimental results in case of disabling ISP with sub-partition width less than 4 pixels. In other words, Table 6 shows the results in case of Test 1.1.2 in [3] implemented on top of VTM-4.0.

Table 5 Experimental results of disabling ISP with sub-partition width or height less than 4 pixels for AI condition.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | 0.00% | 99% | 99% |
| Class A2 | 0.05% | 0.13% | 0.01% | 99% | 100% |
| Class B | 0.20% | 0.24% | 0.26% | 99% | 98% |
| Class C | 0.44% | 0.39% | 0.29% | 98% | 97% |
| Class E | 0.52% | 0.33% | 0.30% | 98% | 99% |
| Overall | 0.25% | 0.23% | 0.19% | 99% | 99% |
| Class D | 0.46% | 0.35% | 0.34% | 98% | 97% |
| Class F | 0.64% | 0.33% | 0.35% | 99% | 98% |

Table 6 Experimental results of disabling ISP with sub-partition width less than 4 pixels for AI condition.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | -0.01% | 100% | 99% |
| Class A2 | 0.00% | 0.12% | -0.02% | 100% | 101% |
| Class B | 0.05% | 0.07% | 0.13% | 99% | 99% |
| Class C | 0.21% | 0.06% | 0.14% | 99% | 99% |
| Class E | 0.43% | 0.38% | 0.20% | 99% | 99% |
| Overall | 0.13% | 0.13% | 0.10% | 99% | 99% |
| Class D | 0.24% | 0.22% | 0.20% | 99% | 99% |
| Class F | 0.32% | 0.19% | 0.09% | 99% | 99% |

Conclusion

In this contribution, it was proposed that a relaxation method of processing dependency for Intra Sub-Partitions (ISP) with small brock. The proposed method changes unit of prediction from sub-partitions to CU for small block. According to the proposed method, all of processing units of intra prediction had at least 4 pixels for width and height in spite of whether ISP was applied or not. Additionally, transformation could be processed in parallel for a small block in the proposed method. Experimental results showed that BD-rate for luma is 0.10%/0.04% (AI/RA) in comparison with VTM-4.0.

- Acknowledgment

We would like to thank XXXX for doing crosschecking of our proposal.

References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Marrakech, MA, Jan. 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Marrakech, MA, Jan. 2019.

- S. De-Luxán-Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe and T. Wiegand, “CE3: Intra Sub-Partitions Coding Mode (Tests 1.1.1 and 1.1.2)”, JVET-M0102, Marrakech, MA, Jan. 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).