| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0160_v1 |
| --- | --- |

| Title: | Non-CE6: Simplification on transform selection for Intra Sub-Partitions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoqiang Cao Liying Xu Fangdong Chen Li Wang | Tel: Email: | caoxiaoqiang@hikvision.com chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This contribution present three simplified methods on transform selection for ISP mode. The first simplified version, it is reported that the BD-rate are 0.00%, -0.01%, -0.01% for AI, RA, and LB respectively. The second simplified version, it is reported that the BD-rate are -0.02%, 0.01%, 0.00% for AI, RA, and LB respectively. The third simplified version, it is reported that the BD-rate are 0.05%, 0.03%, 0.0x% for AI, RA, and LB respectively.

Introduction

In current Intra sub-partition that adopt in VTM4.0[1], the transform choice for ISP mode is implicit. For example, if a CU uses the ISP coding mode, the MTS CU flag will be set to 0 and it will not be sent to the decoder. Therefore, the encoder will not perform RD tests for the different available transforms for each resulting sub-partition. The transform choice for the ISP mode will instead be fixed and selected according the intra mode, the processing order and the block size utilized. Hence, no signalling is required. For example, let tH and tV be the horizontal and the vertical transforms selected respectively for the w×h sub-partition, where w is the width and h is the height. Then the transform is selected as following rules:

- If w=1 or h=1, then there is no horizontal or vertical transform respectively.

- If w=2 or w>16, tH = DCT2

- If h =2 or h >16, tV = DCT2

- Otherwise, the transform is selected as in Table 1.

Table 1 – Transform selection depends on intra mode

| Intra mode | tH | tV |
| --- | --- | --- |
| Planar Ang. 31,32,34,36,37 | DST7 | DST7 |
| DC Ang. 33, 35 | DCT2 | DCT2 |
| Ang. 2, 4, 6…28,30 Ang. 39,41,43…63,65 | DST7 | DCT2 |
| Ang. 3,5,7…27,29 Ang. 38,40,42…64,66 | DCT2 | DST7 |

Proposed Method

In this contribution, the proposed method introduced three simplified versions to derive the transform pair. For the first version of the proposed method, compared with other angular modes based on a parity switching mechanism, it is seeming that no special classification for mode 31,32,33,34,35,36 and 37 are needed, so a unified parity switching mechanism is conducted for all angular mode.

For the second version of the proposed method, compared to the first version, all odd angular mode can use the same transform pair (DST7, DST7) and there is no need to classify the odd angular mode.

For the third version of the proposed method, it just based on block width and block height to derive the transform pair, rather than based on intra mode and block size.

Simplified version 1

- If w=1 or h=1, then there is no horizontal or vertical transform respectively.

- If w=2 or w>16, tH = DCT2

- If h =2 or h >16, tV = DCT2

- Otherwise, the transform is selected as in Table 2.

Table 2 – Transform selection depends on intra mode

| Intra mode | tH | tV |
| --- | --- | --- |
| Planar | DST7 | DST7 |
| DC | DCT2 | DCT2 |
| Ang. [2-34] && even Ang. [35-66] && odd | DST7 | DCT2 |
| Ang. [2-34] && odd Ang. [35-66]&& even | DCT2 | DST7 |

Simplified version 2

- If w=1 or h=1, then there is no horizontal or vertical transform respectively.

- If w=2 or w>16, tH = DCT2

- If h =2 or h >16, tV = DCT2

- Otherwise, the transform is selected as in Table 3.

Table 3 – Transform selection depends on intra mode

| Intra mode | tH | tV |
| --- | --- | --- |
| Planar Ang. [2-66] && odd | DST7 | DST7 |
| DC | DCT2 | DCT2 |
| Ang. [2-34] && even | DST7 | DCT2 |
| Ang. [35-66] && even | DCT2 | DST7 |

Simplified version 3

- If w=1 or h=1, then there is no horizontal or vertical transform respectively.

- If w=2 or w>16, tH = DCT2

- If h =2 or h >16, tV = DCT2

- Otherwise, the transform is selected as in Table 4.

Table 4 – Transform selection depends on ISP sub-block shape

| Sub-block shape | tH | tV |
| --- | --- | --- |
| W=H | DST7 | DST7 |
| W>H | DCT2 | DST7 |
| W<H | DST7 | DCT2 |

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-4.0 is served as the experimental platform, the CTC specified in [2], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

Results for simplified version 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | -0.03% | 100% | 99% |
| Class A2 | 0.00% | 0.08% | -0.03% | 101% | 99% |
| Class B | 0.00% | 0.02% | 0.12% | 100% | 98% |
| Class C | 0.00% | -0.02% | 0.00% | 101% | 101% |
| Class E | -0.01% | 0.02% | -0.04% | 101% | 99% |
| Overall | 0.00% | 0.02% | 0.02% | 101% | 99% |
| Class D | -0.01% | -0.02% | 0.07% | 100% | 99% |
| Class F (optional) | 0.00% | 0.03% | -0.06% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.07% | -0.01% | 100% | 100% |
| Class A2 | -0.03% | 0.00% | 0.01% | 100% | 98% |
| Class B | -0.01% | 0.06% | -0.07% | 100% | 100% |
| Class C | 0.02% | -0.17% | -0.18% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.04% | -0.07% | 100% | 100% |
| Class D | 0.03% | -0.30% | -0.20% | 100% | 101% |
| Class F (optional) | -0.01% | -0.10% | 0.00% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.16% | -0.03% | 100% | 99% |
| Class C | 0.00% | 0.05% | 0.00% | 100% | 100% |
| Class E | -0.04% | -0.07% | 0.81% | 100% | 101% |
| Overall | -0.01% | -0.06% | 0.19% | 100% | 100% |
| Class D | 0.02% | 0.99% | 0.02% | 100% | 100% |
| Class F (optional) | -0.10% | -0.44% | -0.15% | 101% | 99% |

Results for simplified version 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.04% | -0.05% | 100% | 99% |
| Class A2 | -0.02% | 0.05% | -0.06% | 101% | 100% |
| Class B | -0.01% | -0.02% | 0.01% | 100% | 99% |
| Class C | 0.00% | -0.06% | -0.14% | 101% | 100% |
| Class E | -0.05% | -0.10% | -0.17% | 101% | 101% |
| Overall | -0.02% | -0.02% | -0.07% | 101% | 100% |
| Class D | 0.02% | -0.01% | -0.17% | 101% | 102% |
| Class F (optional) | 0.01% | 0.00% | -0.20% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.23% | -0.05% | 100% | 101% |
| Class A2 | -0.01% | -0.07% | 0.05% | 100% | 99% |
| Class B | 0.01% | -0.07% | -0.22% | 100% | 100% |
| Class C | 0.03% | -0.08% | -0.13% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.10% | -0.11% | 100% | 100% |
| Class D | -0.01% | -0.31% | -0.43% | 100% | 101% |
| Class F (optional) | 0.00% | -0.09% | 0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.02% | 0.05% | 100% | 100% |
| Class C | -0.03% | 0.31% | -0.10% | 100% | 100% |
| Class E | 0.04% | 0.66% | 0.95% | 100% | 102% |
| Overall | 0.00% | 0.27% | 0.22% | 100% | 101% |
| Class D | -0.02% | -0.06% | 0.26% | 100% | 99% |
| Class F (optional) | -0.11% | -0.23% | -0.56% | 101% | 100% |

Result for simplified version 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | -0.04% | 100% | 100% |
| Class A2 | 0.02% | 0.13% | 0.00% | 100% | 100% |
| Class B | 0.03% | 0.09% | 0.14% | 100% | 99% |
| Class C | 0.07% | 0.03% | 0.08% | 100% | 101% |
| Class E | 0.13% | 0.16% | 0.13% | 101% | 102% |
| Overall | 0.05% | 0.09% | 0.07% | 100% | 100% |
| Class D | 0.05% | 0.01% | -0.04% | 101% | 100% |
| Class F (optional) | 0.00% | 0.13% | 0.09% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | -0.02% | 100% | 100% |
| Class A2 | 0.00% | 0.06% | -0.07% | 100% | 98% |
| Class B | 0.03% | 0.10% | -0.03% | 100% | 100% |
| Class C | 0.07% | 0.08% | 0.05% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.07% | -0.02% | 100% | 100% |
| Class D | 0.03% | -0.09% | -0.05% | 101% | 102% |
| Class F (optional) | 0.01% | 0.00% | -0.09% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | 101% |
| Class C | -0.02% | 0.34% | -0.12% | 100% | 99% |
| Class E | 0.08% | 0.44% | 0.19% | 101% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | 100% |
| Class D | -0.07% | -0.37% | -0.57% | 100% | 100% |
| Class F (optional) | -0.08% | -0.07% | -0.37% | 100% | 99% |

Conclusion

In this contribution, we have presented three simplifications for the transform selection of ISP mode. All the three simplifications make the derive of transform pair more simple and straight forward, the first and second version of simplification are based on intra mode and block size, experimental results show that they all have no coding loss, the third version of simplification are only based on block size and shape, no intra mode are needed to derive the transform pair, experimental results show that only minor coding loss are occurred. Based on the above results, it is suggested to adopt them into the VTM.

Reference

[1]	S. De-Luxán-Hernández, V. George, J. Ma, et al. “CE3: Intra Sub-Partitions Coding Mode,”JVET-M0102, Marrakech, MA, 9–18 Jan. 2019.

[2]	F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).