| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0162-v2 |
| --- | --- |

| Title: | Non-CE9: Modified enabling condition for DMVR and BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

In this contribution, two modifications on enabling conditions are proposed for decoder motion vector refinement (DMVR) and bidirectional optical flow (BDOF). First, if weighted prediction (WP) or bi-prediction with CU weights (GBI) applys, both of them shoud be disabled. Second, their picture order count (POC) related conditions are unified. It is reported that the first modification has no coding loss for RA configuration compared to VTM-4.0. And the second modification has negligible coding loss for RA configuration compared to VTM-4.0. These two modifications both help to save complexity.

Introduction

Only the following conditions are satisfied, DVMR is enabled:

- sps_dmvr_enabled_flag is equal to 1

- merge_flag[ xCb ][ yCb ] is equal to 1

- both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1

- mmvd_flag[ xCb ][ yCb ] is equal to 0

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 64

Moreover, BDOF is enabled with the following conditions satisfied:

- sps_bdof_enabled_flag is equal to 1.

- predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

- MotionModelIdc[ xCb ][ yCb ] is equal to 0.

- merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

- GbiIdx[ xCb ][ yCb ] is equal to 0.

- cIdx is equal to 0.

Proposed Modification

Modification 1: Only enable DMVR and BDOF for equal weight

Since both DMVR and BDOF have the assumption of constant brightness, if WP or GBI method applys, both of them shoud be disabled. Thus, their enabling conditions should be added with the follwing restricted conditions:

- GbiIdx[ xCb ][ yCb ] is equal to 0.

- luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0

Hence, the text is modified as follows.

- When all of the following conditions are true, dmvrFlag is set equal to 1:

- sps_dmvr_enabled_flag is equal to 1

- merge_flag[ xCb ][ yCb ] is equal to 1

- both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1

- mmvd_flag[ xCb ][ yCb ] is equal to 0

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

- GbiIdx[ xCb ][ yCb ] is equal to 0.

- luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 64

- If all of the following conditions are true, bdofFlag is set equal to TRUE.

- sps_bdof_enabled_flag is equal to 1.

- predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

- MotionModelIdc[ xCb ][ yCb ] is equal to 0.

- merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

- GbiIdx[ xCb ][ yCb ] is equal to 0.

- luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0

- cIdx is equal to 0.

Modification 2: Unified POC condition for DMVR and BDOF

Here, the POC conditions for DMVR and BDOF are unified with the following condition to avoid unnecessary refinement for BDOF, which may help to save some complexity.

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-4.0 is served as the experimental platform. The RA configuration specified in [1], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

Here, we have three tests:

Test A: Only apply Modification 1.

Test B: Only apply Modification 2.

Test C: Apply Modification 1 and Modification 2.

The results are given as Table 1-3. It is reported that the Modification 1 has 0.00% coding loss for RA configuration, and Modification 2 has 0.01% coding loss for RA configuration. When these two modifications are combined, the coding loss is still negligible, but they both help to decrease complexity.

Table 1 Results of Test A

Table 2 Results of Test B

Table 2 Results of Test C

Conclusion

In this contribution, we have two modifications on DMVR and BDOF enabling condition. These two modifications bring no coding loss and help to save complexity. Based on the above results, it is suggested to adopt them into the VTM.

Reference

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).