| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0163-v1 |
| --- | --- |

| Title: | Non-CE9: Simplified refinement process for DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

In this contribution, the refinement process for decoder motion vector refinement (DMVR) is modified. Several one stage search methods are proposed. First, a simplified 25 points method with removing the early termination is proposed. And then a 21 points method and a 13 points method are also proposed. It is reported that the first simplified 25 points method has 0.03% gains for RA compared to VTM-4.0. And the 21 points method and the 13 points method have 0.03% gains and 0.04% loss, respectively, for RA compared to VTM-4.0.

Introduction

In [1], the latest DMVR is proposed, and it is adopted in VTM 4.0. In VTM 4.0, a two stage method is used and Ticket #214 [2] modifies it with a one stage method to increase the pipeline ability. However, the merged Ticket #214 is actually a 26 points method, since the original motion vector (MV) cost is calculated two times. The first calculation is used for early termination and seems redundant. Moreover, we also propose other one stage methods with less points to decrease the complexity.

Proposed Modification

Simplified 25 points searching

The first cost calculation of original MV and the early termination based on the cost are both removed. Thus only 25 points cost are necessary. The 25 points are the same as that given in Ticket #214.

21 points searching

In the simplified 25 points method, the number of search points is reduced to 21 with removing the four points: {MV(-2,-2), MV(-2,2), MV(2,-2), MV(2,-2)}. Here, the error surface is still applicable for centre 9 points, which is the same as the 25 points method.

13 points searching

In the simplified 25 points method, the number of search points is reduced to 13. The 13 points are as follows:

{

Mv(0, 0), Mv(-1, 0), Mv(0,-1), Mv(1, 0), Mv(0, 1),

Mv(-1, 1), Mv(-1,-1), Mv(1,-1), Mv(1, 1),

Mv(0, 2), Mv(-2, 0), Mv(0,-2), Mv(2, 0),

};

Here, the error surface is applicable for centre 5 points (Mv(0, 0), Mv(-1, 0), Mv(0,-1), Mv(1, 0), Mv(0, 1)).

9 points searching

In the simplified 25 points method, the number of search points is reduced to 9. The 9 points are as follows:

{

Mv(-1,-1), Mv(0,-1) , Mv(1,-1),

Mv(-1,0), Mv(0,0) , Mv(1,0),

Mv(-1,1), Mv(0,1) , Mv(1,1),

};

Here, the error surface is only applicable for centre 1 points (Mv(0, 0)).

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-4.0 is served as the experimental platform. The random access configuration (RA) specified in [2], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

We have five tests in the experiments:

Test A: The method in Ticket #214.

Test B: The simplified 25 points searching method.

Test C: The 21 points searching method.

Test D: The 13 points searching method.

Test E: The 9 points searching method.

The results are given in Table 1 to Table 5. It is reported that the simplified 25 points searching method nearly has the same coding gains as the method in Ticket #214. And the 21 points searching method still achieves 0.03% coding gains. For 13 points searching method, 0.04% coding loss is found, but it helps to reduce about half complexity. Compared to the 9 points searching method, which has 0.18% coding loss, the 13 points searching method would be a good trade-off.

Table 1 Results of Test A (26 points)

Table 2 Results of Test B (25 points)

Table 3 Results of Test C (21 points)

Table 4 Results of Test D (13 points)

Table 5 Results of Test E (9 points)

Conclusion

In this contribution, we propose several one stage refinement method for DVMR. The 13 points method is reported to be a good trade-off. Based on the above results, it is suggested to adopt it into the VTM.

Reference

- S. Sethuraman, “CE9: Results of DMVR related Tests CE9.2.1 and CE9.2.2,” JVET-M0147, Marrakech, MA, 9–18 Jan. 2019.

- Ticket #214: M0147 - Mismatch of an adopted element(25 points) in VTM

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).