| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0163-v3 |
| --- | --- |

| Title: | Non-CE9: Simplified refinement process for DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

In this contribution, the refinement process for decoder motion vector refinement (DMVR) is modified. Several one stage search methods are proposed. First, a simplified 25 points method with removing the unnecessary comparison is proposed. And then a 21 points method and a 13 points method are also proposed. It is reported that the first simplified 25 points method has 0.04% gains for RA compared to VTM-4.0. And the 21 points method and the 13 points method have 0.04% gains and 0.05% loss, respectively, for RA compared to VTM-4.0.

Introduction

In [1], the latest DMVR is proposed, and it is adopted in VTM 4.0. In VTM 4.0, a multi-stage method is used and Ticket #214 [2] modifies it with a one stage method to increase the pipeline ability. However, the merged Ticket #214 is actually a 26 points method, since the original motion vector (MV) cost is used two times. The first calculation is used for early termination and the second one is used for comparison again and seems redundant. Moreover, we also propose other one stage methods with less points to decrease the complexity.

Proposed Modification

Simplified 25 points searching

The second cost comparison of original MV in Ticket #214 is removed (it is likely a software cleanup). Besides the zero MV offset (used for early termination), the remaining 24 offset points are as follows:

{

Mv(-2,-2), Mv(-1,-2), Mv(0,-2), Mv(1,-2), Mv(2,-2),

Mv(-2,-1), Mv(-1,-1), Mv(0,-1), Mv(1,-1), Mv(2,-1),

Mv(-2, 0), Mv(-1, 0), Mv(1, 0), Mv(2, 0),

Mv(-2, 1), Mv(-1, 1), Mv(0, 1), Mv(1, 1), Mv(2, 1),

Mv(-2, 2), Mv(-1, 2), Mv(0, 2), Mv(1, 2), Mv(2, 2)

}

21 points searching

In the simplified 25 points method, the number of search points is reduced to 21 with removing the four offset points: {MV(-2,-2), MV(-2,2), MV(2,-2), MV(2,2)}. Here, the error surface is still applicable for centre 9 points, which is the same as the 25 points method.

13 points searching

In the simplified 25 points method, the number of search points is reduced to 13. Besides the zero MV offset, the remaining 12 points are as follows:

{

Mv(-1, 0), Mv(0,-1), Mv(1, 0), Mv(0, 1),

Mv(-1, 1), Mv(-1,-1), Mv(1,-1), Mv(1, 1),

Mv(0, 2), Mv(-2, 0), Mv(0,-2), Mv(2, 0),

};

Here, the error surface is applicable for centre 5 points (Mv(0, 0), Mv(-1, 0), Mv(0,-1), Mv(1, 0), Mv(0, 1)).

9 points searching

In the simplified 25 points method, the number of search points is reduced to 9. Besides the zero MV offset, the remaining 8 points are as follows:

{

Mv(-1,-1), Mv(0,-1) , Mv(1,-1),

Mv(-1,0), Mv(0,0) , Mv(1,0),

Mv(-1,1), Mv(0,1) , Mv(1,1),

};

Here, the error surface is only applicable for centre 1 points (Mv(0, 0)).

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-4.0 is served as the experimental platform. The random access configuration (RA) specified in [2], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

We have five tests in the experiments:

Test A: The method in Ticket #214.

Test B: The simplified 25 points searching method.

Test C: The 21 points searching method.

Test D: The 13 points searching method.

Test E: The 9 points searching method.

The results are given in Table 1 to Table 5. It is reported that the simplified 25 points searching method nearly has the same coding gains as the method in Ticket #214. And the 21 points searching method still achieves 0.04% coding gains. For 13 points searching method, 0.05% coding loss is found, but it helps to reduce about half complexity. Compared to the 9 points searching method, which has 0.18% coding loss, the 13 points searching method would be a good trade-off.

Table 1 Results of Test A (Ticket #214)

Table 2 Results of Test B (25 points)

Table 3 Results of Test C (21 points)

Table 4 Results of Test D (13 points)

Table 5 Results of Test E (9 points)

Conclusion

In this contribution, we propose several one stage refinement methods for DVMR. The 13 points method is reported to be a good trade-off. Based on the above results, it is suggested to adopt it into the VTM.

Reference

- S. Sethuraman, “CE9: Results of DMVR related Tests CE9.2.1 and CE9.2.2,” JVET-M0147, Marrakech, MA, 9–18 Jan. 2019.

- Ticket #214: M0147 - Mismatch of an adopted element(25 points) in VTM

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).