| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0166 |
| --- | --- |

| Title: | Non-CE4: Advanced Multi-hypothesis Inter Prediction for bandwidth reduction in B frame |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yucheng Sun Fangdong Chen Li Wang | Tel: Email: | +86 18105755698 sunyucheng@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This contribution presents advance multi-hypothesis inter prediction (AMHP) proposed in JVET-M0167 for bandwidth reduction in B frame, AMHP is a multi-hypothesis prediction for uni-prediction of AMVP mode. By limiting the motion information of AMHP block, AMHP can be used in small block size in B frame to reduce the bandwidth cost. Experimental results reportedly show 0.06%, 0.12%, 0.16% BD-rate under RA condition, 0.06%, 0.13%, -0.02% BD-rate with 101% and 103% encoding and decoding time under LDB condition.

Proposed Method

In this contribution, advance multi-hypothesis inter prediction (AMHP) proposed in JVET-M0167 is presented to reduce the bandwidth cost in B frame. AMHP is a multi-hypothesis prediction for uni-prediction of AMVP mode with low bandwidth cost.

When applied to improve uni-prediction of AMVP mode, one amhp_flag is signaled to enable or disable AMHP for inter_dir equal to 1 or 2, where 1, 2, and 3 represent list 0, list 1, and bi-prediction, respectively. Moreover, amhp_index is signaled when the amhp_flag is true. amhp_index can indicate two modes in AMHP.

In this way, AMHP turns uni-prediction into restricted bi-prediction, where one motion is acquired using the original syntax elements in AMVP mode while the other is acquired using the AMHP MV derivation scheme in section 1.1.

AMHP MV derivation scheme

AMHP utilize the uni-prediction MV to derive two MV with the same reference frame. Assume the uni-prediction MV is (a, b).

AMHP has two modes is as follows:

Table 1. AMHP modes

| amhp_idx | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| AMHP modes | AMHP_NON_SYM mode |  |  |  | AMHP_SYM mode |  |

In AMHP_NON_SYM mode,

( a, b )  ( a + s × k × Step, b + s × l × Step )

will be assigned to AMHP block;

In AMHP_SYM mode,

( a + k × Step, b + l × Step )  ( a – k × Step, b – l × Step )

will be assigned to AMHP block;

Pre-defined k, l value is as follows:

Table 2. k, l value

| condition | a = 0 | a > 0 | a < 0 |  | condition | b = 0 | b > 0 | b < 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| k | 0 | 1 | -1 |  | l | 0 | 1 | -1 |

Pre-defined s value is as follows:

Table 3. s value

| amhp_idx | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| s value | -1 | 1 | -1 | 1 | - | - |

Pre-defined Step value is as follows:

Table 4. Step value

| amhp_idx | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Step value | 1/4-pel | 1/16-pel | 1/4-pel | 1/16-pel | 1/4-pel | 1/2-pel |

Combination weights

The final prediction uses 1:1 weights to combine these two predictions in AMHP_SYM mode and 5:3 weights to combine these two predictions in AMHP_NON_SYM mode.

Bandwidth calculation

In AMHP mode, the restricted bi-prediction will cost less bandwidth compared to traditional bi-prediction.

| block size |  | bandwidth |  |  |
| --- | --- | --- | --- | --- |
| width | height | uni-prediction | bi-prediction | AMHP |
| 4 | 4 | 7.56 | - | 9 |
| 4 | 8 | 5.16 | - | 6 |
| 8 | 4 | 5.16 | - | 6 |
| 8 | 8 | 3.52 | 7.03 | - |
| 4 | 16 | 3.95 | 7.91 | - |
| 16 | 4 | 3.95 | 7.91 | - |

For uni-prediction:

( width + 7 ) × ( height + 7 )/( width × height )

For bi-prediction:

2 × ( width + 7 ) × ( height + 7 )/( width × height )

For AMHP, the MV difference between the two AMHP mv won’t be larger than 1 pixel:

( width + 7 + 1 ) × ( height + 7 +1 )/( width × height )

In B frame, we disable bi-predction in block size equal to 4×8 or 8×4 and enable AMHP in 4×4, 4×8 and 8×4.

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-4.0 is served as the experimental platform. The CTC specified in [1], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

Table 5 and Table 6 show the test results for AI and RA configuration.

Table 5 Experimental results of Test1 for random-access (RA) test condition; anchor is VTM4.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.10% | 0.09% | 100% | 101% |
| Class A2 | 0.06% | 0.19% | 0.25% | 101% | 99% |
| Class B | 0.04% | 0.09% | 0.17% | 101% | 101% |
| Class C | 0.13% | 0.11% | 0.14% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.12% | 0.16% | 101% | 101% |
| Class D | 0.18% | 0.27% | 0.18% | 101% | 105% |
| Class F | 0.05% | 0.13% | 0.07% | 101% | 103% |

Table 6 Experimental results of Test1 for Low-Delay B (LDB) test condition; anchor is VTM4.0

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.19% | -0.06% | 100% | 102% |
| Class C | 0.09% | 0.57% | 0.23% | 101% | 103% |
| Class E | 0.07% | 0.07% | -0.28% | 101% | 104% |
| Overall | 0.06% | 0.13% | -0.02% | 101% | 103% |
| Class D | 0.22% | 0.53% | 0.18% | 100% | 102% |
| Class F | 0.05% | 0.04% | -0.07% | 101% | 102% |

Reference

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).