| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0168 |
| --- | --- |

| Title: | Non-CE4: AMVR cost calculation modification in motion estimation stage |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yucheng Sun Fangdong Chen Li Wang | Tel: Email: | +86 18105755698 sunyucheng@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

In VTM4.0, AMVR cost calculation double calculate MVD bits and omit mvp_flag cost, which influence the MV decision in motion estimation stage. Experimental results reportedly show 0.xx%, 0.xx%, 0.xx% BD-rate increases, 0.xx%, 0.xx%, 0.xx% BD-rate increases and 0.xx%, 0.xx%, 0.xx% BD-rate increases for Y, Cb, and Cr components respectively, compared to VTM-RA, VTM-LDB and VTM-LDP configurations.

Proposed Method

Proposed modification contains two part:

Part 1: add mvp_flag cost

| original code | int iMvBits = m_auiMVPIdxCost[iMVPIdx][AMVP_MAX_NUM_CANDS]; ... iMvBits += m_pcRdCost->getBitsOfVectorWithPredictor( cTestMv[iMVPIdx].getHor(), cTestMv[iMVPIdx].getVer(), cStruct.imvShift ); uiDist += m_pcRdCost->getCostOfVectorWithPredictor( cTestMv[iMVPIdx].getHor(), cTestMv[iMVPIdx].getVer(), cStruct.imvShift ); |
| --- | --- |
| modification | int iMvBits = m_auiMVPIdxCost[iMVPIdx][AMVP_MAX_NUM_CANDS]; ... iMvBits += m_pcRdCost->getBitsOfVectorWithPredictor( cTestMv[iMVPIdx].getHor(), cTestMv[iMVPIdx].getVer(), cStruct.imvShift ); uiDist += m_pcRdCost->getCostOfVectorWithPredictor( cTestMv[iMVPIdx].getHor(), cTestMv[iMVPIdx].getVer(), cStruct.imvShift ); uiDist += m_pcRdCost->getCost(iMvBits); |

Part 2: delete extra MVD bits calculation

| original code | ruiCost = uiBestDist - m_pcRdCost->getCost(iBestBits) + m_pcRdCost->getCost(ruiBits); ruiBits += m_pcRdCost->getBitsOfVectorWithPredictor(rcMv.getHor(), rcMv.getVer(), cStruct.imvShift); |
| --- | --- |
| modification | ruiCost = uiBestDist - m_pcRdCost->getCost(iBestBits) + m_pcRdCost->getCost(ruiBits); ruiBits += m_pcRdCost->getBitsOfVectorWithPredictor(rcMv.getHor(), rcMv.getVer(), cStruct.imvShift); |

Experimental Results

To evaluate the coding performance of the proposed modfication, the VTM-4.0 is served as the experimental platform. The CTC specified in [1], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

Table 1, Table 2 and Table 3 show the test results for AI and RA configuration.

Table 1 Experimental results of Test1 for random-access (RA) test condition; anchor is VTM4.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | -0.13% | -0.09% | 100% | 100% |
| Class C | -0.03% | -0.08% | -0.16% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | -0.01% | -0.10% | 101% | 104% |
| Class F | -0.01% | -0.03% | -0.03% | 100% | 103% |

Table 2 Experimental results of Test1 for Low-Delay B (LDB) test condition; anchor is VTM4.0

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.03% | 0.06% | -0.21% | 100% | 101% |
| Class E | -0.01% | 0.01% | -0.01% | 100% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.09% | -0.28% | 100% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 3 Experimental results of Test1 for Low-Delay P (LDP) test condition; anchor is VTM4.0

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.01% | -0.14% | -0.37% | 100% | 100% |
| Class E | 0.01% | 0.06% | 0.18% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | -0.13% | 0.00% | 100% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Reference

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).