| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0170_r1 |
| --- | --- |

| Title: | CE3-related: Explicitly signal non angular modes with encoding changes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Yao, J.-Q. Zhu, W.-T. Cai, K. Kazui 601,Yongda International Tower, 2277 Longyang Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5809 yaojie@cn.fujitsu.com zhujianqing@cn.fujitsu.com caiwenting@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Source: | Fujitsu |  |  |

_____________________________

- Abstract

This contribution presents to change the method of adding additional modes into the R-D check list to find a better trade-off between the gain and the encoding run-time with respect to the results of explicitly signal non angular modes(CE3-3.5.1). Instead of adding left intra mode(L) and above intra mode(A) into the R-D check list (VTM-4.0) we add modes with lowest bit cost for mode signaling depend on L and A. Such proposed encoding changes is implemented upon CE3-3.5.1. The results reportedly show an average luma BD rate performance of (−0.18%, −0.16%, −0.12%) for (Y, Cb, Cr) AI, (−0.06%, −0.00%, −0.03%) for (Y, Cb, Cr) RA without increase of encoding running time over VTM-4.0.

Introduction

Unifying MPM derivation process by explicitly signal non angular modes (Planar, DC) was first proposed in JVET-M0210 [1] and was scheduled for doing CE tests (CE3-3.5) [2] on VTM-4.0. It propose to signal a syntax (intra_luma_non_ang_flag) at first for intra block to indicate whether IntraMode is angular or non angular. Figure 1 shows the coding process of the method. For non_angular mode, a syntax (intra_luma_planar_flag) is signaled to indicate whether IntraMode is Planar or DC. For angular modes, MPM flag (intra_luma_mpm_flag), reference line index (intra_luma_ref_idx), and ISP mode (intra_subpartitions_mode_flag) are sequentially coded. MPM list is used and is derived only using angular modes which is the same as the one currently being used for non-zero reference line index in VTM. Figure 2 shows details of the MPM list generation process. No dependency with ISP mode and reference line index for both MPM derivation process and MPM index coding. The detailed syntax and semantics modifications to VVC WD [3] can be found in attached document.

VTM-4.0

parse intra_luma_ref_idx

if (intra_luma_ref_idx == 0)

parse intra_subpartitions_mode_flag

if (intra_subpartitions_mode_flag == 0)

parse intra_luma_mpm_flag

MPM with Planar&DC

else

MPM without DC

else

MPM without Planar&DC

Proposed

parse intra_luma_non_ang_flag

if(intra_luma_non_ang_flag == 1)

// no MPM

parse intra_luma_planar_flag

if (intra_luma_planar_flag)

parse intra_subpartitions_mode_flag

else

parse intra_luma_mpm_flag

if (intra_luma_mpm_flag)

parse intra_luma_ref_idx

if (intra_luma_ref_idx == 0)

parse intra_subpartitions_mode_flag

MPM without Planar&DC

Figure 1. coding process of explicitly signal non angular modes

maxAB = max (leftIntraDir, aboveIntraDir)

minAB = min (leftIntraDir, aboveIntraDir)

if maxAB != minAB, minAB > DC

candModeList[ 0 ] = leftIntraDir

candModeList[ 1 ] = aboveIntraDir

if maxAB − minAB == 1

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )

candModeList[ 4 ] = 2 + ( ( minAB + 60 ) % 64 )

candModeList[ 5 ] = 2 + ( maxAB % 64 )

else if maxAB − minAB == 2

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )

candModeList[ 3 ] = 2 + ( ( minAB + 61 ) % 64 )

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )

candModeList[ 5 ] = 2 + ( ( minAB + 60 ) % 64 )

else if maxAB − minAB > 61

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )

candModeList[ 4 ] = 2 + ( minAB % 64 )

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )

else

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )

candModeList[ 3 ] = 2 + ( ( minAB − 1 ) % 64 )

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )

else if (maxAB == minAB, maxAB > DC) or (maxAB != minAB , maxAB > DC)

candModeList[ 0 ] = maxAB

candModeList[ 1 ] = 2 + ( (  maxAB  + 61 ) % 64 )

candModeList[ 2 ] = 2 + ( ( maxAB − 1 ) % 64 )

candModeList[ 3 ] = 2 + ( (  maxAB  + 60 ) % 64 )

candModeList[ 4 ] = 2 + ( maxAB % 64 )

candModeList[ 5 ] = 2 + ( ( maxAB + 59 ) % 64 )

else

candModeList[ 0 ] = VER

candModeList[ 1 ] = HOR

candModeList[ 2 ] = 2

candModeList[ 3 ] = DIA

candModeList[ 4 ] = 66

candModeList[ 5 ] = 26

Figure 2. MPM list generation process for angular modes

Encoding only changes

Since the MPM list derivation process comprising only angular modes, we change the method of adding additional modes into the R-D check list. Instead of adding left intra mode(L) and above intra mode(A) into the R-D check list (VTM-4.0) we add modes with lowest bit cost for mode signaling depend on L and A.

Figure 3 shows the process of choosing the additional modes. For the case L ==A. If L is angular mode, then only adding the lowest bit cost angular mode into R-D check list. Otherwise only adding the lowest bit cost non-angular mode into R-D check list. For the case L != A, adding both the lowest bit cost angular mode and non-angular mode into R-D check list. The number of the added modes is between 1 and 2 which is the same as VTM-4.0.

if L == A

if L > DC

adding lowest bit cost angular mode into R-D check list

else

adding lowest bit cost non-angular mode into R-D check list

else

adding lowest bit cost angular mode + non-angular mode into R-D check list

Figure 3. process of choosing additional modes

Results of CE3-3.5.1 with encoding only changes

The encoding only changes described in section 2 are implemented upon CE3-3.5.1 and is evaluated for all-intra and random-access configuration with VTM 4.0 according to the common test conditions in JVET-M1010 [4].

Encoding time for RA is not reliable.

Table 1. Experiment results of CE3-3.5.1 with encoding only changes

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -0.39% | -0.32% | 98% | 98% |
| Class A2 | -0.14% | -0.02% | -0.15% | 98% | 99% |
| Class B | -0.18% | -0.12% | -0.09% | 101% | 97% |
| Class C | -0.03% | -0.12% | -0.02% | 102% | 98% |
| Class E | -0.20% | -0.18% | -0.06% | 107% | 97% |
| Overall | -0.18% | -0.16% | -0.12% | 101% | 98% |
| Class D | -0.09% | -0.07% | -0.15% | 107% | 95% |
| Class F | 0.08% | 0.07% | -0.02% | 102% | 98% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.25% | -0.15% | 102% | 99% |
| Class A2 | -0.02% | 0.01% | 0.06% | 101% | 99% |
| Class B | -0.04% | 0.03% | 0.04% | 106% | 99% |
| Class C | -0.04% | 0.13% | -0.08% | 115% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.00% | -0.03% | 106% | 99% |
| Class D | -0.01% | -0.12% | 0.04% | 140% | 100% |
| Class F | 0.04% | -0.04% | 0.02% | 124% | 100% |

Conclusions

This contribution proposes to change the method of adding additional modes into the R-D check list to find a better trade-off between the gain and the encoding run-time for explicitly signal non angular modes.

Such proposed encoding changes is implemented upon CE3-3.5.1. The results reportedly show an average luma BD rate performance of -0.18% (AI), -0.06% (RA) without increase of encoding running time over VTM-4.0. It is proposed to adopt CE3-3.5.1 with the proposed encoding only changes.

References

- Jie Yao, Jianqing Zhu, Wenting Cai and Kimihiko Kazui, “Non-CE3: Intra prediction information coding,” JVET-M0210, Jan 2019.

- Geert Van der Auwera, Ling Li, Alexey Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-M1023, Jan 2019.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” JVET-M1001, Jan 2019.

- F. Bossen, J. Boyce, and etc, “JVET common test conditions and software reference configurations for SDR video,” JVET document, JVET-M1010, Jan 2019.

Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).